3GPP TSG-RAN WG4 Meeting #101b-e              	R4-2200751

Electrical Meeting, 17th Jan- 25th Jan, 2022

Agenda item:	8.9.6.2.2

Source:	Samsung

Title:	View on demodulation requirement for Rel-17 NB-IoT

Document for:	Discussion

# 1	Introduction

Based on the WID of Rel-17 additional enhancements for NB-IoT and eMTC WI, this meeting is the 1st meeting to discuss the performance part of NB-IoT and eMTC. The related objective for NB-IoT part is listed as [1]

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. - Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. - Introduce support for NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.).

In this contribution, the general view on the performance requirements impact was provided.

# 2	BS demodulation

16QAM

This objective is to introduce 16QAM scheduling for both UL and DL to increase the peak date rate, compared with QPSK scheduling. New TBS table is introduced. It is necessary to introduce the BS demodulation requirement for verify 16QAM transmission functionality in RAN4.

Proposal 1: BS demodulation requirement with 16QAM transmission can be considered in RAN4.

Simulation assumption.

Based on RAN1 design, 16QAM can be applied for the basic Rel-13 NPUSCH format1 with 3, 6 and 12 tones transmission, Rel-16 multi-TB scheduling and Rel-16 NPUSCH in PUR transmission.

RAN4 has already defined the related requirement in Rel-13 for NPUSCH format1 and in Rel-16 multi-TB scheduling. To verify the baseband processing with 16QAM functionality, in our view, the basic NPUSCH format1 transmission can fulfill the test purpose. We can apply the simulation assumption specified in Rel-13 as starting point.

As per RAN1 agreement, repetition is not used for 16-QAM in uplink. Therefore, only 1 repetition is considered for 16QAM transmission.

Regarding the number of RU configured and related TBS, the following TBS table can be supported in RAN1.

Table 1: TBS table for 16QAM

| I_TBS | I_RU |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |

Proposal 2: The following simulation assumption for 16QAM requirement can be considered as starting point.

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T2R |
| SCS | 15KHz |
| Frequency Resource | 3, 6, 12 tones |
| Number of Repetition | 1 |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| N_RU | FFS |
| I_TBS | FFS |

# 3	Conclusion

In this contribution, the overview on the performance requirements impact for Rel-17 NB-IoT based on RAN1 specified feature was provided

Proposal 1: BS demodulation requirement with 16QAM transmission can be considered in RAN4

Proposal 2: The following simulation assumption for 16QAM requirement can be considered as starting point.

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T2R |
| SCS | 15KHz |
| Frequency Resource | 3, 6, 12 tones |
| Number of Repetition | 1 |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| N_RU | FFS |
| I_TBS | FFS |

# References

RP-201306, WID on Additional enhancements for NB-IoT and LTE-MTC, Huawei