3GPP TSG-RAN WG4 Meeting #101b-e              	R4-2200755

Electrical Meeting, 17th Jan-25th Jan, 2022

Agenda item:	6.12.3.1

Source:	Samsung

Title:	Simulation results for PUSCH with 256QAM

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the remaining issue on the test parameters for FR1 PUSCH demodulation requirement with 256QAM were further discussed. The related agreement was captured in the WF [1] as

- MCS - MCS: MCS20 - Additional DM-RS configuration (dmrsAdditionalPosition): pos1 - PT-RS configuration: Not configure PT-RS - Phase noise modelling - Not consider explicit phase noise modelling in the alignment results - The phase noise impact can be included in the impairment results, but it is left up to companies - Tx EVM - No explicit Tx EVM modelling - Considering 3.5% Tx EVM impact in the impairment results submitted by companies, but it is up to companies - Number of Tx: Only 1Tx - Number of Rx: 2/4/8 Rx - Number of layer: 1 layer - Propagation condition - TDL30-10 low - SCS and Bandwidth combination - 15KHz SCS: 5MHz and 10MHz - 30KHz SCS: 10MHz, 40MHz and 100MHz - Test applicability rules for different antenna configuration - Reusing the existing test applicability rule defined in clause 8.1.2.0 of TS38.141-1? - Test applicability rules for different SCS and bandwidth combination - Reuse the existing applicability rules defined in 8.1.2.1.1 and 8.1.2.1.2 in TS 38.141-1 for different SCS and bandwidth combination - MU and TT - Reuse the existing MU and TT values for PUSCH demodulation test cases defined in TS 38.141-1, but with square brackets for TE vendors’ checking. - Performance requirements for FDD and TDD with different TDD patterns - Performance requirements for BS type 1-O with 2Rx - Both conducted and radiated performance requirements need to be defined

In this contribution, the simulation results with ideal and impairment are provided to finalize the PUSCH requirement s with 256QAM

# 2	Simulation Results

In this section, the initial simulation results are provided. The simulation assumption can be referred as the following table.

Table 1: simulation assumption for PUSCH with 256QAM

| Parameters | Value |
| --- | --- |
| Antenna configuration | 1T2R, 4R and 8R low |
| SCS and BW | 15KHz: 5MHz, 10MHz, 20MHz 30KHz: 10MHz, 40MHz and 100MHz |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30 KHz SCS 3D1SIU S=10D: 2G: 2U for 15 KHz SCS |
| DM-RS type | Type 1 with single-symbol |
| Number of DMRS symbols | 1+1(l0=2) |
| Symbol length | 14 |
| Start symbol index | 0 |
| Time domain resource allocation type | Type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 low |
| Timing offset frequency offset | 0 |

Table 2:  Results summary of 256QAM for MCS20

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (256QAM) | DMRS configuration | Channel | SNR@70 of TP (ideal) | SNR@70% of TP (Impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case1 | 1T2R | 15KHz, 5MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 17.47 | 19.97 |
| Case2 | 1T2R | 15KHz, 10MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 17.88 | 20.38 |
| Case3 | 1T2R | 30KHz, 10MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 17.62 | 20.12 |
| Case4 | 1T2R | 30KHz, 40MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 18.02 | 20.52 |
| Case5 | 1T2R | 30KHz, 100MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 19.33 | 21.83 |
| Case6 | 1T4R | 15KHz, 5MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 14.57 | 17.07 |
| Case7 | 1T4R | 15KHz 10MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 14.83 | 17.33 |
| Case8 | 1T4R | 30KHz 10MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 14.66 | 17.16 |
| Case9 | 1T4R | 30KHz 40MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 15.07 | 17.57 |
| Case10 | 1T4R | 30KHz 100MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 15.3 | 17.8 |
| Case 11 | 1T8R | 15KHz, 5MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 11.19 | 13.69 |
| Case 12 | 1T8R | 15KHz 10MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 11.32 | 13.82 |
| Case13 | 1T8R | 30KHz 10MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 11.18 | 13.68 |
| Case14 | 1T8R | 30KHz 40MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 11.4 | 13.9 |
| Case15 | 1T8R | 30KHz 100MHz | A | 14 | 20 | 1+1 (2,11) | TDLA30-10 | 11.95 | 14.45 |
| Case16 | 1T2R | 15KHz, 5MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 17.52 | 20.02 |
| Case17 | 1T2R | 15KHz 10MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 17.89 | 20.39 |
| Case18 | 1T2R | 30KHz 10MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 17.68 | 20.18 |
| Case19 | 1T2R | 30KHz 40MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 18.03 | 20.53 |
| Case20 | 1T2R | 30KHz 100MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 19.33 | 21.83 |
| Case21 | 1T4R | 15KHz, 5MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 14.59 | 17.09 |
| Case22 | 1T4R | 15KHz 10MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 14.82 | 17.32 |
| Case23 | 1T4R | 30KHz 10MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 14.69 | 17.19 |
| Case24 | 1T4R | 30KHz 40MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 15.06 | 17.56 |
| Case25 | 1T4R | 30KHz 100MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 15.29 | 17.79 |
| Case26 | 1T8R | 15KHz, 5MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 11.1 | 13.6 |
| Case227 | 1T8R | 15KHz 10MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 11.31 | 13.81 |
| Case28 | 1T8R | 30KHz 10MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 11.2 | 13.7 |
| Case29 | 1T8R | 30KHz 40MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 11.42 | 13.92 |
| Case30 | 1T8R | 30KHz 100MHz | B | 14 | 20 | 1+1 (0,10) | TDLA30-10 | 11.95 | 14.45 |

# 3	Conclusion

In this contribution, the ideal and impairment results are provided for PUSCH 256QAM requirement finalization.

# References

R4-2120717, WF on FR1 PUSCH 256QAM demodulation requirement, Huawei