3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200758

Online Meeting, Jan 17 – 25, 2022

Agenda item:	6.21.2.2

Source: 	Qualcomm Incorporated

Title: 	Simulation results with reduced number of PRS samples

Document for:	Discussion

# Introduction

RAN4 reached some agreements regarding the simulation assumptions for measurement accuracy with a reduced number of samples in RAN4#101-e [1, 2]. This paper includes preliminary simulation results for the following:

- UE Rx-Tx time difference

- PRS-RSRP

# Simulation results for UE Rx-Tx time difference

This section includes simulation results for FR1, SCS = 15 kHz and SCS = 30 kHz, with propagation channels AWGN and TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 120 | 122 | 131 |
|  |  |  |  | 2 | 1 | 118 | 120 | 124 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 118 | 118 | 121 |
|  |  |  |  | 2 | 4 | 118 | 118 | 119 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 52 |  | 128 | 1 | 1 | 58 | 58 | 61 |
|  |  |  |  | 2 | 1 | 58 | 58 | 59 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 58 | 58 | 58 |
|  |  |  |  | 2 | 4 | 58 | 58 | 58 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 31 | 31 | 31 |
|  |  |  |  | 2 | 1 | 31 | 31 | 31 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 31 | 31 | 31 |
|  |  |  |  | 2 | 4 | 31 | 31 | 31 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 1: UE Rx-Tx simulation results for FR1, SCS = 15 kHz, AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 58 | 59 | 66 |
|  |  |  |  | 2 | 1 | 58 | 58 | 61 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 58 | 58 | 60 |
|  |  |  |  | 2 | 4 | 58 | 58 | 59 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 48 |  | 64 | 1 | 1 | 30 | 30 | 31 |
|  |  |  |  | 2 | 1 | 30 | 30 | 30 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 30 | 30 | 30 |
|  |  |  |  | 2 | 4 | 30 | 30 | 30 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 16 | 16 | 16 |
|  |  |  |  | 2 | 1 | 16 | 16 | 16 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 16 | 16 | 16 |
|  |  |  |  | 2 | 4 | 16 | 16 | 16 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 2: UE Rx-Tx simulation results for FR1, SCS = 30 kHz, AWGN.

Observation 1: There are differences in sampling rate captured in the simulation assumptions Tdoc from RAN4#101 [2] vs. the assumptions in Rel-16.

- for FR1, SCS = 15 kHz, num RB = 24, the sampling period was 128 Tc in Rel-16

- for FR2, SCS = 60 kHz, num RB = 24, the sampling period was 32 Tc in Rel-16

- for FR2, SCS = 120 kHz, num RB = 32, the sampling period was 16 Tc in Rel-16

Observation 2: For FR1, with the exception of SCS = 15, num RB = 24, Rel-16 UE Rx-Tx accuracy requirements in AWGN at SINR = -13 dB can be met at SINR ≥ -6 dB with number of samples M1 = 1, 2.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 163 | 164 | 224 |
|  |  |  |  | 2 | 1 | 162 | 163 | 186 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 162 | 163 | 167 |
|  |  |  |  | 2 | 4 | 162 | 162 | 163 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 52 |  | 128 | 1 | 1 | 106 | 106 | 113 |
|  |  |  |  | 2 | 1 | 106 | 106 | 108 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 105 | 105 | 106 |
|  |  |  |  | 2 | 4 | 105 | 105 | 106 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 66 | 66 | 69 |
|  |  |  |  | 2 | 1 | 65 | 66 | 66 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 65 | 65 | 65 |
|  |  |  |  | 2 | 4 | 65 | 65 | 64 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 3: UE Rx-Tx simulation results for FR1, SCS = 15 kHz, TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 106 | 109 | 159 |
|  |  |  |  | 2 | 1 | 105 | 106 | 116 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 105 | 105 | 107 |
|  |  |  |  | 2 | 4 | 104 | 105 | 104 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 48 |  | 64 | 1 | 1 | 69 | 70 | 83 |
|  |  |  |  | 2 | 1 | 69 | 69 | 74 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 69 | 69 | 71 |
|  |  |  |  | 2 | 4 | 69 | 69 | 71 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 42 | 43 | 43 |
|  |  |  |  | 2 | 1 | 42 | 43 | 43 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 42 | 43 | 43 |
|  |  |  |  | 2 | 4 | 43 | 43 | 43 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 4: UE Rx-Tx simulation results for FR1, SCS = 30 kHz, TDL-A.

Observation 3: For FR1, Rel-16 UE Rx-Tx accuracy requirements in TDL-A at SINR = -13 dB cannot be met at SINR = -3 dB with number of samples M1 = 1, 2.

# Simulation results for PRS-RSRP

This section includes simulation results for FR1, SCS = 15 kHz and SCS = 30 kHz, with propagation channels AWGN and TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 0.7 | 0.9 | 2.1 |
|  |  |  |  | 2 | 1 | 0.6 | 0.7 | 1.6 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.5 | 0.6 | 1.2 |
|  |  |  |  | 2 | 4 | 0.4 | 0.5 | 0.9 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 52 |  | 128 | 1 | 1 | 0.5 | 0.6 | 1.6 |
|  |  |  |  | 2 | 1 | 0.4 | 0.5 | 1.1 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 0.8 |
|  |  |  |  | 2 | 4 | 0.3 | 0.3 | 0.6 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 0.3 | 0.5 | 1.2 |
|  |  |  |  | 2 | 1 | 0.3 | 0.3 | 0.8 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.2 | 0.3 | 0.6 |
|  |  |  |  | 2 | 4 | 0.2 | 0.2 | 0.4 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 5: PRS-RSRP simulation results for FR1, SCS = 15 kHz, AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 0.7 | 1.1 | 3.2 |
|  |  |  |  | 2 | 1 | 0.5 | 0.8 | 2.3 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.4 | 0.5 | 1.7 |
|  |  |  |  | 2 | 4 | 0.3 | 0.4 | 1.2 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 48 |  | 64 | 1 | 1 | 0.5 | 0.7 | 1.6 |
|  |  |  |  | 2 | 1 | 0.4 | 0.5 | 1.2 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 0.8 |
|  |  |  |  | 2 | 4 | 0.2 | 0.3 | 0.6 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 0.3 | 0.5 | 1.3 |
|  |  |  |  | 2 | 1 | 0.2 | 0.3 | 0.9 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.2 | 0.2 | 0.6 |
|  |  |  |  | 2 | 4 | 0.1 | 0.2 | 0.4 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 6: PRS-RSRP simulation results for FR1, SCS = 30 kHz, AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 1 | 0.8 | 1.2 | 3.1 |
|  |  |  |  | 2 | 1 | 0.6 | 0.9 | 2.3 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.5 | 0.7 | 1.7 |
|  |  |  |  | 2 | 4 | 0.4 | 0.5 | 1.2 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 52 |  | 128 | 1 | 1 | 0.6 | 0.8 | 2.2 |
|  |  |  |  | 2 | 1 | 0.4 | 0.6 | 1.5 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 1.1 |
|  |  |  |  | 2 | 4 | 0.3 | 0.3 | 0.8 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 104 |  | 64 | 1 | 1 | 0.4 | 0.6 | 1.7 |
|  |  |  |  | 2 | 1 | 0.3 | 0.4 | 1.1 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.2 | 0.3 | 0.8 |
|  |  |  |  | 2 | 4 | 0.2 | 0.2 | 0.5 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 7: PRS-RSRP simulation results for FR1, SCS = 15 kHz, TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 30 | 128 | 1 | 1 | 1.0 | 1.6 | 4.7 |
|  |  |  |  | 2 | 1 | 0.7 | 1.1 | 3.4 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.5 | 0.8 | 2.4 |
|  |  |  |  | 2 | 4 | 0.4 | 0.6 | 1.8 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 48 |  | 64 | 1 | 1 | 0.6 | 0.8 | 2.1 |
|  |  |  |  | 2 | 1 | 0.4 | 0.6 | 1.5 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.3 | 0.4 | 1.0 |
|  |  |  |  | 2 | 4 | 0.3 | 0.3 | 0.7 |
|  |  |  |  | 4 | 4 |  |  |  |
|  | 132 |  | 32 | 1 | 1 | 0.4 | 0.6 | 1.8 |
|  |  |  |  | 2 | 1 | 0.3 | 0.4 | 1.2 |
|  |  |  |  | 4 | 1 |  |  |  |
|  |  |  |  | 1 | 4 | 0.2 | 0.3 | 0.7 |
|  |  |  |  | 2 | 4 | 0.2 | 0.2 | 0.5 |
|  |  |  |  | 4 | 4 |  |  |  |

Table 8: PRS-RSRP simulation results for FR1, SCS = 30 kHz, TDL-A.

Observation 4: For FR1, Rel-16 RSRP accuracy requirements at SINR = -13 dB can be met at SINR = -6 dB with number of samples M1 = 1, 2.

# Conclusions

Observation 1: There are differences in sampling rate captured in the simulation assumptions Tdoc from RAN4#101 [2] vs. the assumptions in Rel-16.

- for FR1, SCS = 15 kHz, num RB = 24, the sampling period was 128 Tc in Rel-16

- for FR2, SCS = 60 kHz, num RB = 24, the sampling period was 32 Tc in Rel-16

- for FR2, SCS = 120 kHz, num RB = 32, the sampling period was 16 Tc in Rel-16

Observation 2: For FR1, with the exception of SCS = 15, num RB = 24, Rel-16 UE Rx-Tx accuracy requirements in AWGN at SINR = -13 dB can be met at SINR ≥ -6 dB with number of samples M1 = 1, 2.

Observation 3: For FR1, Rel-16 UE Rx-Tx accuracy requirements in TDL-A at SINR = -13 dB cannot be met at SINR = -3 dB with number of samples M1 = 1, 2.

Observation 4: For FR1, Rel-16 RSRP accuracy requirements at SINR = -13 dB can be met at SINR = -6 dB with number of samples M1 = 1, 2.

# References

[1] R4-2120419, WF on NR Positioning Enhancements (Part 1), RAN4#101-e

[2] R4-2120330, Link level simulation assumptions for NR PRS measurements with reduced latency, RAN4#101-e