3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200764

Online Meeting, Jan 17 – 25, 2022

Agenda item:	8.9.4.1

Source: 	Qualcomm Incorporated

Title: 	On NB-IoT neighbor cell measurements in RRC_CONNECTED

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for connected mode NB-IoT neighbor cell measurements in RAN4#101-e. Progress was captured in a WF [1]. Agreements are reproduced below.

### Scenarios prioritization

Deprioritize defining CONNECTED mode neighbour cell measurement requirements when the target cell is in enhanced coverage

FFS: Deprioritize defining CONNECTED mode neighbor cell measurement requirements when the serving cell is in enhanced coverage.

In addition, RAN2 reached the following agreements [2].

RAN2#116-e agreements:

- NW signals two separate thresholds for intra- and inter-frequency measurements.

- The values of s-SearchDeltaP and TSearchDeltaP may be different in RRC_CONNECTED and RRC_IDLE, they are signalled in a separate set of parameters.

- s-SearchDeltaP has the same value range as the existing RRC_IDLE parameter

  - FFS how to specify the state change

  - [FFS] An indication that the UE starts measurement is not introduced.

- No enhancement is introduced to have a shorter T310 timer for mobile UEs supporting connected mode measurement.

- For RRC_CONNECTED state, TSearchDeltaP is configured via SIB.

- Working assumption: For RRC_CONNECTED state, TSearchDeltaP range is 10 – 60 seconds.

- For RRC_CONNECTED state, no default value for TSearchDeltaP.

- No limit for how long UE can remain in relaxed neighbour cell monitoring state while it is in  RRC_CONNECTED state.

- For RRC_CONNECTED state, the RRC_IDLE state SSearchDeltaP is not used if the RRC_CONNECTED state SSearchDeltaP is not provided.

- Relaxed neighbour cell monitoring is enabled in RRC_CONNECTED state if TSearchDeltaP and SSearchDeltaP for RRC_CONNECTED state are provided.

In this paper we discuss the following topics:

- Inter-frequency measurement period

- Intra-frequency measurement period

# Inter-frequency measurement period

One of the key requirements to be specified for NB-IoT neighbor cell measurements in connected mode is the measurement period for inter-frequency measurements. In this section we provide our proposals for formulating these requirements.

As agreed earlier by RAN4, inter-frequency neighbor cell measurements in connected mode are opportunistic; the UE can attempt to detect and/or measure inter-frequency neighbors during periods of time where there is no UL/DL data transfer scheduled and it is not required to monitor any DL transmission [3].

- When the carrier frequencies of serving cell and of measurement neighbour cell are different (scenarios B/D/E), UE can perform neighbour cell measurement without gaps without causing interruptions to serving cell using any occasion where the UE is not scheduled which includes any of the following:

  - Vacant slots not scheduled for data transmission, i.e. when not required to do data transmission/reception

  - When not required to do NPDCCH monitoring

  - during the DRX inactive period i.e. when the UE is configured with DRX.

Proposal 1: The measurement period requirements for inter-frequency cell detection and measurements in connected mode shall apply assuming no UL/DL scheduling for the UE during the measurement period.

As discussed in RAN4#101-e, the inter-frequency measurement period requirement will be based on a minimum length per occasion [1]. We propose the values below for measurements and cell-detection.

Proposal 2: Inter-frequency measurement and cell detection requirements are based on minimum length(s) per occasion

- Minimum length of occasion for measurements: Tmeas,  occ=[50] ms

- Minimum length of occasion for cell detection: Tdetect,  occ=[200] ms

Proposal 3: The minimum time interval between adjacent occasions in the same NPDCCH monitoring period is Tg=[50] ms.

Proposal 4: Without DRX and assuming no UL/DL scheduling, the available time for measurements during one NPDCCH monitoring period would be Tavail=TNPDCCH-LNPDCCH-Tproc-2∙TRT, where

  - TNPDCCH is the NPDCCH monitoring period length,

  - LNPDCCH is the duration of a NPDCCH candidate with Rmax repetitions,

  - Tproc is the NPDCCH processing time,

  - TRT is the receiver retuning time.

Figure 1: Illustration of available measurement time for inter-frequency measurements

Observation 1: In general, LNPDCCH > Rmax, because NPDCCH can be transmitted only on valid DL NB-IoT subframes, excluding gaps in NPDCCH scheduling configured by the network via DL-GapConfig-NB and subframes for NPBCH, NPSS, NSSS, SIB1, and SI-NB messages.

Proposal 5: Tproc = 4 ms, consistent with the minimum scheduling gap between NPDCCH and NPDSCH.

Proposal 6: TRT = 1 ms, consistent with the UL/DL transition time requirement for half-duplex FDD.

Note that, in general, it may not be straightforward or practical to formulate the requirements based on exact values of the parameter LNPDCCH calculated for each scenario because of the many different factors involved. Perhaps requirements may be based using an upper bound for LNPDCCH or by limiting the applicability of the requirements to a reduced set of scenarios. Nonetheless, once Tavail has been calculated the measurement period requirement can be derived following the steps outlined below.

Proposal 7: FFS whether to derive the inter-frequency measurement period requirements based on the exact value of LNPDCCH, possibly applicable to a limited set of scenarios, or by using an upper bound for LNPDCCH.

Proposal 8: The following steps are applied to derive the inter-frequency measurement period requirement per carrier without DRX

- Choose the duplexing mode (FDD/TDD) and carrier type (anchor, non-anchor).

- For TDD, choose the UL/DL configuration.

- Calculate TNPDCCH and LNPDCCH for all applicable values of G, Rmax.

- Calculate Tavail=TNPDCCH-LNPDCCH-Tproc-2∙TRT.

- Given Tmeas,  basic = 800 ms for NRS-based measurements in normal coverage, calculate the required number of measurement occasions: Nocc=Tmeas,  basicTmeas,  occ.

- Calculate the number of available measurement occasions per NPDDCH period: Navail=Tavail-Tmeas, occTmeas, occ+Tg+1.

- Set D=Nocc-NoccNavail∙Navail.

- For D>0, the measurement period is given by

Tmeas=NoccNavail∙TNPDCCH+D∙Tocc+Tg-Tg+LNPDCCH+Tproc+2∙TRT

- For D=0, the measurement period is given by

Tmeas=NoccNavail-1∙TNPDCCH+Navail∙Tocc+Tg-Tg+LNPDCCH+Tproc+2∙TRT

The same steps can be applied to derive the requirement for cell detection measurement period, replacing the parameters Tmeas,  basic and Tdetect,  basic with the corresponding ones for cell detection.

Proposal 9: To the derive the inter-frequency cell detection period requirement per carrier without DRX, follow the steps in proposal 8, replacing Tmeas,  basic with Tdetect,  basic=1400 ms and  Tmeas,  occ with Tdetect,  occ.

Observation 2: It may not be necessary to capture the steps above (Proposal 8) in the specifications.

For illustration purposes, we show in the Appendix the results of applying the proposed steps to a FDD non-anchor carrier to derive the measurement period. That scenario is the most favorable to performing neighbor cell inter-frequency measurements because of the high availability of subframes for NPDCCH transmission. In other cases, NPDCCH would take relatively longer and there would be less time available for measurements.

Measurement period duration for FDD non-anchor carrier according to our proposals are shown in Table 6. Even in this most favorable scenario, there are some configurations that would not support inter-frequency measurements, as shown in Table 6. There would be even fewer configurations that support inter-frequency cell detection in the same scenario (with longer occasions) as shown in Table 1.

| T_detect |  | Rmax (npdcch-NumRepetitions) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
| G (npdcch-StartSF-USS) | 1.5 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 5326 | 5838 | 5826 |
|  | 2 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 3534 | 3790 | 3778 | 3754 |
|  | 4 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 3406 | 2510 | 2766 | 2730 | 3754 |
|  | 8 | N/A | N/A | N/A | N/A | N/A | 1774 | 3342 | 2382 | 1962 | 2218 | 2730 | 3754 |
|  | 16 | N/A | N/A | N/A | N/A | 1758 | 1774 | 1794 | 1834 | 1962 | 2218 | 2730 | 3754 |
|  | 32 | N/A | N/A | N/A | 1750 | 1758 | 1762 | 1770 | 1834 | 1962 | 2218 | 2730 | 3754 |
|  | 48 | N/A | N/A | N/A | 2518 | 1758 | 1774 | 1770 | 1834 | 1962 | 2218 | 2730 | 3754 |
|  | 64 | N/A | N/A | 1746 | 1750 | 1746 | 1738 | 1770 | 1834 | 1962 | 2218 | 2730 | 3754 |

Table 1: Cell detection period requirement for FDD non-anchor carrier (ms)

The situation would be significantly worse for FDD anchor carriers and TDD carriers because of higher overhead (NPSS, NSSS, NPBCH, etc.) and lower availability of DL subframes. At least when there is no DRX, it would be beneficial to enable the UE to indicate when it needs to start performing inter-frequency measurements so that the network may try to create more opportunities for the UE to perform the measurements.

Proposal 10: RAN4 should send an LS to RAN2 informing RAN2 that our analysis shows that, in many scenarios, it would be beneficial for the UE to indicate when it needs to start performing inter-frequency measurements so that the serving cell may create measurement opportunities for the UE.

# Intra-frequency measurement period

The proposal below for intra-frequency measurement period in connected mode was discussed [1] in RAN4#101-e. The proposal mostly leverages existing requirements for NB-IoT serving cell measurements in connected mode for cell detection, both when the serving/target cell is in normal coverage conditions. The exception is Tdetect_intra = 5 DRX cycles does not leverage existing requirements and

Intra-frequency requirements

FFS:

When DRX is not configured:

- Tdetect_intra = 1400 ms

- Tmeasure_intra = 800 ms for NRS-based measurement

- Tmeasure_intra = 1600 ms for NSSS-based measurement

When DRX is configured:

- Tdetect_intra = 5 DRX cycles

- Tmeasure_intra = 5 DRX cycles.

We support the above proposal for intra-frequency measurement and cell detection requirements in connected mode when the target cell is in normal coverage.

Proposal 11: The intra-frequency measurement and cell detection requirements in connected mode when the target cell is in normal coverage.

When DRX is not configured:

- Tdetect_intra = 1400 ms

- Tmeasure_intra = 800 ms for NRS-based measurement

- Tmeasure_intra = 1600 ms for NSSS-based measurement

When DRX is configured:

- Tdetect_intra  is FFS

- Tmeasure_intra = 5 DRX cycles.

# Conclusions

Proposal 1: The measurement period requirements for inter-frequency cell detection and measurements in connected mode shall apply assuming no UL/DL scheduling for the UE during the measurement period.

Proposal 2: Inter-frequency measurement and cell detection requirements are based on minimum length(s) per occasion

- Minimum length of occasion for measurements: Tmeas,  occ=[50] ms

- Minimum length of occasion for cell detection: Tdetect,  occ=[200] ms

Proposal 3: The minimum time interval between adjacent occasions in the same NPDCCH monitoring period is Tg=[50] ms.

Proposal 4: Without DRX and assuming no UL/DL scheduling, the available time for measurements during one NPDCCH monitoring period would be Tavail=TNPDCCH-LNPDCCH-Tproc-2∙TRT, where

  - TNPDCCH is the NPDCCH monitoring period length,

  - LNPDCCH is the duration of a NPDCCH candidate with Rmax repetitions,

  - Tproc is the NPDCCH processing time,

  - TRT is the receiver retuning time.

Observation 1: In general, LNPDCCH > Rmax, because NPDCCH can be transmitted only on valid DL NB-IoT subframes, excluding gaps in NPDCCH scheduling configured by the network via DL-GapConfig-NB and subframes for NPBCH, NPSS, NSSS, SIB1, and SI-NB messages.

Proposal 5: Tproc = 4 ms, consistent with the minimum scheduling gap between NPDCCH and NPDSCH.

Proposal 6: TRT = 1 ms, consistent with the UL/DL transition time requirement for half-duplex FDD.

Proposal 7: FFS whether to derive the inter-frequency measurement period requirements based on the exact value of LNPDCCH, possibly applicable to a limited set of scenarios, or by using an upper bound for LNPDCCH.

Proposal 8: The following steps are applied to derive the inter-frequency measurement period requirement per carrier without DRX

- Choose the duplexing mode (FDD/TDD) and carrier type (anchor, non-anchor).

- For TDD, choose the UL/DL configuration.

- Calculate TNPDCCH and LNPDCCH for all applicable values of G, Rmax.

- Calculate Tavail=TNPDCCH-LNPDCCH-Tproc-2∙TRT.

- Given Tmeas,  basic = 800 ms for NRS-based measurements in normal coverage, calculate the required number of measurement occasions: Nocc=Tmeas,  basicTmeas,  occ.

- Calculate the number of available measurement occasions per NPDDCH period: Navail=Tavail-Tmeas, occTmeas, occ+Tg+1.

- Set D=Nocc-NoccNavail∙Navail.

- For D>0, the measurement period is given by

Tmeas=NoccNavail∙TNPDCCH+D∙Tocc+Tg-Tg+LNPDCCH+Tproc+2∙TRT

- For D=0, the measurement period is given by

Tmeas=NoccNavail-1∙TNPDCCH+Navail∙Tocc+Tg-Tg+LNPDCCH+Tproc+2∙TRT

Observation 2: It may not be necessary to capture the steps above (Proposal 8) in the specifications.

Proposal 10: RAN4 should send an LS to RAN2 informing RAN2 that our analysis shows that, in many scenarios, it would be beneficial for the UE to indicate when it needs to start performing inter-frequency measurements so that the serving cell may create measurement opportunities for the UE.

(continued)

Proposal 11: The intra-frequency measurement and cell detection requirements in connected mode when the target cell is in normal coverage.

When DRX is not configured:

- Tdetect_intra = 1400 ms

- Tmeasure_intra = 800 ms for NRS-based measurement

- Tmeasure_intra = 1600 ms for NSSS-based measurement

When DRX is configured:

- Tdetect_intra  is FFS

- Tmeasure_intra = 5 DRX cycles.

# References

[1] R4-2120340, WF on RRM requirements of Rel-17 NB-IoT and LTE-MTC, RAN4#101-e

[2] RP-213365, Status report for Additional enhancements for NB-IoT and LTE-MTC, RAN#94-e

[3] R4-2105800, Reply LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state, RAN4#98-bis-e

# Appendix

Below we include tables showing final and intermediate results of applying the steps in proposal 8 to FDD non-anchor carrier to the derive the inter-frequency measurement period per carrier and without DRX. The calculations assume that all subframes are valid DL NB-IoT subframes where NPDCCH can be transmitted. i.e. LNPDCCH = Rmax. Parameter values in proposals 2, 3, 5 and 6 are used in the calculations.

| T_NPDCCH |  | Rmax (npdcch-NumRepetitions) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
| G (npdcch-StartSF-USS) | 1.5 | 4 | 4 | 6 | 12 | 24 | 48 | 96 | 192 | 384 | 768 | 1536 | 3072 |
|  | 2 | 4 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 |
|  | 4 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 |
|  | 8 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 |
|  | 16 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 | 32768 |
|  | 32 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 | 32768 | 65536 |
|  | 48 | 48 | 96 | 192 | 384 | 768 | 1536 | 3072 | 6144 | 12288 | 24576 | 49152 | 98304 |
|  | 64 | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 | 32768 | 65536 | 131072 |

Table 2: NPDCCH monitoring period (ms)

| L_NPDCCH |  | Rmax (npdcch-NumRepetitions) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
| G (npdcch-StartSF-USS) | 1.5 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
|  | 2 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
|  | 4 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
|  | 8 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
|  | 16 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
|  | 32 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
|  | 48 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
|  | 64 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |

Table 3: Maximum duration of NPDCCH candidate (ms)

| T_avail |  | Rmax (npdcch-NumRepetitions) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
| G (npdcch-StartSF-USS) | 1.5 | 0 | 0 | 0 | 0 | 2 | 10 | 26 | 58 | 122 | 250 | 506 | 1018 |
|  | 2 | 0 | 0 | 0 | 2 | 10 | 26 | 58 | 122 | 250 | 506 | 1018 | 2042 |
|  | 4 | 0 | 0 | 6 | 18 | 42 | 90 | 186 | 378 | 762 | 1530 | 3066 | 6138 |
|  | 8 | 1 | 8 | 22 | 50 | 106 | 218 | 442 | 890 | 1786 | 3578 | 7162 | 14330 |
|  | 16 | 9 | 24 | 54 | 114 | 234 | 474 | 954 | 1914 | 3834 | 7674 | 15354 | 30714 |
|  | 32 | 25 | 56 | 118 | 242 | 490 | 986 | 1978 | 3962 | 7930 | 15866 | 31738 | 63482 |
|  | 48 | 41 | 88 | 182 | 370 | 746 | 1498 | 3002 | 6010 | 12026 | 24058 | 48122 | 96250 |
|  | 64 | 57 | 120 | 246 | 498 | 1002 | 2010 | 4026 | 8058 | 16122 | 32250 | 64506 | 129018 |

Table 4: Available measurement time (ms)

| N_avail |  | Rmax (npdcch-NumRepetitions) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
| G (npdcch-StartSF-USS) | 1.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 3 | 5 | 10 |
|  | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 3 | 5 | 10 | 20 |
|  | 4 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 4 | 8 | 15 | 31 | 61 |
|  | 8 | 0 | 0 | 0 | 1 | 1 | 2 | 4 | 9 | 18 | 36 | 72 | 143 |
|  | 16 | 0 | 0 | 1 | 1 | 2 | 5 | 10 | 19 | 38 | 77 | 154 | 307 |
|  | 32 | 0 | 1 | 1 | 2 | 5 | 10 | 20 | 40 | 79 | 159 | 317 | 635 |
|  | 48 | 0 | 1 | 2 | 4 | 7 | 15 | 30 | 60 | 120 | 241 | 481 | 963 |
|  | 64 | 1 | 1 | 2 | 5 | 10 | 20 | 40 | 81 | 161 | 323 | 645 | 1290 |

Table 5: Number of available occasions per NPDCH period

| T_meas |  | Rmax (npdcch-NumRepetitions) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 | 2048 |
| G (npdcch-StartSF-USS) | 1.5 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 3064 | 6072 | 4408 | 5688 | 5676 |
|  | 2 | N/A | N/A | N/A | N/A | N/A | N/A | 2040 | 4024 | 2872 | 3640 | 3628 | 3604 |
|  | 4 | N/A | N/A | N/A | N/A | N/A | 2008 | 2012 | 2020 | 2036 | 2616 | 2580 | 3604 |
|  | 8 | N/A | N/A | N/A | 1024 | 1992 | 1980 | 1956 | 1808 | 1812 | 2068 | 2580 | 3604 |
|  | 16 | N/A | N/A | 1020 | 1984 | 1964 | 1624 | 1644 | 1684 | 1812 | 2068 | 2580 | 3604 |
|  | 32 | N/A | 1018 | 1980 | 1956 | 1608 | 1612 | 1620 | 1684 | 1812 | 2068 | 2580 | 3604 |
|  | 48 | N/A | 1498 | 1504 | 1516 | 1708 | 1624 | 1620 | 1684 | 1812 | 2068 | 2580 | 3604 |
|  | 64 | 1017 | 1978 | 1952 | 1600 | 1596 | 1588 | 1620 | 1684 | 1812 | 2068 | 2580 | 3604 |

Table 6: Measurement period requirement for FDD non-anchor carrier (ms)