3GPP TSG-RAN WG4 Meeting # 101b-e	R4-2200788

Electronic Meeting, 17th–25th, Jan., 2022

Agenda Item:	6.19.3.2

Source:	Intel Corporation

Title:	Discussion about inter-cell beam management in FeMIMO

Document for:	Discussion

# Introduction

In this meeting, we will provide our view regarding to inter-cell beam management, the WF[1] in last meeting is as follows:

- Define known/unknown cell condition for non-serving cell configured for L1-RSRP measurements - Measurement accuracy for serving cell L1-RSRP can be reused for L1-RSRP measurement for non-serving cell for inter-cell beam management - The L1-RSRP measurement requirements for non-serving cells are only applicable to SSB withing the active DL BWP of the UE. - RAN4 develops UE requirements for at least Nmax = 1, where Nmax is the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting. - RAN4 will further study to specify the requirements for other Nmax value taking RAN1 decision into account - RAN4 will further study timing offset assumptions between non-serving cell and serving cell for L1-RSRP measurement in future RAN4 meetings - Where non-serving cell refers to all the TRPs configured by gNB for inter-cell BM and serving cell(s) is the cell(s)/TRP(s) from which UE receive PDCCH/PDSCH. - RAN4 will further study the FFT implementation assumption for L1-RSRP measurement - RAN4 will further study the Rx beam assumption for L1 RSRP measurement - RAN4 will further study the measurement behaviour for L1-RSRP measurement for non-serving cell whether to perform the non-serving cell L1-RSRP measurement outside SMTC or within SMTC

Furthermore, there is LS[2] from RAN1 to ask for UE behavior assumption in RAN4. We will also provide our view regarding this issue as well.

# Beam measurement for inter cell management

## 2.1 Timing offset and FFT implementation assumptions

From the agreements from RAN1, there is no conclusion about timing offset assumption between serving cell and cells with different PCI.

RAN1 106bis Conclusion On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, there is no consensus in RAN1 on UE timing assumption on reception of signals from TRPs with PCIs different from the serving cell compared to that for serving cell

For FFT implementation assumptions, it was discussed on Rx timing issue from TRPs with different PCIDs, but no conclusion was reached in RAN1 due to concern from some UE vendors either.

Observation 1: There is no conclusion about FFT implementation and timing offset assumption between serving cell and TRP with different PCI in RAN1.

From our understanding, different UE implement can be supported, e.g. multiple FFT or single FFT implementation. It’s fine to use single FFT assumption as baseline to define requirement.

In legacy SSB based L3 measurement, there is no timing offset assumption. However, for inter-cell beam measurement, if timing offset is large, UE may not receive the signal from serving cell and inter-cell simultaneously, which is possible scenario in multi-TRP. Besides, for inter-cell TCI state switching where SSB is associated with TRP with different PCI, extra delay may be spent for DL timing tracking for inter-cell. Therefore, we prefer that the timing offset is smaller than CP. We also have some concern if too strict timing offset requirement is defined, it will have impact on the implementation of the feature. It’s better to further study the impact if timing offset is larger than CP.

## 2.2 Rx beam assumption for L1 RSRP measurement

Here, we mainly discuss the RX beam assumption for FR2.

In legacy L1-RSRP for serving cell, RX beam is assumed to be different for L1 serving cell and L3 neighbor cells measurements. Therefore, L1-RSRP for serving cell is performed outside SMTC. For L1-RSRP for neighbor cell, the RX beam will still be different from L1-RSRP of serving cell. The main question is whether Rx beam for L1 measurement and L3 measurement of neighbor cells can be the same.

From our understanding, first we will perform L3 measurement to find suitable cell, then we need to further evaluate beam quality for the identified neighbor cell. L1-RSRP of neighbor cell beam measurement is performed after L3 neighbor cell measurement. Since L1-RSRP is performed for a specific cell, it’s possible to find a better RX beam to calculate more accurate beam quality for this cell. therefore, Rx beam for L1-RSRP measurement of neighbor cell may be different from that of L3 measurement. It will use the SSBs outside SMTC to find the best RX beam. If SSB outside SMTC is used, another benefit is shorter measure delay for inter-cell L1-RSRP. Beam measurement needs quick response. SMTC periodicity may be much larger than that of SSB. If UE perform L1-RSRP of inter-cell outside SMTC, it will feedback the result more quickly. However, SSB for inter-cell L1-RSRP measurement will be shared with SSB for L1-RSRP/RLM/BFD of serving cell, the measurement delay for serving cell may be extended.

Observation 2: For FR2, if different RX beams are assumed for L1 and L3 measurement of neighbor cell, more accurate beam quality measurement can be achieved and shorter measurement delay is expected for inter-cell beam measurement. However, measurement delay for L1 serving cell will be impacted and extended.

If L1-RSRP of neighbor cell is performed in the same SMTC with that of L3 measurement, since it’s agreed that no L3 measurement will be impacted, it means that L1-RSRP will only use the same beam of L3 measurement, UE may not find more suitable RX beam to calculate the beam quality. The L1-RSRP result may not be so accurate. Besides, SMTC periodicity is longer than SSB, measurement delay for L1-RSRP of neighbor cell will be extended. The advantage of this option is that measurement period for L1 serving cell measurement will not be delayed.

Observation 3: For FR2, if same RX beam is assumed for L1 and L3 measurement of neighbor cell, L1 serving cell measurement will not be delayed. however, beam quality measurement may not be accurate and measurement delay may be longer for inter-cell beam measurement.

It’s a tradeoff between performance loss and measurement delay and complexity. It’s possible that we need to consider a balanced method for RX beam assumption, i.e. SSB inside SMTC and outside SMTC are both used.

L1-RSRP of inter-cell beam measurement will be performed after L3 neighbor cell measurement since L3 measurement will find the proper cell first. When performing L3 measurement, even if the RX beam is not so accurate, UE can still attain some initial RX beam sweeping result for the cell. then the following L1-RSRP measurement can be performed outside SMTC. Different RX beam can be assumed. Then outside SMTC, UE can find the best beam using less samples. In legacy, N=8 is defined for RX beam sweeping. Maybe some smaller number can be defined for RX beam sweeping factor, e.g. N=3 or 4. Therefore, the impact to legacy RLM/L1-RSRP/BFD serving cell measurement will be reduced.

Proposal 1: Both SSBs inside and outside SMTC can be used for L1-RSRP measurement for inter-cell. RX beam sweeping factor can be further reduced.

Proposal 2: If L1-RSRP measurement for inter-cell is performed inside SMTC, it will use the same RX beam of L3 measurement. If L1-RSRP measurement of neighbor cell is performed outside SMTC, different RX can be used.

## 2.3 Inter-cell beam measurement configuration

Here, we will discuss SSB based inter-cell beam measurement configuration firstly. The first issue is how NW configure information which is used for L1 measurement, e.g. SSB location, PCI. In updated WID, it specified that beam measurement and reporting mechanism for inter-cell beam management and inter-cell mTRP will be the same.

Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, a CSI-SSB-ResourceSet configured for L1-RSRP measurement/reporting includes at least a set of SSB indices where PCI indices are associated with the set of SSB indices, respectively. The PCI indices refer to PCIs within the set of PCIs configured for inter-cell beam management or inter-cell multi-TRP. - The additionalInfo associated with SSB(s) with PCI(s) different from the serving cell agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM - Detailed signaling design is up to RAN2 - FFS (to be concluded in RAN1#107-e): Whether the above L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP

From RAN1’s agreement, it’s agreed that SSB indices and PCI indices will be configured. additionalInfo agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM. In item 8.1.2.2, In order to support inter-cell mTRP beam management, RAN1 has send a LS to RAN2[3] to ask RAN2 to help design signaling related to non-serving cell information indication, it’s applied for inter-cell beam management as well, some of the agreements are as follow:

Agreement from RAN1#103-e Agreement The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing NSC information Agreements from RAN1#104-e Agreement NSC information at least includes NSC PCI to support inter-cell multi-DCI multi-TRP operation - FFS: Whether the indication of PCI is implicit or explicit Conclusion Reuse Rel-15/16 QCL rule between the source and target RS/channel for NSC RS/channel. Agreement At least following NSC SSB information are needed in inter-cell MTRP operation - SSB time domain position - SSB transmission periodicity - SSB transmission power FFS: Other NSC information FFS: Whether indication of these information is implicit or explicit Agreements from RAN1#106-e Agreement Introduce a new RRC indicator/signaling (e.g., re-index the NSC) to indicate the NSC information that a TCI state/QCL information is associated with, where the new indicator/signaling is not the exact PCI value - Detailed signaling design is up to RAN2

It shows that SSB related time domain information will be provided by high layer either.

Observation 4: SSB location, index and PCI information will be provided by high layer.

## 2.4 Measurement period for inter-cell measurement

For L1-RSRP on serving cell, 1 or 3 samples is defined, depending on if timeRestrictionForChannelMeasurement is configured or not. timeRestrictionForChannelMeasurement is configured in CSI-ReportConfig which is used for serving cell. We don’t think that it can be used in NSC measurement. We are not sure if there are any other similar signaling defined in inter-cell beam reporting. Therefore, we suggest to define measurement period based on single shot at first stage.

Proposal 3: For inter-cell L1-RSRP measurement, single shot is considered as the baseline for defining measurement period and don’t consider timeRestrictionForChannelMeasurement.

As analyzed before, in order to balance the performance loss and extended measurement, both SSBs inside and outside SMTC can be used for inter-cell measurement. RX beam sweeping factor can be further reduced.

In legacy requirement, P factor is designed to solve this issue when SSB for serving cell measurement(RLM/BFD/L1-RSRP) is overlapped with SMTC and Gap. When performing L1-RSRP for cells with different PCI outside SMTC, it may conflict with the SSB used for serving cell measurement. Therefore, a sharing factor may be needed if such confliction happens. The sharing factor, X, will be in introduced on top of P factor, i.e. X*P.

Proposal 4: For FR2, if SSBs outside SMTC and MG are used for inter-cell L1-RSRP measurement. It will be shared with L1 measurements of serving cell. A sharing factor X is further introduced on top of P factor.

A special case is when SMTC and SSB for NSC L1-RSRP are fully overlapped. inter-cell L1-RSRP can only be performed during SMTC using Rx beam of L3 measurement. As analyzed before, the performance degradation of inter-cell L1-RSRP is expected since no best RX beam will be found. Therefore, we prefer not to define requirement for this case. If requirement is defined, it needs clarify that performance degradation is expected.

Proposal 5: For FR2, when SMTC and SSB for inter-cell L1-RSRP are fully overlapped, no requirement for inter-cell L1-RSRP measurement is expected or clarify that performance degradation is expected.

## 2.5 Cell known or unknown

It’s agreed in last meeting:

- Define known/unknown cell condition for non-serving cell configured for L1-RSRP measurements

The known condition is defined similar with that for HO or SCell activation:

- The UE has sent a valid L3-RSRP measurement report with SSB index

- A cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5.

If a cell is known, then the total measurement delay for inter-cell beam measurement will only consider the delay for L1-RSRP measurements.

We also understand that the known condition will have scheduling restraints on the NW, i.e. NW will schedule L1-RSRP within 5s after L3 measurement. Therefore, the delay requirement for unknown case can also be defined. If a cell is unknown, the total measurement delay will consider extra cell searching time and SSB index acquiring time.

Proposal 6: If a cell is known, the total measurement delay for inter-cell beam measurement will only consider the delay for inter-cell L1-RSRP measurements.

Proposal 7: If a cell is unknown, the total measurement delay will consider extra cell searching time and SSB index acquiring time.

In legacy L3 measurement, TPSS/SSS_sync_intra is used for PSS/SSS detection and cell rough time synchronization. In current inter-cell beam measurement, however, PCI will be provided, therefore, we don’t need to detect PSS/SSS anymore. Besides, the SINR condition is different either. Then the question is what’s the time we need for cell rough time synchronization. It’s similar with the cell search time, Tsearch,  in HO.  For example, one SSB sample may be used for cell timing search.

Proposal 8: If the cell is unknown,  one SSB sample can be used for cell search time.

TSSB_time_index_intra is the time to obtain SSB index by PBCH decoding. Here, since we are performing beam measurement and SSB index is needed for RX beam sweeping as well as beam reporting. Therefore, TSSB_time_index_intra is still needed, the value can’t be re-used since the side condition for L3 measurement and L1 beam measurement is different. The exact value is FFS.

Since it’s intra-frequency case, there are several cases that SSB index deriving time can be skipped. For FR1 TDD and FR2 case, and in FR1 FDD where deriveSSB-IndexFromCell is enabled, SSB index acquiring time can be skipped.

Proposal 9: If the cell is unknown,  further discuss extra time needed for SSB index acquiring.

## 2.5 Nmax

In last meeting, it’s agreed that:

- RAN4 develops UE requirements for at least Nmax = 1, where Nmax is the maximum number of RRC configured PCIs different from the serving cell for measurement/reporting. - RAN4 will further study to specify the requirements for other Nmax value taking RAN1 decision into account

From the WID of FeMIMO, it specified that UE can transmit or receive from only a single cell, therefore, if NW configure UE to measure L1-RSRP for multiple cells with different PCIs, UE will perform the measurement in sequential method. The total delay will be the sum of measurement delay of L1-RSRP for multiple cells.

Proposal 10: If NW configure UE to measure L1-RSRP for multiple TRPs with different PCIs, UE will perform the measurement in sequential method.

# LS from RAN1

Agreement Send an LS to RAN4 to inform and recommend them to investigate the following issue: - On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, the UE behavior when there is overlap for L1-RSRP measurement for SSB associated with serving cell PCI and PCIs different from the serving cell PCI Note: Discussion in UE feature agenda on this issue is not ruled out

The reply LS will depend on the discussion regarding to whether L1-RSRP for inter-cell beam measurement can be performed outside SMTC or not.

If L1-RSRP for inter-cell beam measurement can only be performed inside SMTC, then there will be no overlap case of SSB for serving cell and other cell with different PCI. Since L1-RSRP for serving cell will only be performed outside SMTC.

If L1-RSRP for inter-cell beam measurement can be performed outside SMTC, it’s possible that SSB will be overlapped for serving cell and cells with different PCI. For FR1, UE can perform both L1-RSRP for serving cell and cells with different PCI simultaneously. For FR2, since UE can transmit or receive from only a single cell at one time, RX beams for serving cell and cells with different PCI are different. Therefore, UE can’t measure L1-RSRP for serving cell and cells with different PCI simultaneously.

Proposal 11: The reply LS will depend on the discussion about whether L1-RSRP for inter-cell beam measurement can be performed outside SMTC or not.

# Conclusion

In this contribution, we provide our views regarding inter-cell beam management in FeMIMO:

Observation 1: There is no conclusion about timing offset assumption between serving cell and TRP with different PCI in RAN1.

Observation 2: For FR2, if different RX beams are assumed for L1 and L3 measurement of neighbor cell, more accurate beam quality measurement can be achieved and shorter measurement delay is expected for inter-cell beam measurement. However, measurement delay for L1 serving cell will be impacted and extended.

Observation 3: For FR2, if same RX beam is assumed for L1 and L3 measurement of neighbor cell, L1 serving cell measurement will not be delayed. However, beam quality measurement may not be accurate and measurement delay may be longer for inter-cell beam measurement.

Proposal 1: Both SSBs inside and outside SMTC can be used for L1-RSRP measurement for inter-cell. RX beam sweeping factor can be further reduced.

Proposal 2: If L1-RSRP measurement for inter-cell is performed inside SMTC, it will use the same RX beam of L3 measurement. If L1-RSRP measurement of neighbor cell is performed outside SMTC, different RX can be used.

Observation 4: SSB location, index and PCI information will be provided by high layer.

Proposal 3: For inter-cell L1-RSRP measurement, single shot is considered as the baseline for defining measurement period and don’t consider timeRestrictionForChannelMeasurement.

Proposal 4: For FR2, if SSBs outside SMTC and MG are used for inter-cell L1-RSRP measurement. It will be shared with L1 measurements of serving cell. A sharing factor X is further introduced on top of P factor.

Proposal 5: For FR2, when SMTC and SSB for inter-cell L1-RSRP are fully overlapped, no requirement for NSC L1-RSRP measurement is expected or clarify that performance degradation is expected.

Proposal 6: If a cell is known, the total measurement delay for inter-cell beam measurement will only consider the delay for inter-cell L1-RSRP measurements.

Proposal 7: If a cell is unknown, the total measurement delay will consider extra cell searching time and SSB index acquiring time.

Proposal 8: If the cell is unknown,  one SSB sample can be used for cell search time.

Proposal 9: If the cell is unknown,  further discuss extra time needed for SSB index acquiring.

Proposal 10: if NW configure UE to measure L1-RSRP for multiple TRPs with different PCIs, UE will perform the measurement in sequential method.

Proposal 11: The reply LS will depend on the discussion about whether L1-RSRP for inter-cell beam measurement can be performed outside SMTC or not.

# Reference

[1] R4-2120321, WF on FeMIMO RRM, Samsung

[2] R1-2112762, LS on L1-RSRP measurement behavior when SSBs associated with different PCIs overlap, vivo