3GPP TSG-RAN WG4 Meeting #101BIS-e                                        	R4-2200793

Electronic Meeting, 17th  – 25th January, 2022

Source:	LG Electronics

Title:	TP on update of summary of self-interference analysis for new LTE x bands (xDL/1UL, x=1,2,3,4) and NR 2 bands (2DL/1UL) DC band combinations in Rel-17

Agenda item:			5.19.2

Document for:	Approval

1. Introduction

In this contribution, we provided self- desense analysis results for the new DC band combos in summary Table 5.3-1 and proposed the MSD test configurations in Table 5.3-2.

2. Text Proposal for TR 37.717-11-21

<Start of TP>

5.3	Summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1 band DL/UL & NR 2 bands DL/UL DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_8_n28-n78 | DC_8A-n28 | 4th harmonic into n78 5th harmonic into n78 | 4th & 5th IMDs into n78 | - | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - Same as DC_8_n28-n77A with 2UL_DC_8A_n28A |
|  | DC_8A_n78A | - | 4th IMD into n28 | - | - Same as DC_8_n28-n77A with 2UL_DC_8A_n77A |
| DC_(n)41AA-n78 DC_(n)41CA-n78 DC_(n)41DA-n78 | DC_41A_n78A | - | 4th IMD into n41 | - | B41 and n41 will be operated by synchronous operation. So there is no IMD problem |
| DC_40A_n1-n78 DC_40C_n1-n78 | DC_40A_n1A DC_40C_n1A |  | 4th IMD & 5th IMD into n78 | - | - Same as DC_1A_n40A-n78A with 2UL_DC_1A_n40A |
|  | DC_40A_n78A DC_40C_n78A |  | 4th IMD into n1 | - | -7.6dB by IMD4 |
| DC_3C_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n78A | - | 5th IMD into n1 | - | - Same as DC_3A_n1A-n78A with 2UL_DC_3A_n78A |
| DC_1_n28-n41 | DC_1A_n28A | - | 2nd & 4th IMDs into n41 |  | - Same as DC_1A-41A_n28A with 2UL_DC_1A_n28A |
|  | DC_1A_n41A |  | 2nd & 5th IMDs into n28 |  | - 29.3 dB by IMD2 - 4.5dB by IMD5 |
| DC_3_n28-n41 | DC_3A_n28A |  | 2nd & 3rd IMDs into n41 |  | - Same as DC_3A-41A_n28A with 2UL_DC_3A_n28A |
|  | DC_3A_n41A |  | 2nd & 3rd IMDs into n28 |  | - Same as DC_3A-28A_n41A with 2UL_DC_3A_n41A |
| DC_28_n1-n40 | DC_28A_n1A | - | 4th IMD into n40 | - | - Same as DC_1A_n28A-n40A with 2UL_DC_1A_n28A |
|  | DC_28A_n40A | 3rd harmonic from B28 into n1 | - | - | 3rd harmonic problem was covered in lower DC or CA band combinations |
| DC_7_n40-n78 | DC_7A_n40A | - | - | - | No issue |
|  | DC_7A_n78A |  | 4th IMD into n40 |  | - 9.2 dB by 4th IMD |
| DC_8_n40-n78 | DC_8A_n40A | 4th harmonic from B8 into n78 | 2nd & 3rd IMDs into n78 |  | - 28.7dB by IMD2 |
|  | DC_8A_n78A |  | 2nd IMD into n40 |  | - 27.3 dB by IMD2 |
| DC_1_n3-n77 DC_1_n3-n77(2A) DC_n3-n77_1 DC_n3-n77(2A)_1 | DC_1A_n3A or DC_n3A_1A | 2nd harmonic from B1 or n3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - Same as IMD issues in DC_1A_n3A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_1A_n77A or DC_n77A_1A | - | 2nd , 4th & 5th IMDs into n3 | - | - Same as IMD issues in DC_1A_n3A-n78A |
| DC_8_n3-n77A DC_8_n3-n77(2A) DC_n3-n77A(2A)_8 | DC_8A_n3A DC_n3A_8A | 2nd harmonic from n3 into n77 4th harmonic from B8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_3A_n77A and DC_8A_n77A - Same as DC_3A_n8A-n78A with 2UL_DC_3A_n8A |
|  | DC_8A_n77A DC_n77A_8A | 2nd harmonic from B8 into n3 | 3rd IMD into n3 |  | - Same as DC_3A-8A-n77A with 2UL_DC_8A_n77A |
| DC_11_n28-n77A DC_11_n28-n77(2A) | DC_11A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 |  | - Harmonic problems already covered in CA_n28A_n77A - 17.5 dB by IMD3 - 11.2dB by IMD4 |
|  | DC_11A_n77A | - | 3rd & 4th IMDs into n28 |  | - 15.8 dB by IMD3 - 11.0 dB by IMD4 |
| DC_42A_n3-n28 DC_42C_n3-n28 | DC_42A_n3A DC_42C_n3A | - | - |  | - No issue |
|  | DC_42A_n28A DC_42C_n28A | - | - |  | - No issue |
| DC_42A_n3-n77A DC_42A_n3-n77(2A) DC_42C_n3-n77A DC_42C_n3-n77(2A) | DC_42A_n3A DC_42C_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n3A_n77A - These IMD problem was not impact to n77 due to synchronous TDD operation |
| DC_2_n5-n77 or DC_2A-2A_n5A-n77A | DC_2A_n5A | 2nd harmonic from B2 into n77 5th harmonic from n5 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in DC_2A_n77A or CA_n5A-n77A. - 16.0 dB by IMD3 - 4.1 dB by IMD5 |
|  | DC_2A_n77A | - | 5th IMD into n5 |  | - 3.8 dB by IMD5 |
| DC_13_n2-n77 | DC_13A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n2A-n77A or DC_13A_n77A. - 17.3 dB by IMD3 - 12.4 dB by IMD4 - 4.5 dB by IMD5 |
|  | DC_13A_n77A | - | 3rd IMD into n2 |  | - 16.0 dB by IMD3 |
| DC_66_n2-n77 | DC_66A_n2A | 2nd harmonic from B66 or n2 into n77 | 2nd & 4th IMDs into n77 | - | - Same as DC_2A_n66A-n78A with 2UL_DC_2A_n66A |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 | - | - Same as DC_2A-66A_n78A with 2UL_DC_66A_n78A |
| DC_66_n5-n77 DC_66-66_n5-n77 | DC_66A_n5A | 2nd harmonic from B66 into n77 4th & 5th harmonics from n5 into n77 | 3rd, 4th & 5th IMDs into n77 | - | - These harmonic issue already solved in DC_66A_n77A or DC_5A_n77A - 16.6 dB by IMD3 - 12.1 dB by IMD4 - 3.6 dB by IMD5 |
|  | DC_66A_n77A | - |  | - | - No issue |
| DC_28_n1-n78 | DC_28A_n1A | 5th harmonic from 28 into n78 | 3rd IMD into n78 | - | - Harmonic problems already solved in CA_28A-n78A - Same as DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_28A_n78A | 3rd harmonic from B28 into n1 | 3rd IMD into n1 | - | - 3rd harmonic will be covered in CA_1A-28A - Same as DC_1A-28A_n78A with 2UL_DC_28A_n78A |
| DC_2_n48-n66 | DC_2A_n48A | - | 4th IMD into n66 | - | - Same as DC_2A-66A_n48A with 2UL_DC_2A_n48A |
|  | DC_2A_n66A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
| DC_48_n25-n48 | DC_48A_n25A | - | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_48_n48-n66 | DC_48A_n66A | 2nd harmonic from n66 into n48 | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_66_n25-n48 | DC_66A_n25A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
|  | DC_66A_n48A | - | 2nd IMD into n25 | - | - Same as DC_2A-48A_n66A with 2UL_DC_48A_n66A |
| DC_3_n40-n41C | DC_3A_n40A | 3rd harmonic from B3 into n41 | - | - | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_3A_n41A | - | - | Yes | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n40-n79C | DC_3A_n40A | 2nd harmonic form n40 into n79 | 5th IMD into n79 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n40A |
|  | DC_3A_n79A | - | 5th IMD into n40 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n79A |
| DC_3_n41A-n79C DC_3_n41C-n79A DC_3_n41C-n79C | DC_3A_n41A | 2nd harmonic form n41 into n79 | 2nd & 5th IMDs into n79 | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n41A - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  | DC_3A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n79A |
| DC_8_n40-n41C | DC_8A_n40A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem will be studied in DC_8A_n41A. - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_8A_n41A | - | 5th IMD into n40 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_8_n40-n79C | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n40A |
|  | DC_8A_n79A | - | 4th IMD into n40 | - | - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n79A |
| DC_8_n41A-n79C DC_8_n41C-n79A DC_8_n41C-n79C | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n41A |
|  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD into n41 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n79A |
| DC_39_n40-n41C | DC_39A_n40A | - | 3rd IMD into n41 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_39A_n41A | - | - | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_39_n40-n79C | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD into n79 | - | - The harmonic problem will be treated CA_n40-n79 band combos. - Same as DC_39A_n40A-n79A with 2UL_DC_39A_n40A |
|  | DC_39A_n79A | - | - | - | No issue |
| DC_39_n41A-n79C DC_39_n41C-n79A DC_39_n41C-n79C | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs into n79 | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n41A |
|  | DC_39A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n79A |
| DC_13_n5-n48 | DC_13A_n48A | - | - | - | No issue |
| DC_13_n48-n66 | DC_13A_n48A | - | 3rd IMD into n66 | - | Follow 17.1dB MSD for CA_13A-48A-66A in TS36.101 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 2.8 dB |
| DC_66_n5-n48 | DC_66A_n5A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 3.3 dB |
|  | DC_66A_n48A | - | - | - | No issue |
| DC_11_n3-n28 | DC_11A_n3A | - | 5th IMD into n28 |  | - In rel-16 TR, define as 2.5 dB |
|  | DC_11A_n28A | - | - | - | No issue |
| DC_3_n1-n257 DC_3-3_n1-n257 | DC_3A_n1A | - | - | Yes | No issue |
|  | DC_3A_n257A | - | - | - | No issue |
| DC_7_n1-n257 DC_7-7_n1-n257 | DC_7A_n1A | High order harmonics from B7 or n1 | - | - | No issue |
|  | DC_7A_n257A | - | - | - | No issue |
| DC_3_n78-n258 | DC_3A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n258A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_7A_n78-n258 DC_7C_n78-n258 | DC_7A_n78A DC_7C_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n258A DC_7C_n258A | - | - | - | - No issue |
| DC_28_n78-n258 | DC_28A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_28A_n258A | 5th harmonic from B28 into n78 | - | - | - Harmonic problem already covered in DC_28A_n78A |
| DC_8_n78-n258 | DC_8A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n258A | 4th harmonic from B8 into n78 | - | - | - Harmonic problem already covered in DC_8A_n78A |
| DC_8_n40-n258 | DC_8A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_3_n34-n258 | DC_3A_n34A | 14th & 15th harmonics from B3 into n258 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_3_n41-n258 | DC_3A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_8_n34-n258 | DC_8A_n34A | 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_8_n41-n258 | DC_8A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem already covered in DC_8A_n41A |
| DC_39_n40-n258 | DC_39A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_39_n41A-n258 DC_39_n41C-n258 | DC_39A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_8_n79-n258 | DC_8A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_8A_n258A | 5th harmonic from B8 into n79 | - | - | - Harmonic problem already covered in DC_8A_n79A |
| DC_39_n79A-n258 DC_39_n79C-n258 | DC_39A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_40_n41A-n258 DC_40_n41C-n258 | DC_40A_n41A | 10th harmonic from n41 into n258 11th harmonics from B40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_40A_n258A | - | - | - | - No issue |
| DC_40_n79A-n258 DC_40_n79C-n258 | DC_40A_n79A | 11th harmonics from B40 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_40A_n258A | 2nd harmonic from B40 into n79 |  |  | - These harmonic problem will be covered in DC_40A_n79A |
| DC_41_n79A-n258 DC_41_n79C-n258 | DC_41A_n79A | 10th harmonic from n41 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_41A_n258A | 2nd harmonic from n41 into n79 |  |  | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used |
| DC_8_n77(2A)-n257 | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I | 4th harmonic from B8 into n77 | - | - | - Harmonic problem already covered in DC_8A_n77A |
| DC_11_n77(2A)-n257 | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | - | - | - | No issue |
| DC_28_n77(2A)-n257D | DC_28A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_28A_n257D | 5th harmonic from B28 into n77 | - | - | 5th harmonic problem was solved in DC_28A_n77A |
| DC_2_n7A-n66A DC_2_n7(2A)-n66A | DC_2A_n7A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_12_n7A-n66A DC_12_n7(2A)-n66A | DC_12A_n7A | 3rd harmonic into n66 from B12 | - | - | The 3rd harmonic issue already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | - | - | No issue |
| DC_2_n66A-n78(2A) DC_2_n66(2A)-n78A DC_2_n66(2A)-n78(2A) | DC_2A_n66A | 2nd harmonics into n78 from B2 and n66 | 2nd & 4th IMDs into n78 | - | The 2nd harmonic issue already covered in DC_66A_n78A - Follow same as DC_2A_n66A-n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - For 4th IMD problem, follow same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_12_n66A-n78A DC_12_n66A-n78(2A) DC_12_n66(2A)-n78A DC_12_n66(2A)-n78(2A) | DC_12A_n66A | 2nd harmonic into n78 from n66 5th harmonic into n78 from B12 | 5th IMD into n78 | - | These harmonic issues are already covered in DC_66A_n78A or DC_12A_n78A - 4.1dB by 5th IMD in TR37.716-21-21 |
|  | DC_12A_n78A | 3rd harmonic into n66 from B12 | 3rd IMD into n66 | - | 3rd harmonic issue already covered in DC_12A_n66A - 16.5dB by 3rd IMD in TR37.716-21-21 |
| DC_13_n7-n78 | DC_13A_n7A | - | 2nd & 4th IMDs into n78 | - | - 29.0 dB by 2nd IMD already analyzed in TR37.716-21-21 |
|  | DC_13A_n78A |  | 2nd IMD into n7 | - | - 27.9 dB by 2nd IMD already analyzed in TR37.716-21-21 |
| DC_2_n38-n66 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_66_n38-n66 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_1-3_n3-n77, DC_1-3_n3-n77(2A) or DC_1-3_n3-n78 | DC_1A_n3A | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMDs into n77/n78 | Yes | - Harmonic problems was already covered in DC_3A_n77A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_1A_n77A or DC_1A_n78A |  | 2nd, 4th & 5th IMDs into B3 | Yes | These MSD issues are same as DC_1A-3A_n77A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n3A DC_(n)3AA | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_3A_n77A. - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A or DC_3A_n78A | - | 2nd & 5th IMD into B1 2nd, 4th &5th IMD into n3 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_18_n28-n77 or DC_18_n28-n78 | DC_18A_n28A | 4th & 5th harmonic from B18 and n28 fall into n77/n78 | 5th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_18A_n77A or CA_n28-n77 - 4.0 dB by IMD5 |
|  | DC_18A_n77A or DC_18A_n78A | - | 5th IMD into n77 | - | - 5.7 dB by IMD5 |
| DC_1_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 | - | - 5.0 dB by 5th IMD that follow DC_1A-41A_n3A in TS38.101-3 |
|  | DC_1A_n41A | - | - | Yes | Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_3_n3-n41 | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | 4th IMD into n3 | - | 4th IMD issue already covered by DC_3_n41 in Table 7.3B.2.3.5.1-1 in TS38.101-13 |
| DC_18_n3-n41 | DC_18A_n3A | - | 2nd & 3rd IMDs into B41 | - | 3rd IMD problem already covered in DC_18A-41A_n3A - 29.4 dB for 2nd IMD |
|  | DC_18A_n41A | - | 2nd IMD into n3 |  | - 28.2 dB by IMD2 |
| DC_1-3_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 |  | 5th IMD problem already covered in CA_1A-3A-41A in TS36.101 |
|  | DC_1A_n41A | - | - | - | No issue |
|  | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | - | - | No issue |
| DC_18_n28-n41 | DC_18A_n28A |  | 5th IMD into n41 | - | - 5.1 dB by IMD5 |
|  | DC_18A_n41A |  | 5th IMD into n28 | - | - 4.7 dB by IMD5 |
| DC_1_n41-n77 | DC_1A_n41A | 2nd harmonic from B1 into n77 | 3rd , 4th & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_1_n77 in TS38.101-3 - These IMDs problem follow DC_1_n41-n78 in TS38.101-3 |
|  | DC_1A_n77A | - | 4th & 5th IMDs into n41 | - | - The IMD problem follow DC_1-41_n77 in TS38.101-3 |
| DC_3_n41-n77 | DC_3A_n41A | 2nd harmonic from B3 into n77 | 3rd & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_3_n77 in TS38.101-3 - These IMDs problem follow DC_3_n41-n78 in TS38.101-3 |
|  | DC_3A_n77A |  | 5th IMD into n41 |  | - The IMD problem follow DC_3-41_n77 in TS38.101-3 |
| DC_18_n41-n77 or DC_18_n41-n78 | DC_18A_n41A | 4th harmonic from B18 into n78 | 2nd & 4th IMDs into n77/n78 3rd IMD into n77 | - | - 30.1 dB by IMD2 -16.4 dB by IMD3 - 9.1 dB by IMD4 |
|  | DC_18A_n77A or DC_18A_n78A |  | 2nd IMD into n4177/n78 3rd IMD into n41 in DC_18A_n77A | - | - 28.5 dB by IMD2 |
| DC_41_n3-n41 | DC_41A_n3A | - | 4th IMD into n41 | Yes | For TDD band, the IMD problem is not impact to the 3rd Rx band |
|  | DC_41A_n41A | - | - | Yes | No issue |
| DC_41_n28-n41 | DC_41A_n28A | - | - | - | No issue |
|  | DC_41A_n41A | - | - | - | No issue |
| DC_41_n41-n77 or DC_41_n41-n78 | DC_41A_n41A | - | - | - | No issue |
|  | DC_41A_n77A or DC_41A_n78A | - | 4th IMD into n41 |  | For TDD band, the IMD problem is not impact to the 3rd Rx band |
| DC_(n)5AA-n77A | DC_(n)5AA | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD into n77 |  | 4th & 5th harmonic issues were covered in DC_5A_n77A The 4th & 5th IMD problem will not be impacted due to only single UL switching operation. |
|  | DC_5A_n77A |  | 4th & 5th IMDs |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A |
| DC_12_n5-n77 | DC_12A_n5A | 4th & 5th harmonics from n5 into n77 | 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - 5.4 dB by IMD5 |
|  | DC_12A_n77A |  | 5th IMD |  | - 4.8 dB by IMD5 |
| DC_14_n5-n77 | DC_14A_n5A | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - 10.6 dB by IMD4 -5.1 dB by IMD5 |
|  | DC_14A_n77A |  | 5th IMD |  | - 4.4 dB by IMD5 |
| DC_2_n71-n78 DC_2-2_n71-n78 | DC_2A_n71A | 2nd harmonic from B2 into n78 5th harmonic from n71 into n78 | 3rd IMD into n78 | - | These harmonics issues were already solved in DC_2_n78 or CA_n71_n78 - 8.0 dB by IMD3 |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_7_n71-n78 | DC_7A_n71A | 5th harmonic from n71 into n78 | 4th IMD into n78 | - | - The harmonic issue was already solved in CA_n71_n78 - 9.7 dB by IMD4 |
|  | DC_7A_n78A | - | 5th IMD into n71 | - | - 3.9 dB by IMD5 |
| DC_66_n71-n78 | DC_66A_n71A | -2nd harmonic from B66 into n78 5th harmonic from n71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_66_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_2_n38-n71 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n71A | - | 2nd IMD into n38 | - | 29.2 dB of DC_2A_n41A-n71A can be reused in TS38.101-3 |
| DC_66_n38-n71 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n71A | - | - | - | No issue |
| DC_2_n38-n78 | DC_2A_n38A | 2nd harmonic from B2 | 3rd IMD into n78 |  | Harmonic issue were covered in DC_2A_n78A - 14.8 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_66_n38-n78 | DC_66A_n38A | 2nd harmonic from B66 | 3rd IMD into n78 | - | Harmonic issue were covered in DC_66A_n78A - 15.0 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_71_n38-n78 | DC_71A_n38A | 5th harmonic from B71 into n78 | 2nd & 4th IMDs into n78 |  | Harmonic issue were covered in DC_71A_n78A - 29.1 dB by IMD2 -10.3 dB by IMD4 |
|  | DC_71A_n78A |  | 2nd IMD into n38 |  | - 28.7 dB by IMD2 |
| DC_5_n66-n77 | DC_5A_n66A | -2nd harmonic from B66 into n77 4th & 5th harmonics from B5 into n77 | 3rd , 4th & 5th IMDs into n77 |  | - Harmonics issue will be solved in DC_66A_n77A or DC_5A_n77A - Reuse 16.6dB MSD of DC_5A_n66A-n78A with dual uplink DC_5A_n66A in TS38.101-3 |
|  | DC_5A_n77A | - | 3rd IMDs into n66 |  | - Reuse 13.2dB MSD of DC_5A_n66A-n77A with dual uplink DC_5A_n77A in TS38.101-3 |
| DC_5_n66-n78 | DC_5A_n66A | -2nd harmonic from B66 into n78 4th harmonic from B5 into n78 | 3rd & 5th IMDs into n78 |  | Harmonic issue were covered in DC_66A_n78A or DC_5A_n78A - Same MSD levels can be reused in DC_66_n5-n77 in this TR |
|  | DC_5A_n78A | - | 3rd IMD into n66 |  | 13.2 dB by DC_5A-66A_n78A can be reused in TS38.101-3 |
| DC_71_n66-n78 | DC_71A_n66A | -2nd harmonic from n66 into n78 5th harmonic from B71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_2_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_71A_n78A |  | 3rd IMD into n66 | - | - 15.5 dB by IMD3 |
| DC_5_n38-n66 | DC_5A_n38A | - | - | - | No issue |
|  | DC_5A_n66A |  | 2nd & 3rd IMD into n38 |  | - 28.9 dB by IMD2 - 16.4 dB by IMD3 |
| DC_71_n38-n66 | DC_71A_n38A | - | - | - | No issue |
|  | DC_71A_n66A | - | - | - | No issue |
| DC_2_n2-n38 or DC_2_n2-n41 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n38A or DC_2A_n41A | - | - | - | No issue |
| DC_66_n2-n38 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n38A | - | - | - | No issue |
| DC_12_n2-n38 or DC_12_n2-n41 | DC_12A_n2A |  | 2nd IMD into n38/n41 | - | - 28.7 dB by IMD2 |
|  | DC_12A_n38A or DC_12A_n41A | - | - | - | No issue |
| DC_71_n2-n41 | DC_71A_n2A | 4th harmonic from B71 into n41 | 2nd IMD into n41 | - | Harmonic issue already covered in DC_71_n41 - 29.2dB MSD by DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_71A_n41A | 3rd harmonic from B71 into n2 | 2nd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 26.0dB MSD by DC_2A-71A_n38A can be reused in TS38.101-3 |
| DC_2_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 |  | DC_2A_n2A only allowed single UL switched mode. |
|  | DC_2A_n78A |  | 2nd,4th & 5th IMDs into n2 |  | The IMD problem already solved in DC_2A_n78A in TS38.101-3 |
| DC_7_n2-n78 | DC_7A_n2A | 2nd harmonic from n2 into n78 | 5th IMD into n78 |  | Harmonic issue already covered in DC_2_n78 4.2dB MSD of DC_2A_n7A-n78A can be reused in TS38.101-3 |
|  | DC_7A_n78A | - | 4th IMD into n2 |  | 8.6dB MSD of DC_2A-7A_n78A can be reused in TS38.101-3 |
| DC_71_n2-n78 | DC_71A_n2A | 2nd harmonic from n2 into n78 5th harmonic from B71 into n78 | 3rd IMD into n78 |  | Harmonic issue already covered in DC_2_n78 or DC_71_n78 - 3rd IMD can be reused in DC_2_n71-n78 in this TR |
|  | DC_71A_n78A | 3rd harmonic from B71 into n2 | 3rd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 16.5dB MSD of DC_2A-71A_n78A can be reused in TS38.101-3 |
| DC_2_n66-n71 DC_2-2_n66-n71 | DC_2A_n66A | - | - | - | No issue |
|  | DC_2A_n71A | - | - | - | No issue |
| DC_7_n66-n71 | DC_7A_n66A | - | - | - | No issue |
|  | DC_7A_n71A | - | - | - | No issue |
| DC_66_n66-n71 | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
|  | DC_66A_n71A |  | 4th IMD into n66 |  | 5.0dB MSD for DC_66A_n71A can be reused in TS38.101-3 |
| DC_2_n2-n71 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n71A | 3rd harmonic from n71 into B2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_7_n2-n71 | DC_7A_n2A |  | 2nd & 5th IMDs into n71 |  | 28.7dB MSD for 2nd IMD of DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_7A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_66_n2-n71 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n2-n66 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n66A |  | 3rd & 5th IMDs into n2 |  | 20.0dB MSD of for 3rd IMD DC_2A_n66A can be reused in TS38.101-3 |
| DC_7_n2-n66 | DC_7A_n2A | - | - | - | No issue |
|  | DC_7A_n66A | - | - | - | No issue |
| DC_66_n2-n66 | DC_66A_n2A |  | 5th IMD into n66 |  | 4.0dB MSD of for 5th IMD DC_2A_n66A can be reused in TS38.101-3 |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_71_n2-n66 | DC_71A_n2A | - | - | - | No issue |
|  | DC_71A_n66A | 3rd harmonic from B71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n66-n77 or DC_2-2_n66-n77 | DC_2A_n77A | - | 2nd , 4th & 5th IMDs into n66 | - | - 29.2 dB for 2nd IMD - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2_n66-n77 DC_2-2_n66-n77 | DC_2A_n66A | 2nd harmonic from B2 or n66 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in DC_2_n77 or CA_n66-n77 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_13_n66-n77 | DC_13A_n77A |  | 3rd IMD into n66 |  | - 17.1dB MSD for DC_13-66A_n48A can be reused in TS38.101-3 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 | - | -Harmonic issue will be solved in CA_n66-n77 or DC_13_n77 - 16.4 dB by IMD3 - 11.7 dB by IMD4 - 4.4 dB by IMD5 |
| DC_66_n66-n77 | DC_66A_n77A | - | 2nd & 5th IMDs into B66 | - | - These IMDs problems already covered in DC_66A_n77A |
| DC_11_n3-n77 or DC_11_n3-n77(2A) | DC_11A_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 | - | Harmonic issues already solved in DC_3_n77 - 10.8 dB by IMD4 |
|  | DC_11A_n77A |  | 2nd & 5th IMDs into n3 |  | - 29.0 dB by IMD2 |
| DC_2_n25-n66 | DC_2A_n25A | - | 5th IMD into n66 |  | - Reuse 4.0 dB MSD in DC_2A-66A_n25A in TS38.101-3 |
|  | DC_2A_n66A |  | 3rd & 5th IMDs into n25 |  | Reuse 20.0 dB MSD in DC_2A_n66A for 3rd IMD in TS38.101-3 |
| DC_7_n25-n66 DC_7-7_n25-n66 | DC_7A_n25A | - | - | - | No issue |
|  | DC_7A_n66A | - | - | - | No issue |
| DC_13_n25-n66 | DC_13A_n25A | - | 4th IMD into n66 | - | - Reuse 7.2 dB MSD in DC_13A-66A_n2A in TS38.101-3 |
|  | DC_13A_n66A | - | 4th IMD into n25 | - | - Reuse 6.2 dB MSD in DC_2A-13A_n66A in TS38.101-3 |
| DC_66_n25-n66 | DC_66A_n25A | - | 3rd & 5th IMDs into n66 | - | - Reuse 23.0 dB & 4.0 dB MSD in DC_66A_n25A in TS38.101-3 |
| DC_7_n66-n77 DC_7-7_n66-n77 | DC_7A_n66A | 2nd harmonic from n66 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic issue already covered in CA_n66-n77 - Reuse 16.0dB MSD for 3rd IMD in DC_7A_n66A-n78A in TS38.101-3 |
|  | DC_7A_n77A | - | 4th IMD into n66 | - | - Reuse 8.7dB MSD for 4th IMD in DC_7A-66A_n78A in TS38.101-3 |
| DC_21_n28-n77 | DC_21A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 | - | - 17.3 dB in n77 by IMD3 |
|  | DC_21A_n77A | - | 3rd & 4th IMDs into n28 | - | - Reuse 16.9 dB MSD for 3rd IMD in DC_21A-28A_n77A in TS38.101-3 |
| DC_1_n28-n79 | DC_1A_n28A | 6th & 7th harmonic from n28 into n79 | 3rd & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 14.9dB in n79 by IMD3 |
|  | DC_1A_n79A | - | 3rd & 4th IMDs into n28 |  | - Reuse 15.2 dB/ 10.0 dB MSD for 3rd /4th IMD in DC_1A-28A_n79A in TS38.101-3 |
| DC_3_n28-n79 | DC_3A_n28A | 6th & 7th harmonic from n28 into n79 | 4th & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 9.4dB in n79 by IMD4 |
|  | DC_3A_n79A | - | 4th IMD into n28 | - | - Reuse 10.3 dB MSD for 4th IMD in DC_3A-28A_n79A in TS38.101-3 |
| DC_21_n28-n79 | DC_21A_n28A | 6th & 7th harmonic from n28 into n79 | 4th & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 6.3 dB in n79 by IMD4 |
|  | DC_21A_n79A | - | 5th IMD into n28 | - | - 2.8 dB in n28 by IMD5 |
| DC_21_n28-n78 | DC_21A_n28A | 5th harmonic from n28 into n78 | 3rd & 4th IMDs into n78 |  | - Harmonic issue already covered in CA_n28-n78 - same MSD value and test configuration of DC_21A_n28A-n77A in this TR |
|  | DC_21A_n78A | - | 3rd IMD into n28 | - | - Reuse 16.9 dB MSD for 3rd IMD in DC_21A-28A_n77A in TS38.101-3 |
| DC_2_n2-n77 | DC_2A_n77A | - | 2nd & 4th IMD IMDs into n2 | - | - Reuse 26 dB/ 8dB MSD for 2nd / 4th IMD in DC_2A_n78A in TS38.101-3 |
| DC_5_n2-n77 | DC_5A_n2A | 2nd harmonic from n2 and 4th & 5th harmonics from B5 into n77 | 3rd & 5th IMDs into n77 | - | - Harmonic issue already covered in CA_n2-n77 or DC_5_n77 - 16.0dB MSD by 3rd IMD of DC_2A_n5A-n77A in this TR can be used |
|  | DC_5A_n77A | - | - | - | No issue |
| DC_13_n5-n77 | DC_13A_n77A | - | 5th IMD into n5 | - | - 4.5 dB in n5 by IMD5 |
| DC_66-66_n2-n77 | DC_66A_n2A | 2nd harmonic from B66 or n2 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in CA_n2-n77 or DC_66_n77 - Reuse 29.4 dB/ 8.9dB MSD for 2nd / 4th IMD in DC_2A_n66A-n78A in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 |  | - Reuse 32.1 dB/ 9.1dB/ 2.1dB MSD for 2nd / 4th / 5th IMD in DC_2A-66A_n78A in TS38.101-3 |
| DC_5_n5-n77 | DC_5A_n77A | - | 4th & 5th IMDs into n5 |  | - Reuse 8.3dB MSD for 4th IMD in DC_5A_n78A in TS38.101-3 |
| DC_2-66_n66-n77 | DC_2A_n66A | 2nd harmonic from B2 or n66 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in DC_2_n77 or CA_n66-n77 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n77A | - | 2nd , 4th & 5th IMDs into n66 | - | - 29.2 dB for 2nd IMD - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 |  | - Reuse 32.1 dB/ 9.1dB/ 2.1dB MSD for 2nd / 4th / 5th IMD in DC_2A-66A_n78A in TS38.101-3 |
| DC_5-66_n66-n77 | DC_5A_n66A | -2nd harmonic from B66 into n77 4th & 5th harmonic from B5 into n77 | 3rd , 4th & 5th IMDs into n77 |  | Harmonic issue were covered in DC_66A_n77A or DC_5A_n77A - Same MSD levels can be reused in DC_66_n5-n77 in this TR |
|  | DC_5A_n77A |  | 3rd IMD into B66 or n66 |  | - Reuse 13.2 dB MSD for 3rd IMD in DC_5A-66A_n78A in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd & 5th IMDs into n66 | - | - These IMD problem already solved in DC_66A_n77A |
| DC_5-66_n5-n77 | DC_5A_n77A |  | 3rd IMD into B66 or n66 4th & 5th IMDs into n5 |  | - Reuse 13.2 dB MSD for 3rd IMD in DC_5A-66A_n78A in TS38.101-3 - Reuse 8.3 dB MSD for 4th IMD in DC_5A_n78A in TS38.101-3 |
|  | DC_66A_n5A | -2nd harmonic from B66 into n77 4th & 5th harmonic from B5 into n77 | 3rd , 4th & 5th IMDs into n77 2nd & 5th IMD into B5 |  | Harmonic issue were covered in DC_66A_n77A or DC_5A_n77A - Same MSD levels can be reused in DC_66_n5-n77 in this TR - 30dB MSD for 2nd IMD in DC_5A_n66A in TS38.101-3 can be reused |
|  | DC_66A_n77A | - | - | - | No issue |
| DC_28_n7-n258 | DC_28A_n7A DC_28A_n7B | 10th harmonic from n7 into n258 | - | - | No issue |
|  | DC_28A_n258A/G/H/I | - | - | - | No issue |
| DC_30_n5-n77 | DC_30_n5 | 4th & 5th harmonic from n5 into n77 | 3rd IMD into n77 | - | 4th & 5th Harmonic issues already covered in DC_5A_n77A. - 16.1 dB by 3rd IMD |
|  | DC_30_n77 | - | 3rd & 5th IMDs into n5 | - | - 15.3 dB by 3rd IMD |
| DC_3-3_n8-n78 | DC_3_n8 | 2nd harmonic from B3 into n78 4th harmonic from n8 into n78 | 3rd & 5th IMDs into n78 4th & 5th IMDs into B3 | - | The 3rd IMD problem, follow MSD (16.3dB) for DC_3A_n8A-n78A. The 4th &5th IMDs problem already covered in DC_3A_n8A. |
|  | DC_3_n78 | - | - | - | No issue |
| DC_7-7_n8-n78 | DC_7_n8 | 4th harmonic from n8 into n78 | 2nd & 4th IMDs into n78 |  | 4th Harmonic issues already covered in DC_8A_n78A. The 2nd IMD problem, follow MSD (28.5dB) for DC_7A_n8A-n78A. |
|  | DC_7_n78 | - | 2nd IMD into n8 |  | The 2nd IMDs problem, follow MSD (29.7dB) for DC_7A_n8A-n78A. |
| DC_3_n1-n8 DC_3-3_n1-n8 | DC_3_n1 | - | - | Yes | - Small freq. gap was covered in Table 7.3.1A-0bA in TS38.101-3 |
|  | DC_3_n8 | - | - | - | No issue |
| DC_7_n1-n8 DC_7-7_n1-n8 | DC_7_n1 | - | 5th IMD into n8 | - | - Reuse 4.5dB by 5th IMD from DC_1A-28A_n7A |
|  | DC_7_n8 | 3rd harmonic from n8 into B7 | - | - | The harmonic issue already covered in DC_7A_n8A. |
| DC_n1-n28_3 | DC_n1A_3A | - | - | - | No issue |
|  | DC_n28A_3A | 3rd harmonic from n28 into n1 | 4th IMD | - | - Harmonic problem already covered in DC_1A_n28A - 11.0dB by 4th IMD problem same as DC_1A-3A_28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3_n28-n78 | DC_3C_n78A | - | - | - | No issue |
| DC_3_n7-n78 | DC_3C_n78A | - | - | - | No issues |
| DC_8_n1-n40 | DC_8A_n1A | - | - | - | No issues |
|  | DC_8A_n40A | - | 5th IMD into n1 | - | - 3.3dB by 5th IMD |
| DC_3_n1-n38 | DC_3A_n1A | - | - | - | No issues |
|  | DC_3A_n38A | - | - | - | No issues |
| DC_3_n1-n41 | DC_3A_n1A | - | 5th IMD into n41 | - | - 5.0 dB by 5th IMD that follow DC_1A-41A_n3A in TS38.101-3 |
|  | DC_3A_n41A | - | - | - | No issues |
| DC_1_n3-n79 | DC_1A_n3A |  | 5th IMD | Yes | - 4.7dB by 5th IMD that follow DC_3A_n1A-n79A in TR37.716-21-21 |
|  | DC_1A_n79A |  |  | Yes | Cross isolation issue already solved. |
| DC_8_n3-n79 | DC_8A_n3A |  | 3rd & 5th IMDs | - | - 15.7dB by 3rd IMD |
|  | DC_8A_n79A |  | 4th IMD | - | -8.8dB by 4th IMD that follow DC_3A-8A_n79A in TS38.101-3 |
| DC_8_n28-n79 | DC_8A_n28A | 5th harmonic from B8 into n79 | 5th IMD | - | - Harmonic problem already covered in DC_8A_n79A - 0dB by 5th IMD |
|  | DC_8A_n79A |  | 5th IMD | - | - 3.9dB by 5th IMD |
| DC_8_n77-n79 DC_8_n77(2A)-n79 | DC_8A_n77A | 5th harmonic from B8 into n79 | 2nd , 4th & 5th IMDs |  | - Harmonic problem already covered in DC_8A_n79A - RAN4 agreed not to consider simultaneous Rx/Tx capability for the CA band combinations with n77 and n79. |
|  | DC_8A_n79A | - | 2nd IMD |  | - RAN4 agreed not to consider simultaneous Rx/Tx capability for the CA band combinations with n77 and n79. |
| DC_8_n77-n257D/G/H/I DC_n77-n257A/G/H/I/J/K/L/M_8 DC_n77(2A)-n257A/G/H/I/J/K/L/M_8 | DC_8A_n77A DC_n77A_8A | - | - |  | No issues |
|  | DC_8A_n257D/G/H/I DC_n257A_8A | - | - |  | No issues |
| DC_11_n77-n257D/G/H/I | DC_11A_n77A | - | - |  | No issues |
|  | DC_11A_n257D/G/H/I | - | - |  | No issues |
| DC_2_n12-n77 DC_2-2_n12-n77 | DC_2A_n12A | 2nd harmonic from B2 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A - 16.0 dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_2A_n77A | - | - | - | No issues |
| DC_7_n12-n77 | DC_7A_n12A | - | 3rd & 4th IMDs | - | - 15.2 dB by 3rd IMD |
|  | DC_7A_n77A | - | 2nd IMD | - | - 30.8dB MSD by 2nd IMD from DC_7A-12A_n78A can be reused |
| DC_66_n12-n77 | DC_66A_n12A | 2nd harmonic from B66 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A - 16.0 dB MSD by 3rd IMD from CA_n12-n66-n77 can be reused |
|  | DC_66A_n77A |  | 3rd IMD | - | - 15.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_12_n41-n66 | DC_12A_n41A | 3rd harmonic from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 2nd IMD | - | Edge IMD2 regions will be impact in n41 - 28.7dB MSD by 2nd IMD from DC_12A_n2A-n41A can be reused |
| DC_12_n66-n77 | DC_12A_n66A | 2nd harmonic from n66 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A - 16.0 dB MSD by 3rd IMD from CA_n12-n66-n77 can be reused |
|  | DC_12A_n77A | 3rd harmonic from B12 into n66 | 3rd & 4th IMDs | - | - Harmonic problem already covered in DC_12A_n66A - 13.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_71_n66-n77 | DC_71A_n66A | 2nd harmonic from n66 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A or DC_71A_n77A - 15.9dB MSD by 3rd IMD from CA_n66-n71-n77 can be reused |
|  | DC_71A_n77A | - | 3rd & 4th IMDs | - | - 15.5dB MSD by 3rd IMD from DC_71A_n66A-n78A can be reused |
| DC_2_n71-n77 DC_2-2_n71-n77 | DC_2A_n71A | 2nd harmonic from B2 into n77 5th harmonic from n71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0dB MSD by 3rd IMD from DC_2A_n71A-n78A can be reused |
|  | DC_2A_n77A | - | - | - | No issues |
| DC_7_n71-n77 | DC_7A_n71A | 5th harmonic from n71 into n77 | 3rd & 4th IMDs | - | - Harmonic problem already covered in DC_71A_n77A - 15.2dB by 3rd IMD |
|  | DC_7A_n77A | - | - | - | No issues |
| DC_66_n71-n77 | DC_66A_n71A | 2nd harmonic from B66 into n77 5th harmonic from n71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A or DC_71A_n77A - 15.9dB MSD by 3rd IMD from CA_n66-n71-n77 can be reused |
|  | DC_66A_n77A | - | 3rd IMD | - | - 15.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_5_n2-n41 | DC_5A_n2A | 3rd harmonic from B5 into n41 | 2nd IMD | - | - Harmonic problem already covered in DC_5A_n41A - 29.7dB by 2nd IMD |
|  | DC_5A_n41A | - | - | - | No issues |
| DC_5_n2-n66 | DC_5A_n2A | - | 4th IMD | - | - 7.2dB MSD by 4th IMD from CA_n2-n5-n66 can be reused |
|  | DC_5A_n66A | - | - | - | No issues |
| DC_12_n2-n66 | DC_12A_n2A | 3rd harmonic from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 4th IMD | - | - 0dB MSD in DC_2A-12A_n66A in TS38.101-3 |
| DC_7_n2-n77 | DC_7A_n2A | 2nd harmonic from n2 into n77 | 5th IMD |  | - Harmonic problem already covered in DC_2A_n77A - 4.2dB MSD by 5th IMD from DC_2A_n7A-n78A can be reused |
|  | DC_7A_n77A | - | 4th IMD | - | - 8.6dB MSD by 4th IMD from DC_2A-7A_n78A can be reused |
| DC_12_n2-n77 | DC_12A_n2A | 2nd harmonic from n2 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A - 16.0dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_12A_n77A | - | 3rd & 4th IMDs | - | - 16.5dB MSD by 3rd IMD from DC_2A-12A_n77A can be reused |
| DC_71_n2-n77 | DC_71A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0 dB MSD by 3rd IMD from DC_2A_n71A-n78A |
|  | DC_71A_n77A | - | 3rd & 4th IMDs | - | - 16.5dB MSD by 3rd IMD from DC_2A-71A_n78A can be reused |
| DC_5_n2-n78 | DC_5A_n2A | 2nd harmonic from n2 into n78 4th harmonic from B5 into n78 | 3rd IMD | - | - Harmonic problem already covered in DC_2A_n78A or DC_5A_n78A - 16.0dB MSD by 3rd IMD from DC_2A_n5A-n77A can be reused |
|  | DC_5A_n78A |  | 3rd IMD |  | - 16.5dB MSD by 3rd IMD from DC_2A-5A_n77A can be reused |
| DC_12_n2-n78 | DC_12A_n2A | 2nd harmonic from n2 into n78 | 3rd IMD | - | - Harmonic problem already covered in DC_2A_n77A - MSD will follow from DC_12_n2-n77 |
|  | DC_12A_n78A | - | 3rd IMD | - | - MSD will follow from DC_12_n2-n77 |
| DC_2-2_n41-n66 | DC_2A_n41A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
| DC_2-2_n41-n71 | DC_2A_n41A | - | 2nd & 5th IMDs | - | - 28.7dB MSD by 2nd IMD from DC_2A_n41A-n71A can be reused |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 | - | - | - Harmonic problem already covered in DC_71A_n41A |
| DC_2-2_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 or n66 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2-5_n2-n41 | DC_2A_n2A | - | - | - | No issues when RAN4 only consider the single switched uplink transmission |
|  | DC_2A_n41A | - | - | - | No issue |
|  | DC_5A_n2A | 3rd harmonics from n2 into n41 | 2nd IMD into n41 |  | - Harmonic issue already covered in DC_2_n41 - 29.7dB by 2nd IMD |
|  | DC_5A_n41A | - | - | - | No issues |
| DC_2-12_n2-n41 | DC_2A_n2A | - | - | - | No issues when RAN4 only consider the single switched uplink transmission |
|  | DC_2A_n41A | - | 2nd & 5th IMDs into B12 | - | - 28.7dB MSD by 2nd IMD from DC_2A-12A_n41A can be reused |
|  | DC_12A_n2A | 3rd harmonics from n2 into n41 | 2nd IMD into n41 |  | - Harmonic issue already covered in DC_2_n41 - 28.7 dB by IMD2 from DC_12A_n2A-n41A can be reused |
|  | DC_12A_n41A | - | 2nd IMD into B2 and n2 | - | - 26.0dB MSD by 2nd IMD from DC_2A-12A_n41A can be reused |
| DC_2-66_n2-n41 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n41A | - | - | - | No issues |
|  | DC_66A_n2A | - | - | - | No issues |
|  | DC_66A_n41A |  | 4th IMD into B2 and n2 | - | - 11.0dB MSD by 2nd IMD from DC_2A-66A_n41A can be reused |
| DC_2-71_n2-n41 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n41A | - | 2nd & 5th IMDs into B71 | - | - 28.7dB MSD by 2nd IMD from DC_2A_n41A-n71A can be reused |
|  | DC_71A_n2A | 4th harmonic from B71 into n41 | - | - | - Harmonic problem already covered in DC_71A_n41A |
|  | DC_71A_n41A | 3rd harmonic from B71 into B2/n2 | 2nd IMD into B2 and n2 | - | - Harmonic issue already covered in DC_2_n71 - 26.0dB MSD by DC_71A_n2A-n41A can be reused in TS38.101-3 |
| DC_2-5_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_5A_n2A | - | 4th IMD into n66 | - | - 7.2dB MSD by 4th IMD from CA_n2-n5-n66 can be reused |
|  | DC_5A_n66A | - | - | - | No issues |
| DC_2-7_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_7A_n2A | - | - | - | No issues |
|  | DC_7A_n66A | - | - | - | No issues |
| DC_2-12_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_12A_n2A | 3rd harmonics from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 4th IMD into B2 and n2 |  | - 0dB MSD in DC_2A-12A_n66A in TS38.101-3 |
| DC_2-71_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_71A_n2A | - | - | - | No issues |
|  | DC_71A_n66A | 3rd harmonics from B71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-7_n2-n71 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n71A | - | - | - | No issues |
|  | DC_7A_n2A | - | 2nd & 5th IMDs into n71 | - | - 28.7dB MSD for 2nd IMD of DC_7A_n2A-n71A can be reused in TS38.101-3 |
|  | DC_7A_n71A | 3rd harmonics from n71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-66_n2-n71 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n71A | - | - | - | No issues |
|  | DC_66A_n2A | - | - | - | No issues |
|  | DC_66A_n71A | 3rd harmonics from B71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-7_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A | - | 5th IMD into B7 |  | - 3.4dB MSD for 5th IMD of DC_2A-7A_n77A can be reused in TS38.101-3 |
|  | DC_7A_n2A | 2nd harmonic from n2 into n77 | 5th IMD into n77 |  | - Harmonic problem already covered in DC_2A_n77A - 4.2dB MSD for 5th IMD of DC_2A_n7A-n78A can be reused in TS38.101-3 |
|  | DC_7A_n77A | - | 4th IMD into B2 and n2 | - | - 8.6dB MSD for 4th IMD of DC_2A-7A_n77A can be reused in TS38.101-3 |
| DC_2-12_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A | - |  |  | No issues |
|  | DC_12A_n2A | 2nd harmonic from n2 into n77 | 3rd , 4th & 5th IMDs into n77 |  | - Harmonic problem already covered in DC_2A_n77A - 16.0dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_12A_n77A | - | 3rd & 4th IMDs into B2 and n2 | - | - 16.5dB MSD by 3rd IMD from DC_2A-12A_n77A can be reused |
| DC_2-71_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A |  |  |  | No issues |
|  | DC_71A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0dB MSD |
|  | DC_71A_n77A | - | 3rd & 4th IMDs into B2and n2 | - | - 16.5dB MSD by 3rd IMD from DC_2A-71A_n78A can be reused |
| DC_2-5_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 | - | - Harmonic problem already covered in DC_2A_n78A - Only switched uplink transmission is allowed |
|  | DC_2A_n78A |  | 5th IMD into B5 | - | - 3.8 dB by 5th IMD from DC_2A-5A_n77A can be reused |
|  | DC_5A_n2A | 2nd harmonic from n2 into n78 4th harmonic from B5 into n78 | 3rd IMD into n78 | - | - Harmonic problem already covered in DC_2A_n78A or DC_5A_n78A - 16.0dB MSD by 3rd IMD from DC_2A_n5A-n77A can be reused |
|  | DC_5A_n78A |  | 3rd IMD into B2 and n2 |  | - 16.5dB MSD by 3rd IMD from DC_2A-5A_n77A can be reused |
| DC_2-66_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 | - | - Harmonic problem already covered in DC_2A_n78A - Only switched uplink transmission is allowed |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n2A | 2nd harmonic from n2 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n78A |  | 2nd, 4th & 5th IMDs into B2 and n2 |  | - 32.1dB MSD by 2nd IMD from DC_2A-66A_n78A can be reused |
| DC_66_n2-n78 | DC_66A_n2A | 2nd harmonic from n2 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n78A |  | 2nd, 4th & 5th IMDs into B2 and n2 |  | - 32.1dB MSD by 2nd IMD from DC_2A-66A_n78A can be reused |
| DC_38_n3-n78 | DC_38A_n3A | 2nd harmonic from n3 into n78 | 3rd IMD into n78 |  | - Harmonic issue already covered in DC_3_n78 - 16.1dB MSD by 3rd IMD from DC_3A_n7A-n78A can be reused |
|  | DC_38A_n78A |  | 3rd & 4th IMDs into n3 |  | - 17.6dB MSD by 3rd IMD from DC_3A-7A_n78A can be reused |
| DC_1_n38-n78 | DC_1A_n78A | - | 4th IMD into n38 |  | - 9.1dB MSD by 4th IMD from DC_1A-7A_n78A can be reused |
| DC_3_n38-n78 | DC_3A_n78A | - | - |  | No issues |
| DC_7_n38-n78 | - | - | - |  | No issues |
| DC_20_n38-n78 | DC_20A_n78A | 3rd harmonic from B20 into n38 | 2nd IMD into n38 | - | - Harmonic issue already covered in DC_20_n38 - 30.8dB MSD by 2nd IMD from DC_7A-20A_n78A can be reused |
| DC_1_n28-n75 | DC_1A_n28A | 2nd harmonic from n28 into n75 | - | - | - Harmonic issue already covered in DC_28_n75 |
| DC_3_n28-n75 | DC_3A_n28A | 2nd harmonic from n28 into n75 | 5th IMD into n75 | - | - Harmonic issue already covered in DC_28_n75 - 3.3dB by 5th IMD |
| DC_20_n8-n78 | DC_20A_n8A | 4th harmonic from B20 or n8 into n78 | 4th IMD into n78 |  | - Harmonic issue already covered in DC_20_n78 or DC_8_n78 - 10.3 dB reuse by 4th IMD of DC_8A_n28A-n78A |
|  | DC_20A_n78A | - | 4th IMD into n8 |  | - 12.1 dB reuse by 4th IMD of DC_8A-20A_n78A |
| DC_1_n3-n8 | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n8A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
| DC_1_n8-n77 DC_1_n8-n77(2A) DC_n8-n77_1 DC_n8-n77(2A)_1 | DC_1A_n8A or DC_n8A_1A |  | 3rd IMD into n77 | - | - 14.9 dB MSD by 3rd IMD of DC_1A_n8A-n78A can be reused. |
|  | DC_1A_n77A or DC_n77A_1A | - | 5th IMD into n8 | - | - 3.3dB MSD by 5th IMD of DC_1A-8A_n77A can be reused. |
| DC_1-3_n3-n8 | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n8A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n8A | - | - | - | No issues |
| DC_3_n3-n77 DC_3_n3-n77(2A) DC_n3-n77_3 DC_n3-n77(2A)_3 | DC_(n)3AA | 2nd harmonic from n3 into n77 | 2nd & 4th IMD into n77 | - | - Harmonic problems was already covered in DC_3A_n77A. - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A DC_n77A_3A | - | 2nd, 4th &5th IMD into n3 | - | - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_3_n3-n8 DC_n3-n8_3 | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n8A or DC_n8A_3A | - | - | - | No issues |
| DC_3_n8-n77 DC_3_n8-n77(2A) DC_n8-n77_3 DC_n8-n77(2A)_3 | DC_3A_n8A or DC_n8A_3A | 2nd harmonic from B3 into n77 4th harmonic from n8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A or DC_8A_n77A. - 16.3dB MSD by 3rd IMD in DC_3A_n8A-n78A can be reused |
|  | DC_3A_n77A or DC_n77A_3A | - | 4th IMD into n8 |  | - 9.7 dB MSD by 4th IMD in DC_3A-8A_n77A can be reused |
| DC_8-3_n3-n77 DC_8-3_n3-n77(2A) | DC_(n)3AA | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A | - | 4th IMD into n8 |  | - 9.7 dB MSD by 4th IMD in DC_3A-8A_n77A can be reused |
|  | DC_8A_n3A | 2nd harmonic from B3 into n77 4th harmonic from n8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A or DC_8A_n77A. - 16.3dB MSD by 3rd IMD in DC_3A_n8A-n78A can be reused |
|  | DC_8A_n77A | 2nd harmonic from B8 into B3 | 3rd IMD into B3 and n3 |  | - Harmonic problems was already covered in DC_3A_n8A - 16.5dB MSD by 3rd IMD in DC_3A-8A_n77A can be reused |
| DC_1-3_n3-n257A/G/H/I/J/K/L/M | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n257A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A | - | - | - | No issues |
| DC_1_n77-n257J/K/L/M DC_1_n77(2A)-n257J/K/L/M DC_n77-n257_1 DC_n77(2A)-n257_1 | DC_1A_n77A or DC_n77A_1A | - | - | - | No issues |
|  | DC_1A_n257A or DC_n257A_1A | - | - | - | No issues |
| DC_1_n8-n257A/G/H/I/J/K/L/M DC_n8-n257_1 | DC_1A_n8A or DC_n8A_1A | - | - | - | No issues |
|  | DC_1A_n257A or DC_n257A_1A | - | - | - | No issues |
| DC_3_n3-n257A/G/H/I/J/K/L/M DC_n3-n257_3 | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_3_n77-n257J/K/L/M, DC_3_n77(2A)-n257J/K/L/M DC_n77-n257_3 DC_n77(2A)-n257_3 | DC_3A_n77A or DC_n77A_3A | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_3_n8-n257A/G/H/I/J/K/L/M DC_n8-n257_3 | DC_3A_n8A or DC_n8A_3A | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_8_n3-n257A/G/H/I/J/K/L/M DC_n3-n257G/H/I/J/K/L/M_8 | DC_8A_n3A DC_n3A_8A | - | - | - | No issues |
|  | DC_8A_n257A DC_n257A_8A | 2nd harmonic from B8 into n3 | - | - | - Harmonic problems was already covered in DC_3A_n8A |
| DC_8-3_n3-n257A/G/H/I/J/K/L/M | DC_8A_n3A | - | - | - | No issues |
|  | DC_8A_n257A | 2nd harmonic from B8 into n3 | - | - | - Harmonic problems was already covered in DC_3A_n8A |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A | - | - | - | No issues |
| DC_n3-n257A/G/H/I/J/K/L/M_1-3 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_n257A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_n257A_3A | - | - | - | No issues |
| DC_n3-n77_1-3 DC_n3-n77(2A)_1-3 | DC_n3A_1A | 2nd harmonic from B1 or n3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - Same as IMD issues in DC_1A_n3A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_(n)3AA | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A - Due to apply MPR, no need to define MSD |
|  | DC_n77A_1A | - | 2nd , 4th & 5th IMDs into B3/n3 | - | - Same as IMD issues in DC_1A_n3A-n78A |
|  | DC_n77A_3A | - | 2nd & 5th IMD into B1 2nd, 4th &5th IMD into n3 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_n3-n8_1 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_n8A_1A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
| DC_n3-n8_1-3 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_n8A_1A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_n8A_3A | - | - | - | No issues |
| DC_7_n79-n257A/G/H/I/J/K/L/M | DC_7A_n79A | 6th harmonic from n79 into n257 | - | - | - No harmonic problems by 6th order between FR1 and FR2 The DC_7A_n79A is still open in TS38.101-3 |
|  | DC_7A_n257A/G/H/I/J/K/L/M | 2nd harmonic from B7 into n79 | - | - | DC_7A_n79A will be solved the harmonic problem. |
| DC_7_n79-n258A/G/H/I/J/K/L/M | DC_7A_n79A | 5th & 6th harmonics from n79 into n258 | - | - | - No harmonic problems by 5th order between FR1 and FR2 The DC_7A_n79A is still open in TS38.101-3 |
|  | DC_7A_n258A/G/H/I/J/K/L/M | 2nd harmonic from B7 into n79 | - | - | DC_7A_n79A will be solved the harmonic problem. |
| DC_3_n78-n79C | DC_3A_n78A |  | 3rd IMD into n79 |  | - Reuse 16.3dB of DC_3A_n78A-n79A by dual uplink DC_3A_n78A in TS38.101-3 |
|  | DC_3A_n79A | 2nd harmonic from B3 into n78 | 5th IMD into n78 |  | - 2nd harmonic issue already solved in DC_3A_n78A. - Reuse 4.2dB of DC_3A_n78A-n79A by dual uplink DC_3A_n79A in TS38.101-3 |
| DC_7_n78-n79 | DC_7A_n78A | - | 3rd, 4th & 5th IMDs into n79 | - | - FFS by 3rd IMD |
|  | DC_7A_n79A | - | 4th IMDs into n78 | - | - FFS by 4th IMD |
| DC_8_n1-n28 | DC_8A_n1A | - | 4th IMD into n28 | - | - 11.6dB by 4th IMD |
|  | DC_8A_n28A | 3 harmonic from n28 into n1 | 5th IMD into n1 | - | - Harmonic problem already covered in DC_1A_n28A - When consider the 5MHz UL CBW for DC_8_n28, the 5th IMD does not fall into the B1 own Rx band. This was decision made for DC_1-8_n28 |
| DC_3_n78-n257J/K/L/M DC_3-3_n78-n257G/H/I/J/K/L/M | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n257A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_7_n78-n257G/H/I/J/K/L/M DC_7-7_n78-n257G/H/I/J/K/L/M | DC_7A_n78A | 7th & 8th harmonic from n78 into n257 |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n257A | - | - | - | - No issue |
| DC_8_n1-n257 | DC_8A_n1A | - | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257A | - | - | - | - No issue |
| DC_42_n78-n257 | DC_42A_n257A | - | - | - | - No IMD problem due to synchronous operation between B42 and n78 |
| DC_42_n77-n257 | DC_42A_n257A | - | - | - | - No IMD problem due to synchronous operation between B42 and n77 |
| DC_20_n3-n78 | DC_20A_n3A | 4th harmonic from B20 into n78 | 3rd & 5th IMDs into n78 |  | - Reuse 16.8dB of DC_3A_n20A-n78A by 3rd IMD in TS38.101-3 |
|  | DC_20A_n78A |  | 3rd IMD into n3 |  | - Reuse 17.3dB of DC_3A-20A_n78A by 3rd IMD in TS38.101-3 |
| DC_20_n3-n67 | DC_20A_n3A | 4th harmonic from B20 into n78 | 3rd & 5th IMDs into n78 |  | - Reuse 16.8dB of DC_3A_n20A-n78A by 3rd IMD in TS38.101-3 |
| DC_2_n12-n78 DC_2-2_n12-n78 | DC_2A_n12A | 2nd harmonic from B2 and 5th harmonic from n12 fall into n78 | 3rd IMD into n78 | - | - Harmonic problem already covered in DC_2A_n78A or DC_12A_n78A - Reuse 16.0dB of CA_n2A-n12A-n77A by 3rd IMD in TS38.101-1 |
|  | DC_2A_n78A | - | - | - | - No issue |
| DC_7_n12-n78 | DC_7A_n12A | 5th harmonic from n12 fall into n78 | 4th IMD into n78 |  | - Harmonic problem already covered in DC_12A_n78A - Reuse 9.7dB of CA_n7A-n28A-n78A in TS38.101-1. |
|  | DC_7A_n78A | - | 2nd IMD into n78 | - | - Harmonic problem already covered in DC_2A_n78A or DC_12A_n78A - Reuse 30.8dB of DC_7A-12A_n78A by 2nd IMD in TS38.101-3 |
| DC_66_n12-n78 | DC_66A_n12A | 2nd harmonic from B66 and 5th harmonic from n12 fall into n78 | 5th IMD into n78 |  | - Harmonic problem already covered in DC_66A_n78A or DC_12A_n78A - Reuse 4.1dB of DC_12A_n66A-n78A by 5th IMD in TS38.101-3 |
|  | DC_66A_n78A | - | - | - | - No issue |
| DC_1_n77-n79 | DC_1A_n77A | - | 3rd,4th & 5th IMD fall into n79 |  | - Reuse 15.9dB of DC_1A_n78A-n79A by 3rd IMD in TS38.101-3 |
|  | DC_1A_n79A | 2nd harmonic from B1 fall into n77 | 5th IMD fall into n77 |  | - Harmonic problem already covered in DC_1A_n77A - Reuse 4.6dB of DC_1A_n78A-n79A by 5th IMD in TS38.101-3 |
| DC_8_n1-n3 | DC_8A_n1A | 2nd harmonic from B8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_8A_n3A | - | - | - | - No issue |
| DC_8_n1-n77 DC_8_n1-n77(2A) | DC_8A_n1A |  | 3rd IMD into n77 | - | - 14.9 dB MSD by 3rd IMD of DC_1A_n8A-n78A can be reused. |
|  | DC_8A_n77A | - | 3rd IMD into n1 |  | - 14.4dB MSD by 3rd IMD in DC_1A-8A_n77A can be reused |
| DC_11_n1-n77 DC_11_n1-n77(2A) | DC_11A_n1A | 2nd harmonic from B11 into n77 | 2nd & 5th IMDs into n77 |  | - Harmonic issue already covered in DC_11_n77A - FFS |
|  | DC_11A_n77A | - | 2nd IMD into n1 |  | - Reuse 30.8dB MSD by 2nd IMD in DC_1A-11A_n77A in TS38.101-3 |
| DC_11_n77-n79 DC_11_n77(2A)-n79 | DC_11A_n77A | - | 2nd & 4th IMDs into n79 | Yes | - If simultaneous Rx/Tx capability is supported, need to define MSD requirements. Else then, not need to define MSD |
|  |  | DC_11A_n79A |  | - |  | 2nd IMD into n77 |  | Yes |  | - If simultaneous Rx/Tx capability is supported, need to define MSD requirements. Else then, not need to define MSD |  |
| DC_41_n1-n3 | DC_41A_n1A | - | - | Yes | The cross band isolation issue already covered in TS38.101-1 |
|  | DC_41A_n3A | - | - | - | - No issue |
| DC_41_n1-n77 | DC_41A_n1A | 2nd harmonic from n1 into n77 | 3rd , 4th & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_1_n77 in TS38.101-3 - Reuse 19.6dB by IMD3 in DC_1A_n41A-n78A in TS38.101-3 |
|  | DC_41A_n77A | - | 4th IMD into n1 | - | - Reuse 9.3dB by 4th IMD of DC_1A-41A_n77A in TS38.101-3 |
| DC_42_n1-n3 | DC_42A_n1A | - | - | Yes | The cross band isolation issue already covered in TS38.101-1 |
|  | DC_42A_n3A | - | - | - | - No issue |
| DC_42_n1-n77 | DC_42A_n1A | 2nd harmonic from n1 into n77 | 4th & 5th IMDs into n77 | Yes | - Harmonic issue already covered in DC_1A_n77A - RAN4 do not allow the simultaneous Rx/Tx for DC_42A_n77A. So there was no self desense issue. |
| DC_3_n20-n67 | DC_3A_n20A |  | - 4th IMD into n67 |  | - FFS |
| DC_11_n3-n79 | DC_11A_n3A | - | 3rd IMD into n79 | - | - FFS |
|  | DC_11A_n79A |  | 3rd IMD into n3 |  | - Reuse 17.8dB by 3rd IMD in DC_3A-21A_n79A in TS38.101-3 |

For the high order xDL/2UL (x=4,5,6) DC combinations, the self-interference problems will be covered in 3DL/2UL or lower order DC operation. Hence, RAN4 do not make summary tables for high order xDL/2UL DC band combinations.

Based on these assumptions, we proposed the MSD test configuration and required MSD levels by self-interference as below.

Table 5.3-2: Proposed MSD levels and MSD test configuration for 3DL/2UL DC UE

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | B40 | IMD4 | ¦2*fB40 -2*fn78¦ | 2320 | 5 | 25 | 2320 | 5 | N/A |
|  | n78 |  |  | 3380 | 10 | 50 | 3380 | 10 |  |
|  | n1 |  |  | 1930 | 5 | 25 | 2120 | 5 | 7.6 |
| DC_7A_n40A-n78A | B7 | IMD4 | ¦2*fB7 -2*fn78¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n78 |  |  | 3700 | 10 | 50 | 3700 | 10 |  |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 | 5 | 9.2 |
| DC_8A_n40A-n78A | 8 | IMD2 | ¦fB8 +fn40¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 28.7 |
|  | 8 | IMD3 | ¦fB8 -2*fn40¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n40 |  |  | 2340 | 5 | 25 | 2340 | 5 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 10 | 16.9 |
|  | 8 | IMD2 | ¦fB8 -fn78¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 | 27.3 |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | IMD3 | ¦2*fB11 +fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3629 | 10 | 50 | 3629 | 10 | 17.5 |
|  | 11 | IMD4 | ¦3*fB11 -fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3586 | 10 | 50 | 3586 | 10 | 11.2 |
|  | 11 | IMD3 | ¦2*fB11 - fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3684 | 10 | 50 | 3684 | 10 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 | 15.8 |
|  | 11 | IMD4 | ¦3*fB11 -fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3564 | 10 | 50 | 3564 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.0 |
| DC_2A_n5A-n77A DC_2A-2A_n5A-n77A | 2 | IMD3 | ¦fB2 +2*fn5¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3540 | 10 | 50 | 3540 | 10 | 16.0 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn5¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3980 | 10 | 50 | 3980 | 10 | 4.1 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn77¦ | 1907 | 5 | 25 | 1987 | 5 | N/A |
|  | n77 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n5 |  |  | 844 | 5 | 25 | 889 | 5 | 3.8 |
| DC_13A_n2A-n77A | 13 | IMD3 | ¦2*fB13 +fn2¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 |  |
|  | n77 |  |  | 3444 | 10 | 50 | 3444 | 10 | 17.3 |
|  | 13 | IMD4 | ¦3*fB13 +fn2¦ | 780 | 5 | 25 | 749 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 | 12.4 |
|  | 13 | IMD5 | ¦2*fB13 -3*fn2¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 |  |
|  | n77 |  |  | 4076 | 10 | 50 | 4076 | 10 | 4.5 |
|  | 13 | IMD3 | ¦2*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 3524 | 10 | 50 | 3524 | 10 |  |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 | 16.0 |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A | 66 | IMD3 | ¦fB66 +2*fn5¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 66 | IMD4 | ¦fB66 +3*fn5¦ | 1714 | 5 | 25 | 2114 | 5 | N/A |
|  | n5 |  |  | 827 | 5 | 25 | 872 | 5 |  |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 | 12.1 |
|  | 66 | IMD5 | ¦3*fB66 -2*fn5¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | IMD5 | ¦2*fB18 +3*fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n77/n78 |  |  | 3770 | 10 | 50 | 3770 | 10 | 4.0 |
| DC_18A_n28A-n77A | 18 | IMD5 | ¦4*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 4045 | 10 | 50 | 4045 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 5.7 |
| DC_18A_n3A-n41A | 18 | IMD2 | ¦fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n41 |  |  | 2540 | 5 | 25 | 2540 | 5 | 29.4 |
|  | 18 | IMD2 | ¦fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2655 | 5 | 25 | 2655 | 5 |  |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 | 28.2 |
| DC_18A_n28A-n41A | 18 | IMD5 | ¦4*fB18 -fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 | 5.1 |
|  | 18 | IMD5 | ¦4*fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 4.7 |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | IMD2 | ¦fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 |  |
|  | n77/n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 30.1 |
|  | 18 | IMD3 | ¦2*fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77 |  |  | 4155 | 10 | 50 | 4155 | 10 | 16.4 |
|  | 18 | IMD4 | ¦2*fB18 -2*fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77/n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 9.1 |
|  | 18 | IMD2 | ¦fB18 -fn78¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77/n78 |  |  | 3450 | 10 | 50 | 3450 | 10 |  |
|  | n41 |  |  | 2630 | 5 | 25 | 2630 | 5 | 28.5 |
| DC_12A_n5A-n77A | 12 | IMD5 | ¦4*fB12 +fn5¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3662 | 10 | 50 | 3662 | 10 | 5.4 |
|  | 12 | IMD5 | ¦4*fB12 -fn77¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n77 |  |  | 3707 | 10 | 50 | 3707 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.8 |
| DC_14A_n5A-n77A | 14 | IMD4 | ¦fB14 +3*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 845 | 5 | 25 | 890 | 5 |  |
|  | n77 |  |  | 3328 | 10 | 50 | 3328 | 10 | 10.6 |
|  | 14 | IMD5 | ¦fB14 +4*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 4113 | 10 | 50 | 4113 | 10 | 5.1 |
|  | 14 | IMD5 | ¦4*fB14 -fn77¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n77 |  |  | 4047 | 10 | 50 | 4047 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.4 |
| DC_2A_n71A-n78A or DC_71A_n2A-n78A | 2/n2 | IMD3 | ¦fB2 +2*fn71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | 71/n71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |
| DC_7A_n71A-n78A | 7 | IMD4 | ¦2*fB7 -2*fn71¦ | 2550 | 5 | 25 | 2670 | 5 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n78 |  |  | 3714 | 10 | 50 | 3714 | 10 | 9.7 |
|  | 7 | IMD5 | ¦3*fB7 -2*fn78¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n78 |  |  | 3520 | 10 | 50 | 3520 | 10 |  |
|  | n71 |  |  | 671 | 5 | 25 | 625 | 5 | 3.9 |
| DC_71A_n38A-n78A | 71 | IMD2 | ¦fB71 +fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 |  |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 | 29.1 |
|  | 71 | IMD4 | ¦2*fB71 -2*fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2580 | 5 | 25 | 2580 | 5 |  |
|  | n78 |  |  | 3774 | 10 | 50 | 3774 | 10 | 10.3 |
|  | 71 | IMD2 | ¦fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 |  |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 | 28.7 |
| DC_5A_n66A-n78A | 5 | IMD3 | ¦fn66 +2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 5 | IMD5 | ¦3*fn66 -2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |
| DC_71A_n66A-n78A | 71 | IMD3 | ¦2*fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 15.5 |
| DC_5A_n38A-n66A | 5 | IMD2 | ¦fB5 +fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n38 |  |  | 2590 | 5 | 25 | 2590 | 5 | 28.9 |
|  | 5 | IMD3 | ¦fB5 -2*fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 |  |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 | 16.4 |
| DC_12A_n2A-n38A or DC_12A_n2A-n41A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n38/n41 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |
| DC_2A_n66A-n77A | 2 | IMD2 | ¦fB2 -fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 4060 | 10 | 50 | 4060 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 29.2 |
|  | 2 | IMD4 | ¦3*fB2 -fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 3540 | 10 | 50 | 3540 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 10.4 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 3930 | 10 | 50 | 3930 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 4.0 |
| DC_13A_n66A-n77A | 13 | IMD3 | ¦2*fB13 +fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 3324 | 10 | 50 | 3324 | 10 | 16.4 |
|  | 13 | IMD4 | ¦3*fB13 +fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 4106 | 10 | 50 | 4106 | 10 | 11.7 |
|  | 13 | IMD5 | ¦2*fB13 -3*fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 3716 | 10 | 50 | 3716 | 10 | 4.4 |
| DC_11A_n3A-n77A | 11 | IMD4 | ¦fB11 -3*fn3¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 |  |
|  | n77 |  |  | 3780 | 10 | 50 | 3780 | 10 | 10.8 |
|  | 11 | IMD5 | ¦4*fB11 -fn3¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 |  |
|  | n77 |  |  | 4020 | 10 | 50 | 4020 | 10 | 4.1 |
|  | 11 | IMD2 | ¦fB11 -fn77¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n77 |  |  | 3310 | 10 | 50 | 3310 | 10 |  |
|  | n3 |  |  | 1775 | 5 | 25 | 1870 | 5 | 29.0 |
|  | 11 | IMD5 | ¦4*fB11 -fn77¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n77 |  |  | 3925 | 10 | 50 | 3925 | 10 |  |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 | 3.7 |
| DC_1A_n28A-n79A | 1 | IMD3 | ¦2*fB1 -fn79¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 | 15.2 |
|  | n79 |  |  | 4648 | 40 | 216 | 4648 | 40 | N/A |
|  | 1 | IMD3 | ¦2*fB1 +fn28¦ | 1950 | 5 | 25 | 2140 | 5 | N/A |
|  | n28 |  |  | 730 | 5 | 25 | 785 | 5 |  |
|  | n79 |  |  | 4630 | 40 | 216 | 4630 | 40 | 14.9 |
| DC_3A_n28A-n79A | 3 | IMD4 | ¦3*fB3 -fn79¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n28 |  |  | 725 | 5 | 25 | 780 | 5 | 10.3 |
|  | n79 |  |  | 4530 | 40 | 216 | 4530 | 40 | N/A |
|  | 3 | IMD4 | ¦3*fB3 -fn28¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n28 |  |  | 725 | 5 | 25 | 780 | 5 |  |
|  | n79 |  |  | 4585 | 40 | 216 | 4585 | 40 | 9.4 |
| DC_21A_n28A-n77A DC_21A_n28A-n78A | 21 | IMD3 | ¦2*fB21 -fn78¦ | 1452 | 5 | 25 | 1500 | 5 | N/A |
|  | n28 |  |  | 730.5 | 5 | 25 | 785.5 | 5 | 16.9 |
|  | n77/n78 |  |  | 3689.5 | 10 | 50 | 3689.5 | 10 | N/A |
|  | 21 | IMD3 | ¦2*fB21 +fn28¦ | 1452 | 5 | 25 | 1500 | 5 | N/A |
|  | n28 |  |  | 730.5 | 5 | 25 | 785.5 | 5 |  |
|  | n77/n78 |  |  | 3634.5 | 10 | 50 | 3634.5 | 10 | 17.3 |
| DC_21A_n28A-n79A | 21 | IMD5 | ¦4*fB21 -fn79¦ | 1450.4 | 5 | 25 | 1498.4 | 5 | N/A |
|  | n28 |  |  | 735.5 | 5 | 25 | 790.5 | 5 | 2.8 |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 40 | N/A |
|  | 21 | IMD4 | ¦2*fB21 +2*fn28¦ | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 5 |  |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 | [6.3] |
| DC_13A_n5A-n77A | 13 | IMD5 | ¦4*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 4013 | 10 | 50 | 4013 | 10 |  |
|  | n5 |  |  | 840 | 5 | 25 | 885 | 5 | 4.5 |
| DC_8A_n28A-n79A | 8 | IMD5 | ¦4*fB8 + fn28¦ | 912.5 | 5 | 25 | 957.5 | 5 | N/A |
|  | n28 |  |  | 745.5 | 5 | 25 | 800.5 | 5 |  |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 | 0.0 |
|  | 8 | IMD5 | ¦4*fB8 - fn79¦ | 905 | 5 | 25 | 950 | 5 | N/A |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 5 |  |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 40 | 3.9 |
| DC_8A_n1A-n28A | 8 | IMD4 | ¦3*fB8 -fn1¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n1 |  |  | 1965 | 5 | 25 | 2155 | 5 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.6 |

<End of TP>