3GPP TSG- RAN WG4 Meeting #101-bis-e	R4-2200795

# Electronic Meeting, January 17-25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.101-4 PDSCH TDD demodulation requirements for inter-cell interference MMSE-IRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2021-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PDSCH demodulation requirements for inter-cell interference MMSE-IRC are agreed to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the PDSCH TDD demodulation requirements for inter-cell interference MMSE-IRC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.15(New), 5.2.3.2.16(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

5.2.2.2.15	Minimum requirements for PDSCH Mapping Type A with inter-cell interference

The performance requirements are specified in Table 5.2.2.2.15-3, with the addition of test parameters in Table 5.2.2.2.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.15-1.

Table 5.2.2.2.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A performance with 2 receive antenna conditions in Homogeneous network, when the PDSCH transmission in the serving cell is interferd by [TBD] interfering cells. | 1-1 |
| Verify the PDSCH mapping Type A performance with 2 receive antenna conditions in Heterogeneous network, when the PDSCH transmission in the serving cell is interferd by [TBD] interfering cell. | 1-2 |

Table 5.2.2.2.15-2: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| Duplex mode |  |  | TDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | [TBD] for test 1-1 [TBD] for test 1-2 |
| Transmission rank |  |  | 1 | random rank with 70% and 30% probability for rank 1 and rank 2 | [TBD] for test 1-1 [TBD] for test 1-2 |
| Time offset between Cells |  | us | 0 | [TBD] | [TBD] for test 1-1 [TBD] for test 1-2 |
| Frequency shift between Cells |  | Hz | 0 | 300 | [TBD] for test 1-1 [TBD] for test 1-2 |
| INR |  |  | N/A | [TBD] for test 1-1 [TBD] for test 1-2 | [TBD] for test 1-1 [TBD] for test 1-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | [TBD] | [TBD] |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |

Table 5.2.2.2.15-3: Minimum performance for PDSCH Mapping Type A with inter-cell interference

| Test num | Reference channel |  |  | Bandwidth (MHz) / Subcarrier spacing (kHz) |  |  | Modulation format and code rate |  |  | TDD UL-DL pattern |  |  | Propagation condition |  |  | Correlation matrix and antenna configuration |  |  | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
|  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell 1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |  |  |
| 1-1 | [R.PDSCH.2-2.1 TDD] | [TBD] | [TBD] | 40 / 30 | 40 / 30 | [TBD] | 16QAM, 0.48 | 16QAM randomly modulated | [TBD] | FR1.30-1 | FR1.30-1 | [TBD] | TDLC300-100 | TDLC300-100 | [TBD] | 2x2, ULA Low | 2x2, ULA Low | [TBD] | 70 | [TBD] |
| 1-2 | [R.PDSCH.2-2.1 TDD] | [TBD] | [TBD] | 40 / 30 | 40 / 30 | [TBD] | 16QAM, 0.48 | 16QAM randomly modulated | [TBD] | FR1.30-1 | FR1.30-1 | [TBD] | TDLA30-10 | TDLA30-10 | [TBD] | 2x2, ULA Low | 2x2, ULA Low | [TBD] | 70 | [TBD] |

5.2.3.2.16	Minimum requirements for PDSCH Mapping Type A with inter-cell interference

The performance requirements are specified in Table 5.2.3.2.16-3, with the addition of test parameters in Table 5.2.3.2.16-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.16-1.

Table 5.2.3.2.16-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A performance with 4 receive antenna conditions in Homogeneous network, when the PDSCH transmission in the serving cell is interferd by [TBD] interfering cells. | 1-1 |
| Verify the PDSCH mapping Type A performance with 4 receive antenna conditions in Heterogeneous network, when the PDSCH transmission in the serving cell is interferd by [TBD] interfering cell. | 1-2 |

Table 5.2.3.2.16-2: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
| Duplex mode |  |  | TDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | [TBD] for test 1-1 [TBD] for test 1-2 |
| Transmission rank |  |  | 1 | random rank with 70% and 30% probability for rank 1 and rank 2 | [TBD] for test 1-1 [TBD] for test 1-2 |
| Time offset between Cells |  | us | 0 | [TBD] | [TBD] for test 1-1 [TBD] for test 1-2 |
| Frequency shift between Cells |  | Hz | 0 | 300 | [TBD] for test 1-1 [TBD] for test 1-2 |
| INR |  |  | N/A | [TBD] for test 1-1 [TBD] for test 1-2 | [TBD] for test 1-1 [TBD] for test 1-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | [TBD] for test 1-1 [TBD] for test 1-2 | [TBD] for test 1-1 [TBD] for test 1-2 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |

Table 5.2.3.2.16-3: Minimum performance for PDSCH Mapping Type A with inter-cell interference

| Test num | Reference channel |  |  | Bandwidth (MHz) / Subcarrier spacing (kHz) |  |  | Modulation format and code rate |  |  | TDD UL-DL pattern |  |  | Propagation condition |  |  | Correlation matrix and antenna configuration |  |  | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
|  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell 1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |  |  |
| 1-1 | [R.PDSCH.2-2.1 TDD] | [TBD] | [TBD] | 40 / 30 | 40 / 30 | [TBD] | [16QAM, 0.48] | 16QAM randomly modulated | [TBD] | FR1.30-1 | FR1.30-1 | [TBD] | TDLC300-100 | TDLC300-100 | [TBD] | 2x4, ULA Low | 2x4, ULA Low | [TBD] | 70 | [TBD] |
| 1-2 | [R.PDSCH.2-2.1 TDD] | [TBD] | [TBD] | 40 / 30 | 40 / 30 | [TBD] | [16QAM, 0.48] | 16QAM randomly modulated | [TBD] | FR1.30-1 | FR1.30-1 | [TBD] | TDLA30-10 | TDLA30-10 | [TBD] | 2x4, ULA Low | 2x4, ULA Low | [TBD] | 70 | [TBD] |

<< End of Changes >>