3GPP TSG-RAN WG4 Meeting # 101-bis-e      		R4-2200800

Electronic Meeting, January 17-25, 2022

Agenda Item:	6.12.2.2

Source: 	CMCC

Title: 	Discussion on R17 demodulation enhancement for intra-cell inter-user interference suppressing

Document for:	Discussion

- Introduction

In RAN4#101-e meeting, we discussed the test parameters and interference modeling for intra-cell inter-user interference MMSE-IRC receiver. A way forward was agreed in [1], simulation results and summary has been captured in TR 38.833 [2]. In this contribution, we provide our views on remaining issues.

- Discussion

  - Inter-user interference modelling

PMI matrix selection for Co-scheduled UE for 2TX and 4TX

- Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

- Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE

- Option 3: Use option 2 for rank 1+1 and option 1 for rank 2+2.

This issue has been discussed for several meetings. Both supporters of Option 1 and supporters of Option 2 aim to define the assumption as the typical case. However, in fact, neither Option 1 nor Option 2 is the practical BS implementation in the real network. Based on the simulation results provided by companies, MMSE-IRC can achieve larger performance gain compared to MMSE under random PMI selection. Therefore, Option 2 is our first choice.

Proposal 1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE

- Conclusion

In this contribution, we discuss the PMI matrix selection method for intra-cell inter-user interference MMSE-IRC receiver, our proposal is:

Proposal 1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of target UE is not equal to any column of precoding matrix of interference UE Reference

[1] R4-2120709, Way Forward on MMSE-IRC for intra-cell inter-user interference, Huawei, Hisilicon.

[2] 3GPP TR 38.833: “Further enhancement on NR demodulation performance”