# 3GPP TSG-RAN WG4 Meeting # 101-bis-e                                                   R4-2200827   							R4-220000x

# Electronic Meeting, January 17-25, 2022

Source:	CMCC

Title:	TP for TR 37.717-11-11: DC_n28A_8C

Agenda Item:	5.14.1

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of NR band n28 and LTE band 8 for TR 37.717-11-11.

# Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.6.0

# Text Proposal

----- Start of TP -----

### 6.1.45	DC_n28_8

#### 6.1.45.1	Configuration for DC

Table 6.1.45.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_8A DC_n28A_8C | DC_n28A_8A | No |

#### 6.1.45.2	Maximum output power for DC

Table 6.1.45.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28_8 | 23 | +2/-3 |

#### 6.1.45.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n28_8 are the same with DC_8A_n28A, which have been already defined in the TS38.101-3 spec.

#### 6.1.45.4	MSD analysis for DC

The MSD analysis for DC_n28_8 are the same with DC_8A_n28A, where there are no harmonic and IMD falling into own receive bands.

#### 6.1.45.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n28_8 are the same with DC_8A_n28A, which have already been defined in the TS38.101-3 spec.

#### 6.1.45.6		Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n28_8 configuration. Thus, no MSD is required for this combination.

#### 6.1.45.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28_8.