3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200868

Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on hypothetical PDCCH parameters for RedCap

Agenda item:	6.20.3.1.4

Document for:	Discussion and Decision

Introduction

The RedCap WID [1] defines among its objectives the specification of support for UEs with reduced maximum UE bandwidth and reduced minimum number of Rx branches, which impacts on the UE performance. Since these assumptions are different from the ones used on the definition of the RRM requirements for NR UEs, RAN4 is currently working on defining requirements for RedCap UEs.

In RAN4 #101e, the simulation assumptions for the hypothetical PDCCH were agreed in [2]. RAN4 is investigating whether the PDCCH transmission parameters for in-sync (IS) and out-of-sync (OOS) evaluations need to be changed for RedCap UEs with 1 RX.

In RAN4 #101e, the following was agreed in the WF [3].

Enhancements to hypothetical PDCCH parameters for RLM Companies are encouraged to provide PDCCH simulation results according to R4-2120386.

The following options were discussed during the RAN4 101e meeting, for the enhancements to hypothetical PDCCH parameters for RLM:

- Power boosting

- Increasing CCE level

- Increase test case SNR

- Evaluate the PDCCH performance whether we need to change the PDCCH transmission parameters or not

Companies were also encouraged to evaluate if there is need to change the simulation assumptions.

In this document we present results for the PDCCH performance based on the simulation assumptions agreed in [2].

Discussion

The simulation results for the PDCCH performance for 2% and 10% BLER are shown in …, considering RedCap UEs with 1 and 2 RX branches.

Table 1-PDCCH performance for RedCap UEs with 1 and 2 RX branches

|  |  | SINR for BLER = 2% [dB] |  |  | SINR for BLER = 10% [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 RX | 1 RX |  | 2 RX | 1 RX |  |
| Channel | SCS [kHz] | CCE 4 | CCE 4 | CCE 8 | CCE 8 | CCE 8 | CCE 16 |
| AWGN | 120 | -6.05 | -3.69 | -6.94 | -10.30 | -7.76 | -10.02 |
| TDLA | 120 | -1.67 | 3.15 | -1.17 | -6.35 | -2.94 | -5.95 |
| AWGN | 15 | -6.03 | -3.59 | -6.97 | -10.25 | -7.76 | -10.01 |
| TDLA | 15 | 0.10 | 3.09 | 0.78 | -5.11 | -3.68 | -8.70 |
| TDLB | 15 | -2.96 | 0.33 | -3.21 | -7.48 | -4.89 | -7.94 |
| TDLC | 15 | -5.03 | -1.78 | -5.38 | -9.08 | -6.88 | -8.78 |
| AWGN | 30 | -6.12 | -3.64 | -7.02 | -10.29 | -7.81 | -9.97 |
| TDLA | 30 | -1.65 | -0.53 | -4.01 | -6.53 | -5.46 | -7.97 |
| TDLB | 30 | -2.77 | -0.65 | -3.5 | -8.22 | -5.74 | -8.37 |
| TDLC | 30 | -1.98 | 0.07 | -3.39 | -7.12 | -5.36 | -7.88 |

In the table above, the baseline results for the Hypothetical PDCCH parameter considered for the Rel-15 requirements (for UEs with 2 RX branches) are highlighted in green.

When comparing the hypothetical PDCCH performance with 2RX and 1 RX, there is a degradation on the SINR required for 2% BLER of up to 4.82 dB in FR2 (TDLA, 120 kHz SCS) and up to 3.29 (TDLB, 15kHz SCS), if an aggregation level of 4 CCE is considered.

In most of the cases, the SINR required for 2% BLER target with RedCap UE with 1 RX and CCE 8 aggregation level is lower than the one required by the baseline result.

Increasing the CCE aggregation level from 4 to 8 for UEs with 1 RX reduces the degradation to at most 0.5 dB in FR2 and 0.68 dB in FR1. In most of the cases, the SINR required for reaching the 2% BLER for RedCap UEs with 1 RX and CCE 8 causes no degradation to the baseline performance.

For RedCap UEs with 1 RX, the PDCCH transmission parameters for in-sync evaluation shall consider aggregation level equal to 8 CCE.

When comparing the hypothetical PDCCH performance with 2RX and 1 RX and CCE 8 aggregation level, there is a degradation on the SINR required for 10% BLER of up to 3.4 dB in FR2 (TDLA, 120 kHz SCS), and up to 2.59 dB in FR1 (TDLB, 15 kHz SCS)

Increasing the CCE aggregation level from 8 to 16 for UEs with 1 RX reduces the degradation of the SINR required for the 10% BLER to at most 0.4 dB (TDLA, 120 kHz SCS) when compared to the baseline case in FR2. In FR1, the majority of the simulations also show a performance improvement, which is up to 3.59 dB (TDLA, 15 kHz).

For RedCap UEs with 1 RX, the PDCCH transmission parameters for out-of-sync evaluation shall consider aggregation level equal to 16 CCE.

According to our simulation results, and considering the simulation assumptions agreed in R4-2120386, no other enhancements to the Hypothetical PDCCH parameters are needed to compensate for reducing the number of RX branches.

Conclusion

In this document, Nokia’s views on the parameters of the hypothetical PDCCH for RedCap UEs with 1 RX are discussed. The following observations and proposals are made, based on the simulation results:

- When comparing the hypothetical PDCCH performance with 2RX and 1 RX, there is a degradation on the SINR required for 2% BLER of up to 4.82 dB in FR2 (TDLA, 120 kHz SCS) and up to 3.29 (TDLB, 15kHz SCS), if an aggregation level of 4 CCE is considered.

Increasing the CCE aggregation level from 4 to 8 for UEs with 1 RX reduces the degradation to at most 0.5 dB in FR2 and 0.68 dB in FR1. In most of the cases, the SINR required for reaching the 2% BLER for RedCap UEs with 1 RX and CCE 8 causes no degradation to the baseline performance.

- For RedCap UEs with 1 RX, the PDCCH transmission parameters for in-sync evaluation shall consider aggregation level equal to 8 CCE.

When comparing the hypothetical PDCCH performance with 2RX and 1 RX and CCE 8 aggregation level, there is a degradation on the SINR required for 10% BLER of up to 3.4 dB in FR2 (TDLA, 120 kHz SCS), and up to 2.59 dB in FR1 (TDLB, 15 kHz SCS)

Increasing the CCE aggregation level from 8 to 16 for UEs with 1 RX reduces the degradation of the SINR required for the 10% BLER to at most 0.4 dB (TDLA, 120 kHz SCS) when compared to the baseline case in FR2. In FR1, the majority of the simulations also show a performance improvement, which is up to 3.59 dB (TDLA, 15 kHz).

For RedCap UEs with 1 RX, the PDCCH transmission parameters for out-of-sync evaluation shall consider aggregation level equal to 16 CCE.

According to our simulation results and considering the simulation assumptions agreed in R4-2120386, no other enhancements to the Hypothetical PDCCH parameters are needed to compensate for reducing the number of RX branches.

- References

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson

- R4-2120386, “Simulation assumption on hypothetical PDCCH performance for RLM and BFD for RedCap UE”, Huawei, HiSilicon

- R4-2120418, “WF on RedCap RRM requirements”, Ericsson

Appendix A

Table A1: Simulation assumptions for PDCCH performance for RLM

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | FR1, 4 GHz | FR2, 30 GHz |
| DCI format / payload size | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs) | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs) |
| Number of control OFDM symbols | SSB based RLM Option 1: CCE = 16 {2 symbols, 15 kHz, 48 PRB} {2 symbols, 30 kHz, 48 PRB} Option 2: CCE = 4,8 {2 symbols, 15 kHz, 24 PRB} {2 symbols, 30 kHz, 24 PRB} CSI-RS based RLM {2 symbols, 15 kHz, 48 PRB} {2 symbols, 30 kHz, 48 PRB} | SSB based RLM Option 1: CCE =16 {2 symbols, 60 kHz, 48 PRB} {2 symbols, 120 kHz, 48 PRB} Option 2: CCE =4,8 {2 symbols, 60 kHz, 24 PRB} {2 symbols, 120 kHz, 24 PRB} CSI-RS based RLM {2 symbols, 60 kHz, 48 PRB} {2 symbols, 120 kHz, 48 PRB} |
| Bandwidth (MHz) |  |  |
| Sub-carrier spacing (kHz) |  |  |
| Aggregation level (CCE) | OOS:8 and 16 IS: 4 and 8 | OOS:8 and 16 IS: 4 and 8 |
| Ratio of hypothetical PDCCH RE energy to average RLM-RS RE energy | OOS:4 dB IS: 0 dB and 3dB | OOS:4 dB IS: 0 dB and 3dB |
| Ratio of hypothetical PDCCH DMRS energy to average RLM-RS RE energy | OOS: 4dB IS: 0 dB and 3dB | OOS: 4dB IS: 0 dB and 3dB |
| Precoder granularity in frequency domain | REG bundle size | REG bundle size |
| REG bundle size | 6 | 6 |
| CP length | Normal | Normal |
| Mapping from REG to CCE | Distributed | Distributed |
| SNR | [-14, -13, ., 1, 0] | [-14, -13, ., 1, 0] |
| Propagation condition | AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns | AWGN TDL-A 30ns |
| Antenna configuration | 2x2 2x1 | 2x2 2x1 |
| UE speed | 30km/h | 3km/h |
| Note: Companies are encouraged to clarify whether the 2Tx simulation results are based Tx diversity or not. |  |  |

The simulation results shown in this contribution are based on Precoder Cycling.