3GPP TSG-RAN WG4 #101-bis-e	R4-2200876

Electronic meeting, Jan. 17-25, 2022

Source: 	Ericsson

Title:  	Inter-frequency measurements enhancement for NR HST in FR1

Agenda Item:	6.8.2.2

Document for:	Discussion

# Introduction

According to work plan [1] on enhancement for NR high speed train scenario in FR1, work continued during the RAN4#101e meeting, with outcome in terms of agreements and open issues captured in WF [2].

This contribution mainly discusses Inter-frequency measurements issues which are still open.

# Discussion

## Sub-topic 2-1: inter-frequency measurement in idle state

In last meeting [2], Agreements on inter-frequency measurement in idle mode were made as follows:

Issue Issue 2-1-1: how to perform the enhancement for inter-frequency measurement in idle mode for HST - Agreements in GTW DRX cycle length [s]; Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). 0.32; [3.2 x M2 (10 x M2)]; [0.32 x M3 (1 x M3)]; 0.96 x M4 (3 x M4). 0.64; [6.4 (10)]; [0.64 (1)]; 1.92 (3). 1.28; [10.24 (8)]; 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2.

There’re some brackets left in table still, we suppose to keep present values in brackets upon compromises in last meeting.

Proposal 1: Regarding enhancement for inter-frequency measurement in idle mode, keep values in below table:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 3.2 x M2 (10 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 6.4 (10) | 0.64 (1) | 1.92 (3) |
| 1.28 | 10.24 (8) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |

## Sub-topic 2-2: inter-frequency measurement with MG, connected state

Issue 2-2-1: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST - Agreements in 2nd round: DRX cycle; T PSS/SSS_sync_inter for FR1 HST. No DRX; max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7. DRX cycle ≤ 160ms; max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2. 160ms < DRX cycle ≤ 320ms; ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2. DRX cycle>320ms; FFS. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.

Only one FFS when DRX cycle>320ms is left in table. We recommend to use ‘N4 x DRX cycle x CSSFinter

N4 = 4 x M2’ in same manner which has been agreed for T SSB_measurement_period_intra.

Proposal 2: Regarding inter-frequency measurement with MG, T PSS/SSS_sync_inter when DRX cycle>320ms is specified as:

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | N4 x DRX cycle x CSSFinter N4 = 4 x M2 |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

# Conclusion

Proposal 1: Regarding enhancement for inter-frequency measurement in idle mode, keep values in below table:

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 3.2 x M2 (10 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 6.4 (10) | 0.64 (1) | 1.92 (3) |
| 1.28 | 10.24 (8) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |

Proposal 2: Regarding inter-frequency measurement with MG, T PSS/SSS_sync_inter when DRX cycle>320ms is specified as:

| DRX cycle | T PSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | max(600ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(600ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | N4 x DRX cycle x CSSFinter N4 = 4 x M2 |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

# References

[1] R4-2017267, Work plan for enhancement for NR high speed train scenario in FR1, CMCC

[2] R4-2120285, WF on RRM for FR1 HST, CMCC