3GPP TSG-RAN WG4 #101-bis-e	R4-2200881

Electronic meeting, Jan. 17-25, 2022

Source: 	Ericsson

Title:  	Number of RX beams for HST FR2

Agenda Item:	6.9.4.2

Document for:	Discussion

# Introduction

Work on RRM requirements for train-mounted UE in high speed train scenario in FR2 [1] continued during the RAN4#101e meeting, with outcome in terms of agreements and open issues captured in WFs [2, 3].

In this contribution, we focus on issues RX beams specifications due to HST FR2 deployment.

# Disucssion

## The scope of RRM requirements

Requirements for Scenario-A and Scenario-B

GtW Agreements: - Define only two sets of enhanced RRM requirements in terms of number of RX beams (i.e. RX beam sweeping scaling factor) per UE - Set 1: 2 RX beams - Set 2: 6 RX beams - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements - Note: the applicability of Set 1/2 requirements to the FR2 HST scenarios will be captured in the TR

Proposal 1: Send LS to RAN2 to define the following signaling:

- Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements

## Scenario-B requirements

Requirements for RRH deployment on both sides of the track

Agreements: Consideration of RRH positions at one/both sides of rail track doesn’t have impact on 6Rx beams agreement in Scenario B (set 2) Way forward: FFS, whether it is necessary and how to address scenario-B (Set 2) with two-side RRH

From the standpoint of UE's capability, UE must cover both sides of the RRH. Zooming in on the transition time during switching measurements from one-side RRH to another-side RRH, additional detection time on the other side is questionable if UE does not always keep measurements on both sides.

Proposal 2: There are several options to deal with the case of scenario-B (Set 2) with two-side RRH:

- No special consideration for two-side RRH, rely on UE’s implementation to cover both directions.

- Introduce signaling to indicate the queue of directions of RRHs in one cell to UE.

- Extra scaling factor is used for scenario-B (Set 2) to allow UE more time if UE justifies the necessity.

We prefer option2 but we’re open to discussion.

# Conclustion

Proposal 1: Send LS to RAN2 to define the following signaling:

- Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements

Proposal 2: There are several options to deal with the case of scenario-B (Set 2) with two-side RRH:

- No special consideration for two-side RRH, rely on UE’s implementation to cover both directions.

- Introduce signaling to indicate the queue of directions of RRHs in one cell to UE.

- Extra scaling factor is used for scenario-B (Set 2) to allow UE more time if UE justifies the necessity.

We prefer option2 but we’re open to discussion.

# References

- RP-202118 “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia

- R4-2120292 “WF on FR2 HST RRM (part 1)”, Nokia, Nokia Shanghai Bell

- R4-2120293 “WF on FR2 HST RRM (part 2)”, Samsung