3GPP TSG-RAN WG4 #101-bis-e	R4-2200885

Electronic meeting, Jan. 17-25, 2022

Source: 	Ericsson

Title:  	Interruption requirements for extending NR operation to 71GHz

Agenda Item:	6.16.7.3

Document for:	Discussion

# Introduction

The WF for extending current NR operation to 71 GHz [1] was approved in RAN#101-e meeting.  In this contribution we present some proposals on Interruption related.

# Disucssion

## Measurement gaps

In last meeting [1], the issue related to interruption was agreed in the following:

Interruption requirements - General principles - The impact of receive timing difference on interruption requirements can be at most 1 additional slot which refers to an asynchronous case. - The interruption requirements for FR2-2 are defined considering only two bands. - Do not define the interruption requirements for inter-band CA/DC for FR2-2 for the case when aggressor cell is on FR2-2 since currently there is only one agreed band in FR2-2. The agreement can be revised if more bands agreed in RF - Do not to define interruption requirements for FR2-2 for the case when aggressor cell is on FR2-1 - For the interruptions when identifying CGI of an FR2-2 NR cell with autonomous gap and interruptions at inter-frequency SFTD measurement, it can be hold on as the feature is deprioritized depending on agreements in [2]. - Interruption requirements - For Standalone NR CA and NR-DC, when an UL carrier or supplementary UL carrier is configured or de-configured, the interruption requirements to be defined as in table below NR Slot length (ms); Interruption length (slots). 0; 1; 1. 1; 0.5; 2. 2; 0.25; 4. 3; 0.125; 8. 5; 0.03125; 32. 6; 0.015625; 64. - For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps: - Option 1: the interruption requirements to be defined as in table below Interruption length X1, Y1 and Y2 during measurements with autonomous gaps NR Slot length (ms) of victim cell; Interruption length X1 (slots); Interruption length Y1 (slots); Interruption length Y2 (slots). 0; 1; 6; 7; 6. 1; 0.5; 12; 13; 10. 2; 0.25; 24; 25; 19. 3; 0.125; 48; 49; 37. 5; 0.03125; 192; 193; 145. 6; 0.015625; 384; 385; 289. - Option 2: do not define interruption requirements

Proposal 1: Regarding Issue ‘For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps:’ Support option 2, do not define interruption requirements.

# Conclustion

Proposal 1: Regarding Issue ‘For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps:’ Support option 2, do not define interruption requirements.

# References

[1] 	R4-2120317- WF on NR extension to 71 GHz – RRM - Part 2, Qualcomm