3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200901

Electronic Meeting, January 17-25, 2022

Source:	China Telecom

Title:	On CSI requirements for UE MMSE-IRC receiver for inter-cell interference suppression

Agenda Item:	6.12.2.1.2

Document for:	Discussion

- Introduction

In the RAN4#101-e meeting, we discussed the test parameters for CQI reporting requirements for UEs supporting MMSE-IRC receiver for suppressing inter-cell interference, and the WF was agreed in [1].

The only remain issue is the antenna configuration for the serving cell. In this paper, our views are given.

- Discussion

Antenna configuration for serving cell

Status in [1]:

Issue 2-3-1: Antenna configuration for serving cell

  - Option 1: 2T2R, 2T4R, ULA Low

  - Option 2: 1x2/1x4 ULA Low antenna configuration

The only difference between option 1 and option 2 is the Tx number. We prefer to reuse the same Tx number we used for all the existing CQI tests in NR FR1, i.e., 2T2R and 2T4R.

There is concern on how to configure the NZP CSI-RS from the target cell with 2 ports, which should be overlapped with the NZP CSI-RS from the neighbor cell with 1 port. We propose to configure the target NZP CSI-RS with 2 ports to observe neighbor NZP CSI-RS interference on one RE and to observe interference data on the other RE. At the same time, PDSCH should be scheduled on the slots containing NZP CSI-RS on the neighbor cell.

In our understanding, this should not cause problem to the interference estimation since no precoding will be used for the neighbor cell with 1Tx only.

Proposal 1: Reuse the same Tx number we used for all the existing CQI tests in NR FR1, i.e., 2T2R and 2T4R.

Proposal 2: Configure the target NZP CSI-RS with 2 ports to observe neighbor NZP CSI-RS interference on one RE and to observe interference data on the other RE. At the same time, PDSCH should be scheduled on the slots containing NZP CSI-RS on the neighbor cell.

- Conclusions

The following proposals were given for CQI reporting requirements:

Proposal 1: Reuse the same Tx number we used for all the existing CQI tests in NR FR1, i.e., 2T2R and 2T4R.

Proposal 2: Configure the target NZP CSI-RS with 2 ports to observe neighbor NZP CSI-RS interference on one RE and to observe interference data on the other RE. At the same time, PDSCH should be scheduled on the slots containing NZP CSI-RS on the neighbor cell.

- References

R4-2120708, WF on CQI reporting requirements for inter-cell interference MMSE-IRC, Ericsson, RAN4 #101-e, Nov 2021.