3GPP TSG-RAN WG4 Meeting # 101-Bis-e		R4-2200923

Electronic Meeting, 17th – 25th January 2022

Title:	[Draft] LS on CORESET#0 impact of CBW narrower than 40MHz of n79

Response to:

Release:	Rel-17

Work Item:	NR_bands_R17_BWs

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact person:	Taekhoon Kim

kuhn.kim@samsung.com

+82-10xxxxxxxx

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

In RAN#94-e, the updated channel bandwidth (CBW) support to the band n79 was approved for the new scenarios (e.g., RedCap) based on RAN4’s agreements. Since the band n79 newly introduces {10, 20, 30, 70, 90} MHz to existing channel bandwidths {40, 50, 60, 80, 100} MHz, the minumum channel bandwidth of n79 has been changed from 40 MHz to 10MHz. As updated in TS 38.101-1 [1], most of the issues has been resolved for the band n79 by adding a step size of the n79 sync raster for the narrower channel bandwidths from RAN4’s perspective.

However, after RAN#94-e, RAN4 also identified a potential non-backward compatible issue due to the minimum bandwidth change of n79. In TS 38.213, upon detection of SS/PBCH block, UE determines CORESET#0 from MIB by looking up the table for controlResourceSetZero. The table is determined according to subcarrier spacing of SSB, subcarrier spacing of PDCCH, and minimum channel bandwidth of the frequency band where UE located. For example, a new UE supporting 10 MHz the minimum channel bandwidth for n79 will have a different table for CORESET#0 configuration with a legacy UE still supporting 40 MHz. Then, how to determine the table in this scenario can be an issue that needs further clarifications. RAN4 has come up with three potential solutions as follows:

- Alt-1: Add narrower channel bandwidth to n79, and indicate different table to legacy UE and new UE for CORESET#0 configuration.

- Alt-1a: Add narrower channel bandwidth to n79, and indicate the same table to legacy UE and new UE for CORESET#0 configuration.

- Alt-2: Add narrower channel bandwidth to new band nX instead of n79.

|  | Pros | Cons | Specification impact |
| --- | --- | --- | --- |
| Alternative 1 | - Limited specification effort (Rel-17) | - Only one CORESET#0 configuration can be used for n79 | - Add clarification on restriction of CORESET#0 configuration in RAN1 |
| Alternative 1a | - Limited specification (Rel-17) - More configuration flexibility comparing Alt.1 | - Lose some configuration flexibility compared with 10MHz table. | - Add clarification on CORESET #0 configuration table determination in RAN1 |
| Alternative 2 | - No impact on RAN1 | - New band shall be introduced (Rel-18) - Potential increase in UE complexity to support both n79 and nX | - Previous RAN4 agreements on n79 is reverted - Add new band in RAN4 |

After the discussion with options above, RAN4 came to a conclusion that it should be consulted with RAN1 on the potential non-backward compatible issue due to the minimum bandwidth change of n79. The decision is up to RAN1.

2	Actions

To RAN1

ACTION: 	RAN4 respectively asks RAN1 to provide the view on each alternative above, and choose the best option from RAN1’s perspective. Given the potential impact on RAN1/4, the decision shall be informed by RAN4#102-e.

3	Dates of next TSG RAN WG 4 meetings

3GPP RAN4#102-e	21st February – 3rd March 2022