3GPP TSG-RAN WG4 Meeting # 101-Bis-e													R4-2200925

Electronic Meeting, 17th – 25th January 2022

Agenda item:			6.19.2

Source:	Samsung

Title:	On RF requirements for further enhancements on MIMO

Document for:	Approval

Introduction

At its latest meeting of FeMIMO, RAN4#101-e, RAN4 continued the discussion on the RF impact to the remaining FFS topics given the status of other WGs. Based on the input contributions, the discussion on the impact to RF requirements resumed for following sub-topics:

- Impact for multi-panel reception

- Impact for MPE

- Impact for SRS enhancement

Some progress has been made, but the discussion needs a great deal more. In this contribution, we provide our view on topics to take a step forward.

Discussion

Additional requirement for multi-panel reception

In the last meeting, RAN4 continued to check whether the multi-panel reception due to the multi-TRP operation of Rel-17 has an impact to the current reception requirements of RAN4. It was agreed that RAN4 would further determine if additional requirement is required for the multi-panel reception as follows [1].

- For the rest of Rel-17 RAN4 meetings, RAN4 focuses on finding out the requirements to verify the simultaneous reception with multi-panels from multi-TRPs - Companies are encouraged to provide views on following conditions - Test system and environment - Type of the requirement - In RAN4#101-bis-e, RAN4 will conclude if specifying additional requirements within Rel-17 is necessary and feasible

In our understanding, even if a UE has a capability of simultaneous reception of RS with different QCL type D, it may not have much impact to existing RF reception requirements. This is because most FR2 requirements do not preclude the simultaneous reception of different DL beams with multi-panels although the Rx requirements of RAN4 specified in Rel-15 did not assume the multi-TRP operation. Hence, the UE is not only able to activate two panels simultaneously, but also switch the panels based on the UE implementation.

In addition, from RF requirement’s perspective, following simulation results can also identify the fact that the change of the current spherical coverage requirement does not work for the feature evaluation.

(a)                                                                                           (b)

Figure 1: Examples of simulation results (n261)

Figure 1-(a) shows that the UE spherical coverage performance does not quite rely on the number of panels in a UE. Instead, it can be said that some 2-panel UEs would have better performance than 3-panel UEs at the lower %-tile CDF point in a certain case due to other factors. Since a UE has the limited form factor, its antenna performance can be more affected by housing condition, panel design/performance, or panel location rather than the number of panels per UE.

Regarding the panel location, as shown in Figure 1-(b), even though a UE has multiple panels which are lined up in a row, left and right side, in order to well receive two different signals from antiparallel directions, such design would not be always as good as other placements. For example, its deviation is even higher at the lower CDF %-tile point due to the large uncertainty such as unintended beams. Therefore, it can be concluded that the CDF %-tile of the spherical coverage requirement does not necessarily mean the %-tile range that UE is able to receive two different AoAs from the entire sphere.

Observation 1: CDF %-tile of the spherical coverage requirement does not necessarily mean the %-tile range that UE is able to receive two different directions from the entire sphere.

Given the consideration above, rather than simply changing the existing requirement or criterion without sufficient verifications of the feature, it would be a more proper and meaningful way to start the discussion about the requirement with the RF test system for 2AoA DL in future releases.

Proposal 1: No RAN4 requirement assuming the simultaneous reception channel/RS with different QCL type D is specified in Rel-17.

Proposal 2: RAN4 can start the discussion about the requirement with the RF test system for 2AoA DL in Rel-18.

Impact of MPE enhancements

On the impact of the MPE enhancement, after a long wait, RAN4 knuckled down to the work based on the latest RAN1 agreement. RAN1 made the agreement on MPE mitigation enhancement on top of the RAN4 scheme of Rel-16, i.e., reporting P-MPR in PHR via MAC CE. Essentially, it is replacing UE-specific P-MPR with N “per-beam” P-MPR values with their SSBRI/CRIs. During the discussion in RAN4, most companies supported not to make changes to the existing P-MPR related definition/capabilities for the Rel-17 MPE enhancements. On the other hand, some companies shared that RAN4 still needs further check if the current definition/capability could accommodate the enhancement. Based on the discussion, following agreements were reached for further check [1].

- RAN4 will further check if the current definition/capability could implement the enhancement for a following case or not - Relation between each of the reported P-MPR values(N≤4) tied with the corresponding respective(M=1) SSBRI(s)/CRI(s) and Pcmax - For the next meeting, companies are encouraged to provide views if changes are required considering RAN1 agreements and above

In Rel-16 framework of RAN4, UE provides per-UE level P-MPR for MPE together with PHR only for the beam used for the transmission based on current configuration and scheduling information. Hence, the RAN4 solution adopted in Rel-16 cannot address potential multiple candidate UL beams (N). Also, RAN4 does not have any per-beam level transmit power related definition, nor have the strict definition of how to derive P-MPR reporting but stating that "this field indicates the applied power back off to meet MPE requirements". However, from UE perspective, if multiple UL beams are possible to be activated, and different P-MPR values are corresponding to different UL beams, it is reasonable for UE to assume the worst case by reporting the largest P-MPR value only.

Observation 2: It is reasonable for UE to assume the worst case by reporting the largest P-MPR value only.

Therefore, the most straightforward method to reflect the enhancement would be keeping the existing per-UE based P-MPR to the configured UE maximum output power calculation while measuring the multiple per-beam level P-MPR values, i.e., reporting the largest P-MPR value only. In that sense, what RAN4 can do would be only to modify the current note of the P-MPR based on the enhancement in RAN1 without a change of the P-MPR related definition, e.g., PCMAX,f,c. Relevant UE capabilities can be added as RAN4 did in Rel-16 if required, e.g., introduce a NOTE to explain UE capabilities and relation between measured multiple P-MPRs (N) and the reported P-MPR.

Observation 3: RAN4 does not have to change the P-MPR related definitions in the RF spec except the notes.

Conclusion

In this paper, the RF impact of Rel-17 FeMIMO was discussed on topics remained for further study. Following observations and proposals were provided.

Observation 1: CDF %-tile of the spherical coverage requirement does not necessarily mean the %-tile range that UE is able to receive two different directions from the entire sphere.

Proposal 1: No RAN4 requirement assuming the simultaneous reception channel/RS with different QCL type D is specified in Rel-17.

Proposal 2: RAN4 can start the discussion about the requirement with the RF test system for 2AoA DL in Rel-18.

Observation 2: It is reasonable for UE to assume the worst case by reporting the largest P-MPR value only.

Observation 3: RAN4 does not have to change the P-MPR related definitions in the RF spec except the notes.

Reference

[1] R4-2114994, WF on FeMIMO impacts to RF requirements, RAN4#100-e