3GPP TSG-RAN4 WG4 Meeting #101bis-e	R4-2200955

# Electronic meeting, January 17-25, 2022

Source: 	vivo

Title: 	Link level Simulation results for pi/2 BPSK

Agenda Item:	7.4.4

Document for:	Information

Introduction

In this paper, we provide link level simulation results for pi/2 BPSK based on the below parameter list in [1].

Recommended Parameter list

| Parameter | Value |
| --- | --- |
| Pulse shaping filter | Filter configuration conforms to 38.101-1 |
| Channel model | TDL-C300ns, TDL-A30, TDL-D30 |
| MCS | 0 |
| Waveform | DFTS OFDM with pi/2 BPSK filtered by same filter as for Rel-16 DMRS |
| # of DMRS symbols/slot | 2 |
| # of data symbols/slot | 12 |
| # of RBs | [2, 4, 8, 16, 64] |
| TX/RX configuration | 1TX/4RX |
| BW | 100 MHz |
| UE speed | 3 km/h |
| SCS | 30 kHz |
| HARQ configuration | No retransmissions |

Simulation results

In the RAN# 94e meeting, a LS was sent from RAN to RAN4. The agreements related to transparent spectral shaping were agreed as follows:

Transparent spectral shaping Agreements - Both data and DMRS are filtered - The choice of filters is up to UE implementations and transparent to the network

In addition, as pointed out in the last meeting [2], both transmitter and receiver performances should be the deciding criterion for filter evaluation. Therefore, in this contribution, we compare with the frequency domain characteristics and Bler performance under different configuration parameters of three-tap filter.

The frequency domain characteristic curves are shown in figure 1. It can be observed that for the more aggressive spectral shape filter, the curve is narrower in the frequency domain. And it also shows that the correlation between time-domain data would be greater and the PAPR would be smaller. Therefore, it is helpful to boost the power of UE output.

However, for the spectral shape filter, the receive demodulation performance would also be reduced compared with no filter since the FDSS would cause non-uniform distribution of SNR on each subcarrier and impact the channel equalization.

Figure 1: Frequency characteristics of different spectral shape filters

Observation 1: For the more aggressive spectral shape filter, the PAPR would be smaller.

The Bler curves for different configuration parameters of three-tap filter are shown is figure 2.

Figure 2: Bler performance of different spectrum shape filters

It can be observed that for all channel model (e.g., TDL-A, TDL-C, TDL-D) and RBs (2RB and 8RB), for the more aggressive spectral shape filter (e.g., [0.33 1 0.33]), the Bler performance would be worse. In addition, the maximum SNR difference between no-filter and FDSS filter when the Bler is 10% is about is 1dB.

Observation 2: For all channel model (e.g., TDL-A, TDL-C, TDL-D) and RBs (2RB and 8RB), for the more aggressive spectral shape filter (e.g., [0.33 1 0.33]), the Bler performance would be worse.

Observation 3: The maximum SNR difference between no-filter and FDSS filter when the Bler is 10% is about 1dB.

Based on the above analyse, we can see that it is very necessary to select the appropriate filter, PAPR performance and Bler performance must be taken into consideration at the same time.

Observation 4: PAPR performance and Bler performance must be taken into consideration at the same time for FDSS filter selection.

Conclusion

Observation 1: For the more aggressive spectral shape filter, the PAPR would be smaller.

Observation 2: For all channel model (e.g., TDL-A, TDL-C, TDL-D) and RBs (2RB and 8RB), for the more aggressive spectral shape filter (e.g., [0.33 1 0.33]), the Bler performance would be worse.

Observation 3: The maximum SNR difference between no-filter and FDSS filter when the Bler is 10% is about 1dB.

Observation 4: PAPR performance and Bler performance must be taken into consideration at the same time for FDSS filter selection.

- References

- R4-2108018 WF on Waveform configuration, parameters for link simulations and agreements.

- R4-2120057 WF on Optimisations of Pi/2 BPSK UL Power in NR, Huawei, HiSilicon, Qualcomm, Intel.