3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2200977  
Electronic Meeting, January 17-25, 2022

Agenda item:	6.2.1

Source:	vivo

Title:	TP to TR 38.834 on Phantom definition

Document for:	Approval

1	Introduction

In RAN4#101-e meeting, the OTA phantoms related agreements were captured in the WF [1].

Issue 1-4-2: OTA phantoms

Agreements:

- For test method development, keep current use cases as it is to ensure that the test method can cover different size of smartphone.

  - Wide Grip Hand for UE with Width >72mm and ≤92mm

  - PDA Grip Hand for UE with Width ≥56mm and ≤72mm

In the Reply LS from CTIA [2], the coordination between CTIA and 3GPP on OTA phantom related work was confirmed:

It could be possible for 3GPP to reference the phantoms within the CTIA Certification OTA test plan.  A no-cost non-exclusive license agreement could be created giving 3GPP the permission to reference the appropriate sections of the test plan along with “© CTIA Certification. Referenced with permission.”

Given the completion date of core part for test method development is March 2022, to facilitate the technical report editing work, this paper provides the text proposals for OTA phantoms definition.

2	References

- R4-2120689, “WF on FR1 TRP TRS,” vivo, 3GPP RAN4#101-e, Nov 2021

- R4-2114591, Reply LS on Head and Hand Phantoms for 5G FR1 OTA Testing, CTIA Certification, RAN4#100e, August, 2021

- 3GPP TR 38.834 v0.2.0

3	Text Proposal to TR 38.834

--------------Start of text proposal 1-------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.827: “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone".

[6]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[7]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment ".

[8]	3GPP TR 25.914: "Measurements of radio performances for UMTS terminals in speech mode".

[9]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE.

[10]	3GPP TR 37.902: "Measurements of User Equipment (UE) radio performances for LTE/UMTS terminals; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test ".

[11]	CTIA Test Plan: "CTIA Certification Test Plan for SISO Over-the-Air Performance Ver 3.9.3", https://ctiacertification.org/wp-content/uploads/2021/02/CTIA-Certification-Test-Plan-for-SISO-Over-the-Air-Performance_3_9_3.pdf

[12]	3GPP TS 37.544: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing ".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

--------------End of text proposal 1 -------------

--------------Start of text proposal 2-------------

6	UE positioning guidelines

6.1	Free space

For Free space configuration, the centre of the reference coordinate system shall be aligned with the geometric centre of the DUT in order to minimize the offset between antenna arrays integrated at any position of the UE and the centre of the quiet zone.

Table 6.1-1: UE positioning for Free space

| Test condition | DUT orientation | Diagram |
| --- | --- | --- |
| Free space DUT | α = 0º; β = 0º; γ = 0º |  |

6.2	Hand phantom only (Browsing mode)

6.2.1	General

According to the UE use scenarios for TRP TRS test defined in Section 4.2.1, the positioning specified in this clause is used for the test cases for Browsing Mode with Hand Phantom. The characteristics of the Hand Phantom are specified in Annex D. Browsing mode is used to simulate user cases were the DUT is held in hand, but not pressed against ear e.g. web browsing and navigation. The DUT shall be mounted in a suitable hand phantom and oriented such that the DUT’s main display is tilted 45 degrees from vertical:

-  Wide Grip Hand for UE with Width >72mm and ≤92mm

-  PDA Grip Hand for UE with Width ≥56mm and ≤72mm

6.2.2	Wide Grip Hand

This positioning guideline is suitable for DUTs with width >72mm and ≤92mm.

The positioning guideline defined in CTIA OTA Test Plan section A.1.5.3 [11] © CTIA Certification, referenced with permission, is used for FR1 TRP TRS testing for UE with Width >72mm and ≤92mm in this technical report.

6.2.3	PDA Grip Hand

This positioning guideline is suitable for DUTs with width ≥56mm and ≤72mm.

To help achieve a consistent positioning, the DUT is aligned to a PDA palm spacer. No alignment tool is required. The PDA spacer features side and bottom walls to ensure consistent alignment of DUTs of various sizes.

- Place the DUT on the PDA spacer between the fingers and align the DUT to the side wall of the PDA.

- If the DUT is shorter than 135 mm, then align the top of the DUT with the top of the PDA spacer. Otherwise, align the bottom of the DUT with the bottom wall of the PDA spacer.

- While keeping the DUT in the hand phantom in the position defined in previous steps, place the DUT and the hand phantom against the head phantom in such way that the DUT is in 6°tilt angle as described in Subclause 4.3.3.

Figure 6.2.1-1: Right-handed PDA hand phantom with a spacer

NOTE: Use left-handed (mirror-imaged) spacers with left-handed phantoms.

6.3	Head and Hand phantom (Talk Mode)

6.3.1	General

According to the UE use scenarios for TRP TRS test defined in Section 4.2.1, the positioning specified in this clause is used for the test cases for Talk Mode using Head & Hand Phantom. The characteristics of the Hand Phantom and Head Phantom are specified in Annex D.

Talk mode is used to simulate user cases where the DUT is placed into a hand phantom, which is holding the DUT against the SAM head phantom, presenting a realistic voice call operation of the DUT. Same as Browsing mode, the DUT for talk mode shall also be mounted in a suitable hand phantom:

-  Wide Grip Hand for UE with Width >72mm and ≤92mm

-  PDA Grip Hand for UE with Width ≥56mm and ≤72mm

In this section, the procedure provides the guideline on how to place the DUT+hand against the head phantom. The detailed DUT positioning into hand phantom for talk mode is defined in section 6.3.2 and 6.3.3.

For talk mode, the DUT is attached to the head phantom in "cheek" position. The DUT performance is measured on both left and right side of the head. Three points as shown in Figure 6.3.1-1 define the reference plane: centre of the right ear piece (RE), centre of the left ear piece (LE) and centre of mouth (M).

Definition of the 'Cheek' position:

- Align the ear piece of the phone (see Figure 4.3.1-1) at the line RE-LE. Then, position the DUT beside the phantom so that the vertical line (see Figure 4.3.1-3) is parallel to the reference plane in Figure 4.3.1-2 and is aligned with the line M-RE on the reference plane (see Figure 4.3.1-3).

- Position the DUT so that the ear piece of the DUT touches the ear piece of the phantom head on the line RE-LE. Tilt the DUT chassis towards the cheek of the phantom having the vertical line aligned with the reference plane until any point on the front side of the DUT is in contact with the cheek or until the contact with the ear is lost.

NOTE:	A holder fixture made of e.g. plastic may be used to position the handset against the phantom.

Figure 6.3.1-1: Reference plane on head phantom, front view

Figure 6.3.1-2: Reference plane on head phantom, side view

In addition, 6°tilt angle from the cheek is being used instead of having direct contact between the cheek and DUT. A mask may be used to help configuration of cheek + 6° tilt angle. The mask is a 32 mm wide conformal strip, created by sweeping the surface of the head phantom through a 6º rotation about the ear. Direct DUT contact against the mask thus establishes the required 6º spacing away from the cheek, regardless of DUT form factor. The material for the head phantom mask spacer shall have dielectric constant of less than 1.3 and a loss tangent of less than 0.003. Material additions can be used to help fixing of the mask spacer onto the head phantom.

6.3.2	Wide Grip Hand and Head

This procedure is suitable for talk mode use with DUTs of width >72mm and ≤92mm. The positioning of the DUT in the Wide PDA hand for talk mode is identical to that for browsing mode described in section 6.2.2.

6.3.3	PDA Grip Hand and Head

This procedure is suitable for talk mode use with DUTs of width ≥56mm and ≤72mm. The positioning of the DUT in the PDA hand for talk mode is identical to that for browsing mode described in section 6.2.3.

--------------End of text proposal 2-------------

--------------Start of text proposal 3-------------

Annex D:
Phantom Definition

< Editor’s note: Detailed structure of the subclause is TBD. Needs collaborations with CTIA on this part>

D.1	Head Phantom

The basic head phantom is based on the “SAM” head phantom in IEEE Std 1528-2003, which is also described in TS 37.544 Annex A.2 [12]. For TRP TRS test, the IEEE SAM head model has been extended with the neck region, which is specified in CTIA OTA Test Plan [11].

The Head phantom defined in CTIA OTA Test Plan section C.2 [11] © CTIA Certification, referenced with permission, is used for FR1 TRP TRS testing in this technical report.

D.2	Hand Phantom

D.2.1	PDA Hand

The PDA Hand described in TS 37.544 Annex A.2.2 [12], which is identical to that defined in CTIA OTA Test Plan section C.4.8 [11], is adopted for TRP TRS testing for the UE with width ≥56mm and ≤72mm.

D.2.2	Wide Grip Hand

The Wide Grip hand defined in CTIA OTA Test Plan section C.4.9 [11] © CTIA Certification, referenced with permission, is used for FR1 TRP TRS testing for UE with Width >72mm and ≤92mm in this technical report.

--------------End of text proposal 3-------------