3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2200983

# Electronic Meeting, Jan.17-25,2022

Title: 	Discussion on test setup of CRS-IM receiver for 15 kHz SCS

Source: 	Huawei, HiSilicon

Agenda item:	6.12.2.3.2

Document for:	Discussion

Background

According to WF [1], there are some open issues for CRS-IM receiver. In this paper, we provide our corresponding discussions

Discussions

Interference power level

The options are listed as follows:

- Interference power level - INR1 = 10.45 dB and INR2 = 4.6 dB will be used as the INR for defining requirements. - FFS other INR value can be included. - Option 1 for the other INR value: Add one set of INRs with smaller INR values where reasonable CRS-IM gain over the reference scheme can still be shown, for example, select INR values that achieve ~1dB CRS-IM gain over the reference scheme. - Proponents for option 1 are encouraged to provide exact numbers on the additional interference INR/loading level.

High performance gain for LLR weighting can be met under such interference power level. We don’t think option 1 is feasible since our intention is to verify the performance gain of CRS-IM rather than the conditions of turning on CRS-IM.

Proposal 1: Consider INR1=10.45dB and INR2=4.6dB

PDSCH loading level on interference cell

The options are listed as follows:

PDSCH loading level on interference cell - 20% loading level with full PRB allocation will be used for defining requirements. - FFS other interference loading level can be included.

20% loading level is a good condition that high performance gain for LLR weighting can be met. Performance will severely degrade if higher load level is introduced. Therefore, we propose to only consider 20% loading level.

Proposal 2: Only consider 20% loading level.

LTE CRS port number

The options are listed as follows:

Further discuss the LTE CRS port number - Option 1: Only cover 4 CRS ports - Option 2: Cover 2 and 4 CRS ports - Option 3: Only cover 2 CRS ports

We propose to only consider 4 ports since it has the largest interference of all options. Therefore there is no need to consider 2 CRS ports.

Proposal 3: Only consider 4 CRS ports

Conclusion

In this paper, we provide our proposals for test setup of CRS-IM receiver for 15 kHz SCS. The proposals are:

Proposal 1: Consider INR1=10.45dB and INR2=4.6dB

Proposal 2: Only consider 20% loading level.

Proposal 3: Only consider 4 CRS ports

Reference

- R4-2120705, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR, China Telecom, RAN4 #101e, Nov 2021.