3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2201007

# Electronic Meeting, 17th - 25th Jan, 2022

Title: 	Discussion on PUSCH with UL timing adjustment requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	6.9.5.3.2

Document for:	Discussion

Background

During RAN#101-e meeting, WF [1] on FR2 HST demodulation was approved. In this contribution, we provide our simulation results about PUSCH with UL timing adjustment requirements for FR2 HST.

Simulations

Here we provide the updated simulation results for FR2 HST PUSCH with UL timing adjustment for alignment as Figure 2-1 and Table 2-2 shows, based on simulation assumption as Table 2-1 shows.

Table 2-1 Simulation assumption for PRACH requirement

| Parameters | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350km/h |
| Waveform | CP-OFDM |
| CP length | Normal |
| A | AWGN |
| Δω | 1T2R |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | 1DMRS+PT-RS(L=1,K=2), 2DMRS+PT-RS(L=1,K=2), 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |

Figure 2-1 Ideal simulation results for FR2 HST PUSCH with UL timing adjustment

Table 2-2 Ideal simulation results for FR2 HST PUSCH with UL timing adjustment

| Case number | CBW(MHz) | DMRS | SNR (dB) |
| --- | --- | --- | --- |
| 1 | 50 | 1 | 6.56 |
| 2 | 200 | 1 | 6.40 |
| 3 | 50 | 1+1 | 6.41 |
| 4 | 200 | 1+1 | 6.40 |
| 5 | 50 | 1+1+1 | 6.43 |
| 6 | 200 | 1+1+1 | 6.40 |

Conclusion

In this contribution, we provide our updated simulation results for PUSCH with UL timing adjustment for NR HST FR2.

Reference

R4-2120703, WF on BS demodulation requirement for FR2 HST, RAN4#101-e, Samsung