3GPP TSG-RAN WG4 Meeting #101bis-e	R4-2201012

# Electronic Meeting, 17th - 25th Jan, 2022

Title: 	Discussion and simulation results on 1024QAM SDR

Source: 	Huawei, HiSilicon

Agenda item:	6.6.5.3

Document for:	Discussion

Background

As per WF [1], demodulation part of 1024QAM WI should be discussed from this meeting. In this contribution, we share our views about the 1024QAM SDR requirements.

Discussion

SDR requirements

- Define SDR requirements for 1024 QAM with rank 1 and rank 2. - Add the MCS indexes for SDR test with 1024QAM based on Table 2. - Interested companies are encouraged to propose MCSpracticalj to decide MCSreqj, where - MCSreqj= min⁡(MCSupperboundj, MCSpracticalj) Table 2 MCS indexes for SDM test with 1024QAM vLayersj; Qmj; fj; MCSupperboundj; MCSpracticalj; MCSreqj. 1; 10; 1; 25; TBD; TBD. 1; 10; 0.8; 21; TBD. 1; 10; 0.75; 19; TBD. 1; 10; 0.4; 9; TBD. 2; 10; 1; 25; TBD; TBD. 2; 10; 0.8; 21; TBD. 2; 10; 0.75; 19; TBD. 2; 10; 0.4; 9; TBD.

Here we provide evaluation for SDR requirements as Figure 2.1-1 shows.

Figure 2.1-1 Simulation results for 1024QAM SDR requirements

We can see that MCS26 cannot be achieved for rank1 and MCS24 cannot be achieved for rank2 for SDR requirements. Therefore we propose to Select MCSpracticalj = 25 for rank 1 and 23 for rank 2.

Select MCSpracticalj = 25 for rank 1 and 23 for rank 2.

Proposals

In this contribution, we discuss on UE SDR requirements for 1024QAM. Our observations and proposals are:

- Select MCSpracticalj = 25 for rank 1 and 23 for rank 2.

Reference

R4-2102699, WF on DL1024QAM demodulation and CQI reporting requirements, RAN4#101-e, Ericsson