3GPP TSG-RAN WG4 Meeting #101-bis-e                                                                  R4-2201047

Online Meeting, 17th -25th, Jan, 2022

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-02: CA_n18A-n41A-n77A

Agenda item:			5.11.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n18-n41-n77 according to the request in [1].

This contribution is a resubmission from last meeting contribution which was noted for uncompleted fallback mode.

2. Reference

- R4-2118209 Revised WID on Rel-17 NR Inter-band Carrier Aggregation Dual Connectivity for 3 bands DL with 2 bands UL

- R4-2118598 TR 38.717-03-02 v0.7.0

# 3. Text Proposal

<Start of Text Proposal>

    - 5.1.x	CA_n18-n41-n77

      - 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n18-n41-n77 | n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200MHz | 3300 MHz | – | 4200 MHz | TDD |

      - 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n41A-n77A | CA_n18A-n41A CA_n18A-n77A CA_n41A-n77A | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

      - 5.1.x.3	UE co-existence studies

For CA combinations with 3 bands DL and 2 bands UL, only IMD issues due to dual Tx operation of two bands falling into the third band Rx need to be considered.

IMD2, IMD3, IMD4 generated by dual Tx n18 and n41 fall into the third band n77 Rx

IMD2 and IMD3 generated by dual Tx n18 and n77 fall into the third band n41 Rx

IMD2, IMD3 and IMD5 generated by dual Tx n41 and n77 fall into the third band n18 Rx.

      - 5.1.x.4	REFSENS requirements

Based on co-existence studies on 5.1.x.3, MSD values are defined below, IMD2 of dual Tx fall into n77 and IMD2 of dual Tx fall into n41 are reused from DC_18A_n41A-n77A and DC_18A-41A_n77A.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n41-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2570 | 5 | 25 | 2570 | N/A | TDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 30.1 | TDD | IMD22,4 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n41 | 2630 | 5 | 25 | 2630 | 28.5 | TDD | IMD24 |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n18 | 825 | 5 | 25 | 870 | 29.3 | FDD | IMD21,4 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

<End of Text Proposal>