3GPP TSG-RAN WG4 Meeting #101-bis-e                             R4-2201061

Online, 17th - 25th Jan, 2022

Source:	Samsung, Telus, Bell Mobility

Title:	TP for TR 37.717-41-11: DC_2-7-29-66_n78

Agenda item:			5.17.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-41-11 to update the reference sensitivity exceptions for DC_2-7-29-66_n78.

2. Reference

- R4-2118610 Revised Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1UL)

- R4-2118609 draft TR 37.717-41-11-0.7.0

# 3. Text Proposal

<Start of Text Proposal>

5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-29A-66A_n78A DC_2A-7C-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |

5.1.X.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are specified below:

Table 5.1.X.2-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-29-66_n78 DC_2-7-7-29-66_n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 29 | 0 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.X.2-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-7-29-66_n78 DC_2-7-7-29-66_n78 | 2 | 0.2 |
|  | 7 | 0.5 |
|  | 29 | 0.2 |
|  | 66 | 0.5 |
|  | n78 | 0.5 |

5.1.X.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

<End of Text Proposal>