3GPP TSG-RAN WG4 Meeting #101-bis-e                 R4-2201067

# Electronic Meeting, 17th-25th, Jan, 2022

Source:	Samsung

Title:	Discussion on simultaneous Rx/Tx capability for FDD-TDD FR1 band combination

Agenda Item:	5.39.1

Document for:	Approval

# 1	Introduction

In last meeting, whether simultaneous Rx/Tx capability could be default supported and changed to optional for the FR1 FDD-TDD band combos whose MSD exceed a threshold (some options in WF[1] are as below) were discussed but no conclusion. In this contribution, we provide our views.

- Way forward for MSD threshold

According to the WF in last RAN4 meeting, MSD threshold is a remaining issue for FR1+FR1 FDD-TDD band combination, and there are several options proposed for this meeting

- Option 1: The threshold should be higher than 29.8(2 antenna ports) / 32.5(4 antenna ports)

- Option 2: 14dB MSD is taken as the criteria for optional simultaneous RxTx capability

- Option 3: NW controlled approached can be further considered if single MSD criteria is difficult to be aligned

- Option 4: Others

- 2	Discussion

According to 38.822 [2], simultaneous reception and transmission for inter-band CA or EN-DC (TDD-TDD or TDD-FDD) could be mandatory or optional, it is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1/2/3. It is also summarized in the WID [3] that currently this capability is conditional mandatory and for the combinations which have no such indication, the capability is optional. It is noted that in Rel-15, whether a FDD-TDD band combination shall mandatory support simultaneous Rx/Tx is based on operators’ request. In the latest specs it could be found that this capability for most FR1 FDD-TDD combinations are optional.

In our understanding, simultaneous Rx/Tx capability is optional means that for one band combination it depends on UE’s claim supporting simultaneous Rx/Tx, otherwise, the capability is not present

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

Observation1: Simultaneous Rx/Tx capability is optional means that for one band combination it depends on UE’s claim supporting simultaneous Rx/Tx, otherwise, the capability is not present

In parallel, there is also some threshold discussion in “Low MSD” and if simultaneous Rx/Tx capability is not present means worse MSD performance than a threshold, does it conflict with the potential “Low MSD” capability which will be discussed in Rel-18? For example, if one FR1 FDD-TDD band combo reports Low MSD capability, does it have to support mandatory simultaneous Rx/Tx? If Low MSD threshold is defined per source as an absolute value, would it be better to align the Low MSD threshold of IMD and the threshold of mandatory simultaneous Rx/Tx? Any other undetected issues?

Observation 2: If simultaneous Rx/Tx capability is not present means worse MSD performance than a threshold, it may conflict with potential “Low MSD” capability which will be discussed in Rel-18

Proposal 1: It is proposed to consider specify mandatory simultaneous Rx/Tx capability case by case for FR1 FDD-TDD band combinations at least in Rel-17, i.e. whether a FDD-TDD band combination mandatory support simultaneous Rx/Tx is based on operator’s request.

- 3 Conclusion

In this contribution, we provide our views on simultaneous Rx/Tx capability for FR1 FDD-TDD band combination.

Observation1: Simultaneous Rx/Tx capability is optional means that for one band combination it depends on UE’s claim supporting simultaneous Rx/Tx, otherwise, the capability is not present

Observation 2: If simultaneous Rx/Tx capability is not present means worse MSD performance than a threshold, it may conflict with potential “Low MSD” capability which will be discussed in Rel-18

Proposal 1: It is proposed to consider specify mandatory simultaneous Rx/Tx capability case by case for FR1 FDD-TDD band combinations at least in Rel-17, i.e. whether a FDD-TDD band combination mandatory support simultaneous Rx/Tx is based on operator’s request.

- 4	Reference

[1] R4-2119947 WF on simultaneous RxTx capability, Huawei, HiSilicon, RAN4#101-e (to be noted)

[2] 38.822 v16.2.0

[3] TR 38.839 v 0.1.0, Huawei, HiSilicon