3GPP TSG-RAN WG4 Meeting #101-bis-e			  										R4-2201083 Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.849 updating clause 5.1 for the lower 6GHz

Agenda item: 	5.1.5

Document for:	Approval

# 1 Introduction

The latest WID as given [1] was updated at RAN#94. This contribution provides a TP for clause 5.1 in TR 38.849 to address the band definition based on the agreement during RAN#94 in [2] and captured in chair minutes that:

Agreements:

- •	A new band for unlicensed operation in the lower 6GHz band is defined as 5.925-6.425GHz.

- •	The EU/CEPT NS flag(s) will be explicitly limited to the 5.945-6.425GHz range.

# 2 Text Proposal

******************************* Start of TP ***************************************

## 5.1	Band definition

### 5.1.1 Band definition for lower 6GHz NR unlicensed operation

A new NR band for unlicensed operation in the range 5925-6425 MHz is defined as:

Table 5.1.1-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n102 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD3 |
| NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213. |  |  |  |

Table 5.1.1-2: Channel bandwidths and SCS per operating band in FR1

| NR Band | SCS (kHz) | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |
| n102 | 30 |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |
|  | 60 |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  |  |

******************************* End of TP ***************************************

# References

- RP-213604, Revised WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Nokia Shanghai Bell

- RP-213628, Moderator's summary of discussion [94e-51-R17-Lower6GHz-NR-U], RAN4 chair.