3GPP TSG-RAN WG4 Meeting#101bis      	R4-2201086

E-meeting, 17th – 25th Jan, 2022

Title:	TP to TR 38.717-03-02 Addition of CADC_n7A-n46-n78A

Source:	Nokia, BT

Agenda item:	5.11.1

Document for:	Approval

# 1	Introduction

This contribution introduces CADC_n7A-n46-n78A into TR 38.717-03-02.

******************************* Start of TP ***************************************

### 5.1.x	CA_n7-n46-n78

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n7-n46-n78 | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n78 | 3300MHz | – | 3800MHz | 3300MHz | – | 3800MHz | TDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration

| CA Config | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n46A-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n46 |  |  |  | 20 |  |  |  | 40 |  | 60 |  | 80 |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n46C-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n46 | See CA_n46C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n46D-n78A | CA_n7A-n46A CA_n7A-n78A CA_n46A-n78A | n7 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n46 | See CA_n46D Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

#### 5.1.x.3	Co-existence studies

For UE coexistence study of Band n7 + Band n46 Band n7 + Band n78 and Band n46 + band n78 the 2nd 3rd 4th and 5th order harmonics are already analyzed in 3DL/1UL WI. The 2nd 3rd 4th and 5th order intermodulation products are calculated and presented in the following.

For CA_n7-n46 the 5th IMD product is falling inside n78 as shown in the following analysis.

Table 5.1.x.3-1: Band n7 and Band n46 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 5150 | 5925 |
| DL Frequency [MHz] | 2620 | 2690 | 5150 | 5925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2580 | 3425 | 7650 | 8495 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 925 | 10 | 7730 | 9350 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10150 | 11065 | 12800 | 14420 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1575 | 2560 | 12880 | 15275 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6850 | 5160 | 15300 | 16990 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 12650 | 13635 | 17950 | 20345 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 21200 | 18030 | 5130 | 4075 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 12775 | 10310 | 2590 | 4350 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 23100 | 26270 | 15150 | 16205 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 20450 | 22915 | 17800 | 19560 |

For CA_n7-n78 the 5th IMD product is falling inside n46 as shown in the following analysis.

Table 5.1.x.3-2: Band n7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 3300 | 3800 |
| DL Frequency [MHz] | 2620 | 2690 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

For CA_n46-n78 the 2nd IMD product is on the lower 5 MHz of n7 as shown in the following analysis.

Table 5.1.x.3-3: Band n46 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 5150 | 5925 | 3300 | 3800 |
| DL Frequency [MHz] | 5150 | 5925 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2625 | 1350 | 8450 | 9725 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6500 | 8550 | 675 | 2450 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 15650 | 11750 | 13525 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 11650 | 14475 | 3975 | 6250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2700 | 5250 | 16900 | 19450 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 18750 | 21575 | 15050 | 17325 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10050 | 7275 | 20400 | 16800 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1100 | 1950 | 11175 | 7850 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18350 | 21125 | 23900 | 27500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 20200 | 23250 | 22050 | 25375 |

Co-existence studies shows that

- The 5th IMD generated by dual uplink of Band n7 + Band n46 may fall into own Rx of band n78.

- The 5th IMD generated by dual uplink of Band n7 + Band n78 may fall into own Rx of band n46.

- The 2nd IMD generated by dual uplink of Band n46 + Band n78 may fall into own Rx of band n7. However, this is only the lower 5 MHz and given the possible channel placements for n46 it is not possible for the IMD to hit the Rx band of n7 in practice. Hence no MSD is defined for this case.

#### 5.1.x.4	REFSENs requirements

MSD have been defined for the two 5th IMD issues. For the 2nd IMD issue no MSD is defined as it is only the lower 5 MHz of n7 which is impacted and given the possible channel placements for n46 it is not possible for the IMD to hit the Rx band of n7 in practice

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7A-n46A-n78A CA_n7A-n46C-n78A CA_n7A-n46D-n78A | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n46 | 5460 | 20 | 100 | 5460 | N/A | TDD | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 17 | TDD | IMD5 |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n46 | 5260 | 20 | 100 | 5260 | 14 | TDD | IMD5 |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |

## 6.x	DC_n7-n46-n78

### 6.x.1	Configurations for DC_n7-n46-n78

Table 6.x.1-1: Inter-band NR DC configurations (three bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n7A-n46A-n78A DC_n7A-n46C-n78A DC_n7A-n46D-n78A | DC_n7A-n46A DC_n7A-n78A DC_n7A-n78A |

******************************* End of TP ***************************************