3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-220110

Electronic Meeting, Jan. 17th – 26th, 2021

Title:	TP to TR 38.717-03-01: Addition of CA_n2-n12-n30

Source:	Nokia, AT&T

Agenda item:	5.9.1

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-03-01 to add CA_n2-n12-n30.

************************************* Start of TP*****************************************

## 6.x	CA_n2-n12-n30

### 6. x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2A-n12A-n30A | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |

#### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n12A-n30A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n2(2A)-n12A-n30A | - | n2 | See CA_n2(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Co-existence studies of CA_n2-n12-n30 with 1UL are already covered in the constituent fall-back modes.

### 6.x.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n2, n12 and n30, the TIB,c and RIB,c values are shown in table 6.x.4-1 and table 6.x.4-2, respectively. They are taken from the existing combination DC_2-12_n30.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n12-n30 | n2 | 0.5 |
|  | n12 | 0.3 |
|  | n30 | 0.3 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n12-n30 | n2 | 0.4 |
|  | n12 | 0 |
|  | n30 | 0.5 |

### 6.x.5	REFSENS requirements

There are no additional MSD requirements for this band combination

************************************* End of TP*****************************************