3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201114

Electronic Meeting, Jan. 17th – 26th, 2021

Title:	TP to TR 38.717-03-02: Addition of CA_n12-n30-n66

Source:	Nokia, AT&T

Agenda item:	5.11.1

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-03-02 to add CA_n12-n30-n66 with 2UL.

************************************* Start of TP*****************************************

### 5.1.x	CA_n12-n30-n66

#### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n12A-n30A-n66A | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n12A-n30A-n66A | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n12A-n30A-n66(2A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n12A-n30A-n66(3A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

Co-existence studies for dual uplink operation of two bands, i.e. CA_n12A-n30A, CA_n12A-n66A and CA_n30A-n66A have been captured in TR 38.717-02-01 where:

-	There are no IMD products produced by Band n12 and n30 that might fall in Rx of band n66.

-	There are no IMD products produced by Band n12 and n66 that might fall in Rx of band n30.

-	IMD5 products are produced by Band n30 and n66 that might fall in Rx of band n12.

#### 5.1.x.4	REFSENS requirements

The IMD issue specific to 3DL/2UL is that the 5th IMD product falls inside band n12. There is no need for MSD test points to be defined since the overlap is only 1MHz (729MHz - 730MHz) and there is no possible test configuration available.

************************************* End of TP*****************************************