3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201128

# Electronic Meeting, 17 – 25 January, 2022

Source:	LG Electronics

Title:	A-MPR analysis results for NR-U(VLP) considering regulatory parameters in Korea.

Agenda item:	5.2.3

Document for:	Approval

Introduction

In RAN#92-e meeting, New WI [1] on “Introduction of operation in full unlicensed band 5925-7125MHz” was approved.

In this contribution, based on the WF[4] of RAN4#101-e, we provide A-MPR analysis results for NR-U(VLP) considering

regulatory parameters in Korea.

Discussion

Simulation assumptions for A-MPR are as follows.

- PA calibration : 1dB MPR for PC5 and DFT-s-OFDM QPSK 100RB3 20MHz waveform - Determine back-off required to meet Korea regulatory parameters - Out-of-band emissions [3] : -34 dBm/MHz (f ≤ 5925MHz, f ≥ 6445MHz) - Maximum PSD for in-band emissions [3] : +1 dBm/MHz(5925 – 6425MHz) - Waveform : CP-OFDM , DFT-s-OFDM - A-MPR approach for VLP mode - LPI-based VLP-capable devices : AMPR of VLP mode is calculated relative to +20 dBm(PC5). - VLP-dedicated devices : AMPR of VLP mode is calculated relative to +14 dBm(PC6). ※ In both cases above, the PA calibration point is based on +20dBm(MPR 1dB). - Modulation order - PI/2 BPSK/QPSK/16QAM/64QAM/256QAM - Channel Bandwidth - 20/40/60/80MHz - Uplink configuration - Full allocation , Interlaced Allocation

Our proposal for A-MPR approach to VLP mode is additionally described in [5].

NR-U devices that support VLP can be classified into two types. LPI-based VLP-capable devices and VLP-dedicated

devices that support VLP only. Our simulation assumption is that in both cases the PA calibration point is based on +20

dBm (MPR 1 dB). Our simulation results for LPI-based VLP-capable devices are in Table1. Tx power difference between

PC5 and PC6 is 6dB. PC6(+14dBm) A-MPR of VLP- dedicated devices with PC5 PA can be reused with 6dB scaling by

reusing the proposed PC5(+20dBm) A-MPR table 1 for VLP in South Korea.

Table 1.  Proposed PC5(+20dBm) A-MPR table for VLP in South Korea

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 7.5 | ≤ 10.0 |
|  | 16 QAM | ≤ 8.0 | ≤ 10.5 |
|  | 64 QAM | ≤ 8.5 | ≤ 10.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.0 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 11.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. |  |  |  |

Table 2.  Proposed PC6(+14dBm) A-MPR table for VLP in South Korea

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 1.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 2.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 2.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 3.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 3.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 3.0 | ≤ 5.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. |  |  |  |

Based on A-MPR analysis result, we propose as follow

Proposal 1: Define PC5(+20dBm) A-MPR table for VLP in South Korea as provided in Table 1.

Proposal 2: Define PC6(+14dBm) A-MPR table for VLP in South Korea as provided in Table 2.

Conclusion

In this contribution, we provide A-MPR analysis results for NR-U(VLP) considering regulatory parameters in Korea.

Based on A-MPR analysis result, we propose as follow

Proposal 1: Define PC5(+20dBm) A-MPR table for VLP in South Korea as provided in Table 1.

Proposal 2: Define PC6(+14dBm) A-MPR table for VLP in South Korea as provided in Table 2.

- Reference

[1] RP-211602, “New WID for Introduction of operation in full unlicensed band 5925-7125MHz, Apple Inc”

[2] TR 37.890, “3GPP;Technical Specification Group Radio Access Network;Feasibility Study on 6 GHz for LTE and NR

in Licensed and Unlicensed Operations (Release 17)”

[3] Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations"

- URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

- URL: https://www.law.go.kr/admRulLsInfoP.do?chrClsCd=010202&admRulSeq=2100000196973

[4] R4-2119863,”WF on introduction of the full unlicensed band, Apple Inc”

[5] R4-2201125, “Discussion on the impact of the New power class definitions for NR-U UE in other countries, LGE”