3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201133

Electronic Meeting, January 17-25, 2022

Agenda item:	6.8.2.3

Source:	OPPO

Title:	Other RRM requirements for Rel17 FR1 HST

Document for:	Discussion

- Introduction

In last RAN4 meeting, a WF on FR1 HST RRM was approved [1]. In this contribution, we will provide our views on other remaining issues for signalling.

- Discussion

- Agreements on Signaling - For IDLE mode, introduce network signalling to inform UE whether the enhanced inter-frequency HST measurement requirements shall be applied per each inter-frequency carrier. The signalling design is up to RAN2. - For connected state, NW shall NOT indicate UE for which inter-frequency carriers, the enhanced inter-frequency measurement requirements shall be applied. - It is RAN4 understanding that the requirements will be defined under assumption that all carriers in the connected mode are high-speed train carriers and enhanced inter-frequency measurement requirements shall be applied - The common understanding is to inform RAN2 that Rel-16 signaling already covers CA case and inform that Rel-17 we have new feature for RRM CA. It is up RAN2 decision on how to define signalling. - For CA, introduce UE capability to indicate whether the enhanced requirements for HST CA are supported or not. Issue 3-4-4: for inter-frequency measurement, whether the network signalling for CA enhancement can be reused for inter-frequency measurement - Q1: whether the network signalling for CA enhancement can be reused for inter-frequency measurement - Option 1 (Ericsson): Yes, highSpeedMeasFlag-r16 can be reused for inter-frequency measurement - Option 2 (MTK, CMCC, Apple, MTK, QC, HW, CATT, vivo): No, introduce a new network signalling to indicate the application of the enhanced requirements for inter-frequency - Q2: if new signalling is agreed, whether it is per cell level or per frequency layer level (pending on Issue 3-4-2) - Option 1 (CMCC, QC): the signalling is a cell-level signalling - Option 2 (HW): the indication is per inter-frequency layer configuration

Rel-16 signaling already covers CA case and inform that Rel-17 we have new feature for RRM CA. But highSpeedMeasFlag-r16 cannot be reused for inter-frequency measurement. We prefer to introduce a new network signalling to indicate the application of the enhanced requirements for inter-frequency. Per cell-level signalling is ok.

Proposal 1: Introduce a new cell-level signalling to indicate the application of the enhanced requirements for inter-frequency.

Issue 3-5-1: release independent - FFS whether Rel-17 NR HST RRM enhancement can be release independent from Rel-15

For release independent, we suggest to Rel-17 NR HST RRM enhancement can be release independent from R17.

Proposal 2: Rel-17 NR HST RRM enhancement can be release independent from R17.

- Conclusion

In this contribution, we provide our views on R17 HST enhancement in FR1.

Proposal 1: Introduce a new cell-level signalling to indicate the application of the enhanced requirements for inter-frequency.

Proposal 2: Rel-17 NR HST RRM enhancement can be release independent from R17.

References

- R4-2120285, WF on FR1 HST RRM, CMCC

- R4-2118360, Other RRM requirements for Rel17 FR1 HST, OPPO