3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2201171

# Electronic Meeting, January 17-25, 2022

Source:	LG Electronics

Title:	Discussion on Rx RF requirements in FR2-2

Agenda item:	6.16.3.2

Document for:	Discussion

# Introduction

In this paper, we provide our view on Rx RF requirements in FR2-2.

# Discussion

RAN4 has discussed the number of antenna elements for FR2-2 Tx RF requirements of handheld UE and vehicular UE.

The related agreements in WF are captured.

- Handheld UE antenna array number of elements assumption - Agreement: - Commercial FR2-1 antenna module physical dimension can be treated as the feasible FR2-2 antenna module dimension. - Commercial FR2-1 antenna module is equipped with 1x4 or 2x2 antenna elements - Address the concerns related to the following identified factors into the next meeting - Vehicular UE antenna array assumption - Agreement based on the majority of companies - Option 1: Vehicular array size is equal to or larger than the handheld - FWA UE antenna array assumption - Agreement: If a single power class is defined for FWA in Rel-17, the number of antenna element assumption is anywhere in the range between 32 and 64 elements

Table 2.1 and Table 2.2 summarize the reference sensitivity and the EIS spherical coverage which correspond to channel bandwidth of 400MHz for power class 1, 2, 3, 4 and 5 in FR2-1.

Table 2.1: Reference sensitivity for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Frequency | REFSENS (dBm) / 400MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PC1 | PC2 | PC3 | PC4 | PC5 |
| n257 | 28GHz(26500MHz – 29500MHz) | -88.5 | -83.0 | -79.3 | -88.0 | -83.6 |
| n258 | 24GHz(24250MHz – 27500MHz) | -88.5 | -83.0 | -79.3 | -88.0 | -83.8 |
| n259 | 39GHz(39500MHz – 43500MHz) |  |  | -75.7 |  | -80.7 |
| n260 | 39GHz(37000MHz – 40000MHz) | -85.5 |  | -76.7 | -86.0 |  |
| n261 | 28GHz(27500MHz – 28350MHz) | -88.5 | -83.0 | -79.3 | -88.0 |  |
| n262 | 47GHz(47200MHz – 48200MHz) | -83.5 | -77.8 | -73.8 | -82.0 |  |

Table 2.2: EIS spherical coverage for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | EIS at Xth %-tile CCDF (dBm) / 400MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC3 | PC4 | PC5 |
|  | X = 85 | X = 60 | X = 50 | X = 20 | X = 85 |
| n257 | -80.5 | -72.0 | -68.4 | -79.0 | -75.6 |
| n258 | -80.5 | -72.0 | -68.4 | -79.0 | -75.8 |
| n259 |  |  | -62.9 |  | -72.7 |
| n260 | -77.5 |  | -64.1 | -74.0 |  |
| n261 | -80.5 | -72.0 | -68.4 | -79.0 |  |
| n262 | -75.3 | -65.9 | -60.7 | -69.9 |  |

Table 2.3 summarize the difference between the reference sensitivity and EIS spherical coverage which correspond to each X%-tile for power class.

Table 2.3: Difference between the reference sensitivity and EIS spherical coverage for Power Class 1,2,3,4 and 5 in FR2-1

| Operating band | Difference between the reference sensitivity and EIS spherical coverage (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PC1 | PC2 | PC3 | PC4 | PC5 |
|  | X = 85 | X = 60 | X = 50 | X = 20 | X = 85 |
| n257 | 8.0 | 11.0 | 10.9 | 9.0 | 8.0 |
| n258 | 8.0 | 11.0 | 10.9 | 9.0 | 8.0 |
| n259 |  |  | 12.9 |  | 8.0 |
| n260 | 8.0 |  | 12.9 | 12.0 |  |
| n261 | 8.0 | 11.0 | 10.9 | 9.0 |  |
| n262 | 8.2 | 11.9 | 13.1 | 12.1 |  |

The difference is from the difference between the minimum peak EIRP and EIRP at X%-tile CDF.

For handheld UE, considering the commercial FR2-1 antenna module physical dimension and the equipped antenna elements, 8 antenna elements is possible for FR2-2.

For reference sensitivity, -75.3dBm/400MHz is analyzed for 8 antenna elements as seen in Table 2.4 with following formula. The value is in range of the reference sensitivity of power class 3 in FR2-1.

- REFSENS = -174dBm(kT) + 10*log10(Max. RX BW) + NF – Total Ant. gain - diversity gain + SNR + ILs

Table 2.4: Reference sensitivity for handheld UE in FR2-2

| Parameter | Unit | Value |
| --- | --- | --- |
| Frequency range | GHz | 57~71 |
| Modulation |  | QPSK |
| SNR requirement | dB | -1 |
| Bandwidth | MHz | 400 |
| 10*log10(Max. RX BW), Max.Rx BW = 264RB@120kHz | dB | 85.8 |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure(NF) | dB | 15 |
| # of antennas in an array |  | 8 |
| Array gain | dB | 9.0 |
| Average antenna element gain | dBi | 4.6 |
| Antenna gain roll-off over frequency | dB | -2.5 |
| Realized antenna array gain | dBi | 11.1 |
| Diversity gain | dB | 0 |
| Total Implementation loss(ILs) | dB | 10 |
| REFSENS @ 400MHz | dBm | -75.3 |

Observation 1: For handheld UE, reference sensitivity at channel band width of 400MHz is

- -75.3 dBm/400MHz for 8 antenna elements

For EIS spherical coverage, the difference between the minimum peak EIRP and EIRP at 50%-tile CDF can be reused as FR2-1. In [2], the difference is analyzed as follows depending on number of panel.

- X = 14 dB for 1 panel

- X = 9 dB for 2 panels

Observation 2: For handheld UE, EIS spherical coverage at 50%-tile CCDF is X dB higher than reference sensitivity for 8 antenna elements.

- X = 14 dB for 1 panel

- X = 9 dB for 2 panels

EIS spherical coverage of 2 panels are 5dB better than 1 panel.

We propose the observations to be considered for requirements of handheld UE.

Proposal 1: For handheld UE, consider the following observations for 8 antenna elements.

- Reference sensitivity

    - -75.3 dBm for CBW of 400MHz

- EIS spherical coverage at 50%-tile CCDF is X dB higher than Reference sensitivity

    - X = 14 dB for 1 panel

    - X = 9 dB for 2 panels

For vehicular UE, based on the agreement in WF, 16 antenna elements is considered.

For reference sensitivity, -80.3dBm/400MHz is analyzed for 16 antenna elements as seen in Table 2.7.

Table 2.7: Reference sensitivity for vehicular UE in FR2-2

| Parameter | Unit | Value |
| --- | --- | --- |
| Frequency range | GHz | 57~71 |
| Modulation |  | QPSK |
| SNR requirement | dB | -1 |
| Bandwidth | MHz | 400 |
| 10*log10(Max. RX BW), Max.Rx BW = 264RB@120kHz | dB | 85.8 |
| Thermal noise | dBm/Hz | -174 |
| Noise Figure(NF) | dB | 15 |
| # of antennas in an array |  | 16 |
| Array gain | dB | 12.04 |
| Average antenna element gain | dBi | 4.6 |
| Antenna gain roll-off over frequency | dB | -2.5 |
| Realized antenna array gain | dBi | 14.1 |
| Diversity gain | dB | 0 |
| Total Implementation loss(ILs) | dB | 8 |
| REFSENS @ 400MHz | dBm | -80.3 |

Observation 3: For vehicular UE, reference sensitivity at channel band width of 400MHz is -80.3dBm for 16 antenna elements.

For EIS spherical coverage, the difference between the minimum peak EIRP and EIRP at 60%-tile CDF can be reused as FR2-1. In [2], the difference is analyzed as follows depending on number of panel for 16 antennal elements(2x8).

- X = 15.1 dB for 1 panel

- X = 7.5 dB for  2 panels

Observation 4: For vehicular UE, EIS spherical coverage at 60%-tile CCDF is X dB higher than reference sensitivity for 16 antenna elements.

- X = 15.1 dB for 1 panel

- X = 7.5 dB for 2 panels

We propose the observations to be considered for requirements of vehicular UE.

Proposal 2: For vehicular UE, consider the following observations for 16 antenna elements.

- Reference sensitivity

    - -80.3 dBm for CBW of 400MHz

- EIS spherical coverage at 60%-tile CCDF is X dB higher than Reference sensitivity

    - X = 15.1 dB for 1 panel

    - X = 7.5 dB for 2 panels

# Conclusion

In this contribution, we provided our views on reference sensitivity and EIS spherical coverage for handheld UE and vehicular UE for FR2-2.

For handheld UE

Proposal 1: For handheld UE, consider the following observations for 8 antenna elements.

- Reference sensitivity

    - -75.3 dBm for CBW of 400MHz

- EIS spherical coverage at 50%-tile CCDF is X dB higher than Reference sensitivity

    - X = 14 dB for 1 panel

    - X = 9 dB for 2 panels

For vehicular UE

Proposal 2: For vehicular UE, consider the following observations for 16 antenna elements.

- Reference sensitivity

    - -80.3 dBm for CBW of 400MHz

- EIS spherical coverage at 60%-tile CCDF is X dB higher than Reference sensitivity

    - X = 15.1 dB for 1 panel

    - X = 7.5 dB for 2 panels

# Reference

[1] R4-2120062, “	WF on 60GHz UE RF requirements”, Qualcomm

[2] R4-22xxxxx, “Discussion on Tx RF requirements in FR2-2”, LG Electronics.