3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201184

Electronic Meeting, January 17-25, 2022

Title: 	Discussion on	6.20.3.1.2	Mobility requirements due to UE complexity reduction

Source: 	Huawei, HiSilicon

Agenda item:	6.20.3.1.2

Document for:	Discussion

Introduction

The revised WID on support of reduced capability (RedCap) NR devices has been approved in [1]. RAN4 has started work on the support of RedCap NR devices, where the related study item has been concluded in TR 38.875. The WID has the following objectives on UE complexity reduction:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. - Relaxed maximum modulation order: - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE. - No other relaxations of maximum modulation order are specified for a RedCap UE. - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

It is concluded in RAN#100e meeting that there is no RRM impact due to Maximum number of DL MIMO layers and Relaxed maximum modulation order [2]. This contribution further provides analysis on the mobility requirements due to reduced UE BW, reduced RX branch.

Discussion

Handover

As we know, the UE supported band would be reported to network. The cross FR handover types can be supported by UE who supports both FR1 and FR2 band. From UE implementation perspective, RF and baseband are different for FR1 and FR2. Whether RedCap UE supporting both FR1 and FR2 depends on UE product form. As RedCap UE is a reduced complexity UE type, there is a high probability that UEs support only one type of FR. Moreover the realistic application scenarios for FR1-FR2 handover and FR2-FR1 handover are not clear. We suggest not to specify the cross FR handover scenario at this stage.

HO between FR1 and FR2 - Option 1: Develop requirements for HO between: - NR FR2- NR FR1 Handover - NR FR1- NR FR2 Handover

Proposal 1: At this stage, not develop requirements for FR2-FR1 and FR1-FR2 handover.

For Redcap UE with 1Rx, the time to identify target NR cell, Tidentify_intra_NR and Tidentify_inter_NR, are determined through simulation campaign at SINR =-2 dB. Based on the simulation results in [3], at least 3 samples are required at high side condition.

Proposal 2: 3 samples are required for Tsearch (in HO) for Redacap UE with 1Rx.

For RedCap UEs with HD-FDD operation, collisions may happen between configured SSB and configured uplink transmission. This issue has reached agreement in RAN1 that the SSB is prioritized over dynamically scheduled UL transmission.

Agreements: - For HD-FDD operation for RedCap UEs, collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 8: Dynamic or semi-static DL vs. valid RO - Case 9: Collision due to direction switching

Agreement: - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

Therefore the restriction in handover requirements for RedCap UEs with HD-FDD operation can be: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement.

Proposal 3: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement.

RRC re-establishment

Based on the logic of legacy re-establishment requirements, when the side condition is ≥ -8dB, for RedCap UE Tidentify in known NR cell case could refer to SSB measurement requirement with 1Rx, and the Tidentify in unknown NR cell case could refer to PSS/SSS detection + SSB measurement requirement with 1Rx. Based on our simulation results in [3], at least 6 samples are needed for cell detection with1RX in FR1@SINR=-6dB. The measurement period is supposed to be unchanged with relaxed measurement accuracy. Therefore when serving cell SSB SINR≥ -8dB, Tidentify_intra_NR for known case is 6 x TSMTC, Tidentify_intra_NR for unknown case is 11 x TSMTC for FR1.

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 80 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Proposal 4: For RedCap UE with 1Rx, when serving cell SSB SINR≥ -8dB the re-establishment requirements are defined:

-Tidentify_intra_NR for known case is 6 x TSMTC,

-Tidentify_intra_NR for unknown case is 11 x TSMTC for FR1.

There are some discussion on the lower bound during last meeting. The delay extension due to 1RX is derived from cell detection rather than measurement period. Therefore for known cell, lower bound 200ms is supposed to be unchanged. For unknown case, as the current 800ms (200ms+600ms) has reserved sufficient margin for cell detection, therefore the need of further extension is not observed.

As we discussed in handover section 2.1, HD-FDD requires the measurement opportunities and would not extend the measurement delay. The same conclusion can be applied for re-establishment requirements as well.

Proposal 5: For RedCap UE, the lower boundary in Max function for reestablishment delay requirement shall not be changed.

RRC connection release with redirection

In legacy RRC connection release with redirection requirements, Tidentify-NR includes cell detection time (4 samples), AGC settling (3 samples) and measurement period (4 samples). The side condition for redirection is applicable down to -4dB.

Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 11 x Trs) |
| FR2 | MAX (880 ms, 8x11 x Trs) |
| Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

For RedCap UE with 1 RX, ased on our simulation results in [3], at least 5 samples for cell detection are needed for 1RX in FR1@SINR=-4dB (considering the performance degradation compared with 2RX). The measurement period is supposed to be unchanged with relaxed measurement accuracy. The sample number for AGC settling could reuse the legacy value. Therefore Tidentify-NR for RRC connection release with redirection is 12 x TSMTC for FR1.

For the lower bound, it is the same situation as RRC re-establishment.

Proposal 6: For RedCap UE with 1Rx, Tidentify-NR for RRC connection release with redirection is 12 x TSMTC for FR1.

Proposal 7: For RedCap UE, the lower boundary in Max function for RRC connection release with redirection shall not be changed.

Conclusions

This contribution provides analysis on the signaling characteristics due to complexity reduction for RedCap UE. The following proposals are provided:

Proposal 1: At this stage, not develop requirements for FR2-FR1 and FR1-FR2 handover.

Proposal 2: 3 samples are required for Tsearch (in HO) for Redacap UE with 1Rx.

Proposal 3: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement.

Proposal 4: For RedCap UE with 1Rx, when serving cell SSB SINR≥ -8dB the re-establishment requirements are defined:

-Tidentify_intra_NR for known case is 6 x TSMTC,

-Tidentify_intra_NR for unknown case is 11 x TSMTC for FR1.

Proposal 5: For RedCap UE, the lower boundary in Max function for reestablishment delay requirement shall not be changed.

Proposal 6: For RedCap UE with 1Rx, Tidentify-NR for RRC connection release with redirection is 12 x TSMTC for FR1.

Proposal 7: For RedCap UE, the lower boundary in Max function for RRC connection release with redirection shall not be changed.

Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson

- R4-2115358, WF on RedCap RRM requirements, Ericsson

- R4-2201189, Simulation results for cell detection, L3 measurement and L1 RSRP measurement