3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201186

Electronic Meeting, January 17-25, 2022

Title: 	Discussion on	signaling characteristics due to UE complexity reduction

Source: 	Huawei, HiSilicon

Agenda item:	6.20.3.1.4

Document for:	Discussion

Introduction

The revised WID on support of reduced capability (RedCap) NR devices has been approved in [1]. RAN4 has started work on the support of RedCap NR devices, where the related study item has been concluded in TR 38.875. The WID has the following objectives on UE complexity reduction:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. - Relaxed maximum modulation order: - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE. - No other relaxations of maximum modulation order are specified for a RedCap UE. - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

It is concluded in RAN#100e meeting that there is no RRM impact due to Maximum number of DL MIMO layers and Relaxed maximum modulation order [2]. This contribution further provides analysis on the signaling characteristics due to reduced UE BW, reduced RX branch.

Discussion

RLM and BFD

- Hypothetical PDCCH parameters for RLM and BFD

Regarding CSI-RS based RLM and BFD, the bandwidth is 48 PRBs in PDCCH transmission parameters tables. One identified issue is 48RBs@ SCS 60kHz exceeds Redcap UE maximum BW in FR1. It is agreed in the last meeting that RAN4 to not introduce requirements for 60 kHz SCS in FR1 for RedCap RLM and BFD requirements [3]. Therefore reduced bandwidth has no impact on RedCap RLM and BFD. In the following the impact due to reduced minimum number of Rx branches are discussed.

To facilitate discussion, the existing PDCCH transmission parameters for out-of-sync and in-sync for SSB based RLM and for CSI-RS based RLM are duplicated as below,

- SSB based RLM Table 8.1.2.1-1: PDCCH transmission parameters for out-of-sync evaluation Attribute; Value for BLER Configuration #0. DCI format; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 8. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 4dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 4dB. Bandwidth (PRBs); 24. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Table 8.1.2.1-2: PDCCH transmission parameters for in-sync evaluation Attribute; Value for BLER Configuration #0. DCI payload size; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 4. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 0dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 0dB. Bandwidth (PRBs); 24. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. - CSI-RS based RLM Table 8.1.3.1-1: PDCCH transmission parameters for out-of-sync evaluation Attribute; Value for BLER Configuration #0. DCI format; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 8. Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy; 4dB. Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy; 4dB. Bandwidth (PRBs); 48. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Table 8.1.3.1-2: PDCCH transmission parameters for in-sync evaluation Attribute; Value for BLER Configuration #0. DCI payload size; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 4. Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy; 0dB. Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy; 0dB. Bandwidth (PRBs); 48. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed.

In R15 (considering 2RX) RLM and BFD test case, we observe Qout around -10dB and Qin around -1.5dB. For RedCap UE, reduction of RX branch from 2RX to 1RX would result in reception gain degradation. In order to guarantee the compatible downlink coverage as 2RX, the same Qout and Qin are supposed to be reused.

To achieve the same SINR level as 2Rx, increasing CCE level and/or power boosting can be considered for 1 RX. It is noted that in current RLM out-of-sync and BFD parameter tables, the ratio of hypothetical PDCCH RE energy to average SSS RE energy for out-of-sync is 4dB, which already touches the upper bound of boosting. Therefore power boosting is not a reasonable way for RLM out-of-sync and BFD. Although there is room for boost the power of in-sync. However in our understanding, the method applied for out-of-sync and in-sync shall be the same. In other words, it is not able to only boost in-sync level.

The simulation assumption [4] on hypothetical PDCCH parameter for RLM with 1RX was agreed at last meeting. Based on the agreed simulation assumption, the simulation results were present in [5]. There are some observations for hypothetical PDCCH mapping with 1RX:

1. SSB based RLM

- For out-of-sync evaluation

As the maximum CCE level for 24RB is 8, the current CCE level has reached the upper bounder. By increasing bandwidth 24RBs to 48RBs, and increasing CCE level from 8 to 16, Qout can reach -10dB. We think the same principle can be applied for BFD.

- For in-sync evaluation

Although the CCE level for in-sync is 4, however it is not reasonable to increase CCE level of in-sync to 8 and the CCE level of out-of-sync keeps unchanged. The method for in-sync shall be aligned with that for out-of-sync. From simulation results, it is observed that through increasing CCE level to 8 and increasing bandwidth to 48 RBs, Qin is about -1.5dB.

Proposal 1: For RedCap UE with 1RX, it is proposed to increase CCE level to 16 and increase bandwidth to 48 RBs in hypothetical PDCCH parameter for

-SSB based out-of-sync evaluation, and

-SSB based beam failure detection

Proposal 2: For RedCap UE with 1RX, it is proposed to increase CCE level to 8 and increase bandwidth to 48 RBs in hypothetical PDCCH parameter for SSB based in-sync evaluation.

2. CSI-RS based RLM

- For out-of-sync evaluation

The legacy bandwidth 48RBs can keep unchanged. By increasing CCE level from 8 to 16, Qout is about -10dB. We think the same principle can be applied for BFD.

- For in-sync evaluation

From simulation results, it is observed that through increasing CCE level from 4 to 8, Qin is about -1.5dB.

Proposal 3: For RedCap UE with 1RX, it is proposed to increase CCE level to 16 in hypothetical PDCCH parameter for

- CSI-RS based out-of-sync evaluation, and

- CSI-RS based Beam failure detection

Proposal 4: For RedCap UE with 1RX, it is proposed to increase CCE level to 8 in hypothetical PDCCH parameter for CSI-RS based in-sync evaluation.

- Evaluation period for RLM and BFD

In current requirements, SSB based RLM evaluation periods for FR1 and FR2 are specified separately in below tables. In non-DRX, 10 samples are needed for monitoring out-of-sync and 5 samples are needed for monitoring in-sync.

Table 8.1.2.2-1: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR1

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2

| Configuration | TEvaluate_out_SSB (ms) | TEvaluate_in_SSB (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(10 P N) TSSB) | Max(100, Ceil(5 P N) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil(15 P N) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil(10 P N) TDRX | Ceil(5 P N) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

In TS38.133, CSI-RS based RLM evaluation periods for FR1 and FR2 are duplicated separately in below tables where Mout = 20 and Min = 10.

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P)×TCSI-RS) | Max(100, Ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P) × TDRX | Ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P×N)×TCSI-RS) | Max(100, Ceil(Min×P×N) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P×N)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P×N)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P×N) × TDRX | Ceil(Min×P×N) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

When defining out-of-sync evaluation period for RLM in R15 (considering 2RX), we observe Qout around side condition -10dB. For 1RX RedCap UE, through increasing CCE, Qout for 1RX can be kept at -10dB as well. From the simulation results in [5], there are some observations for SSB based out-of-sync evaluation period:

- It is up to 1 dB delta-SINR difference between 2RX@ -10dB and 1RX@ -10dB with 10 sample numbers;

- For SSB based RLM, delta-SINR for (1RX @ -10dB, 20 samples) is comparable to that for (2RX@ -10dB, 10 samples).

Proposal 5: For RedCap UE with 1RX, sample number in SSB based RLM OOS evaluation period can be increased from 10 to 20.

From the simulation results in [5], there are some observations for CSI-RS based RLM out-of-sync evaluation period:

- delta-SINR difference between 2RX@ -10dB and 1RX@ -10dB with same sample numbers is within 0.5dB. As the legacy sample number for CSI-RS based out-of-sync is 20 samples, the accuracy gain derived from further increasing sample numbers is insignificant.

Therefore legacy CSI-RS based RLM out-of-sync evaluation period (i.e., 20) can be reused for 1 RX Redcap.

Proposal 6: Legacy CSI-RS based RLM out-of-sync evaluation period (i.e., 20 samples) can be reused for 1 RX Redcap.

In R15 for 2RX we observe Qin-sync around side condition -1.5dB. According to SSB based and CSI-RS based RLM simulation results, delta-SINR accuracy for 1RX is comparable to that for 2RX under high side condition(-1.5dB) for various sample numbers in L1 evaluation period, e.g., 10, 20, 40. Therefore In-sync evaluation sample number can be kept unchanged.

Proposal 7: Legacy SSB and CSI-RS based RLM In-sync evaluation period can be reused for 1 RX Redcap.

The legacy evaluation period for SSB based and CSI-RS based BFD are specified as below. The sample numbers are in yellow highlight. For CSI-RS based BFD, MBFD = 10.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_SSB for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N) TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 P N) Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 P N) TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P PBFD] TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, [1.5 × MBFD P PBFD] Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD P PBFD] TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Table 8.5.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, [MBFD P N PBFD] TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, [1.5 × MBFD P N PBFD] Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD P N PBFD] TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Regarding BFD, during defining out-of-sync evaluation period for BFD in R15 (considering 2RX), Qout is observed around side condition -6dB. From the simulation results in [5], there are some observations:

- for SSB based BFD evaluation

  - It is up to 0.5 dB delta-SINR difference between 2RX@ -6dB and 1RX@ -6dB with 5 sample numbers;

  - The delta-SINR accuracy for 1RX@ -6dB can already satisfy the L1-SINR accuracy.

- for CSI-RS based BFD evaluation

  - It is up to 0.5 dB delta-SINR difference between 2RX@ -6dB and 1RX@ -6dB with 10 sample numbers;

  - The delta-SINR accuracy for 1RX@ -6dB can already satisfy the L1-SINR accuracy.

Therefore the BFD evaluation sample number for both SSB and CSI-RS based BFD can be kept unchanged.

Proposal 8: Legacy SSB and CSI-RS based BFD evaluation period can be reused for 1 RX Redcap.

CBD including L1-RSRP measurement

It is agreed in last meeting [3],

SSB-based L1-RSRP: measurement period without measurement restriction For RedCap UE with 1RX SSB-based L1-RSRP measurement period can be unchanged for FR1 and FFS for FR2. CSI-RS based L1-RSRP (excluding 60 kHz SCS): measurement period without measurement restriction For RedCap UE with 1RX CSI-RS based L1-RSRP measurement period can be unchanged for FR1 and FFS for FR2

Based on the SSB based and CSI-RS based L1-RSRP simulation results shown in [5], the following observations were identified:

- when side condition is -3dB and L1 physical sample is 1 (without measurement restriction), the measurement accuracy with 1RX is about 3dB worse than that of 2 Rx for both FR1 and FR2;

- when side condition is -3dB and L1 physical sample is 3, the measurement accuracy with 1RX is 1dB worse than that of 2 Rx for both FR1 and FR2;

- The same observation is applicable for both SSB based and CSI-RS based L1-RSRP measurement.

As the legacy accuracy requirement for L1-RSRP are based on one sample, the measurement accuracy of L1-RSRP for RedCap UE with 1RX can follow the same principle.

Proposal 9: For RedCap UE with 1RX, the accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB.

TCI state switching

In last meeting, there is one view that new Active TCI state switching delay requirements need to be introduced for Rel-17 RedCap if new L1-RSRP measurement requirements are introduced. According to the progress on L1-RSRP measurement, the L1-RSRP measurement period is not unchanged for RedCap UE. Therefore the legacy TCI state switching can be reused for RedCap UE.

Proposal 10: Legacy TCI state switching can be reused for RedCap UE.

Conclusions

This contribution provides analysis on the signaling characteristics due to complexity reduction for RedCap UE. The following proposals are provided:

Proposal 1: For RedCap UE with 1RX, it is proposed to increase CCE level to 16 and increase bandwidth to 48 RBs in hypothetical PDCCH parameter for

-SSB based out-of-sync evaluation, and

-SSB based beam failure detection

Proposal 2: For RedCap UE with 1RX, it is proposed to increase CCE level to 8 and increase bandwidth to 48 RBs in hypothetical PDCCH parameter for SSB based in-sync evaluation.

Proposal 3: For RedCap UE with 1RX, it is proposed to increase CCE level to 16 in hypothetical PDCCH parameter for

- CSI-RS based out-of-sync evaluation, and

- CSI-RS based Beam failure detection

Proposal 4: For RedCap UE with 1RX, it is proposed to increase CCE level to 8 in hypothetical PDCCH parameter for CSI-RS based in-sync evaluation.

Proposal 5: For RedCap UE with 1RX, sample number in SSB based RLM OOS evaluation period can be increased from 10 to 20.

Proposal 6: Legacy CSI-RS based RLM out-of-sync evaluation period (i.e., 20 samples) can be reused for 1 RX Redcap.

Proposal 7: Legacy SSB and CSI-RS based RLM In-sync evaluation period can be reused for 1 RX Redcap.

Proposal 8: Legacy SSB and CSI-RS based BFD evaluation period can be reused for 1 RX Redcap.

Proposal 9: For RedCap UE with 1RX, the accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB.

Proposal 10: Legacy TCI state switching can be reused for RedCap UE.

Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson

- R4-2115358, WF on RedCap RRM requirements, Ericsson

- R4-2120418, WF on RedCap RRM requirements, Ericsson

- R4-2120386, Simulation assumption on hypothetical PDCCH performance for RLM and BFD for RedCap UE, Huawei, HiSilicon

- R4-2201187, Simulation results for RLM and BFD due to complexity reduction for RedCap UE, Huawei, HiSilicon