3GPP TSG-RAN WG4 Meeting # 101-bis-e												           R4-2201190

Electronic Meeting, January 17-25, 2022

Title: 					Discussion on Extended DRX enhancements for RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	6.20.3.2

Document for:	Discussion

Introduction

The revised WID on support of reduced capability (RedCap) NR devices has been approved in [1]. The WID has the following objectives on introduction of extended DRX:

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]: - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4. - RAN2 to decide which Node(s) configure eDRX in RRC_Idle and RRC_Inactive.

In RAN4 RRM, some discussion on eDRX were carried out at last meeting. This contribution provides further analysis on the measurement requirements for RedCap UE when eDRX is configured.

Discussion

In RAN2, there are some progress on eDRX for Redcap UE. The agreements are duplicated as below.

In RAN2#113bis, the related agreements are,

- RAN decides and configures eDRX via RRC for RRC_INACTIVE (FFS on the need and details of coordination with the CN) - At least for eDRX cycle, the configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different (FFS for PTW, e.g. length and starting point, when eDRX cycles are longer than 10.24s) - RAN2 assumes that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC). - eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC - RAN2 confirms that CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN. - The two parametners are used same as NR R15 - RAN2 confirms that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX)

In RAN2#114e, the related agreements are,

Agreements via email - from offline 110:

- Lower bound for eDRX configuration in RRC_IDLE and RRC_INACTIVE is 2.56 seconds. Inform SA2/CT1 and check if there is any concern.

- It is up to RAN to configure the length for PTW for RAN paging, the RAN PTW length can be different from the CN PTW length.

- When RAN and CN paging coincide in the same PH, the PTW starting locations are the same. FFS how to calculate the PTW starting location so that it is the same for RAN and CN PTW.

Agreements online:

- Continue in the next meeting the discussion on how UE is expected to monitor RAN and CN PTW, e.g. whether UE in RRC_INACTIVE monitors for only RAN PTW or both CN and RAN PTW when they overlap

In RAN2#115e, the related agreements are,

Agreements:

- When IDLE eDRX cycle is longer than 10.24s, PH calculation formula defined in LTE is re-used, i.e.

PH_CN:  H-SFN mod TeDRX,_CN,H= (UE_ID_H mod TeDRX_CN,H)

-  where TeDRX_CN,H is equal to IDLE eDRX cycle.

2.	When IDLE eDRX cycle is longer than 10.24s, CN PTW_end calculation formula defined in LTE is re-used, i.e.

PTW_end is radio frame satisfying SFN = (PTW_start + L*100 - 1) mod 1024,

- where L is PTW length configured by upper layers.

3.	For RRC_IDLE UE, when eDRX cycle is no longer than 10.24s, T is determined by IDLE eDRX cycle. When eDRX cycle is longer than 10.24s, during the CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, and default paging cycle.

4.	For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is not configured, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle.

5.	For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is no longer than 10.24s, outside CN PTW, T is determined by INACTIVE eDRX cycle.

Agreements via email - from offline 105 second round:

- RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured. FFS whether to capture this restriction in RAN2 spec.

- RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle. FFS whether to capture this restriction in RAN2 spec.

- The maximum PTW length is 40.96s when IDLE eDRX cycle is longer than 10.24s.

- The minimum PTW length is 1.28s and the step length/granularity of PTW length is 1.28 when IDLE eDRX cycle is longer than 10.24s.

- Introduce an additional new IE for INACTIVE eDRX to contain all values of INACTIVE eDRX cycles (also include values >10.24, if agreed in future).

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, T is determined by the shortest of IDLE eDRX cycle and INACTIVE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle.

- eDRX feature is optional for any UE (including RedCap and non-RedCap UEs).

Agreements online:

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle.

Option 2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle and default paging cycle.

Option 2: T is determined by RAN paging cycle.

In RAN2#116e, the related agreements are,

Agreements via email - from offline 105: - The max eDRX cycle length for RRC Inactive is 10.24s in Rel-17 - PO determination for non-overlapping CN/RN case is applicable to eDRX - When IDLE eDRX and INACTIVE eDRX are configured and both cycles are no longer than 10.24s, PO is determined by IDLE eDRX. - When IDLE eDRX is configured and is no longer than 10.24s, INACITVE eDRX cycle is not configured, PO is determined by IDLE eDRX. - During CN PTW when IDLE eDRX is configured and longer than 10.24s, and INACTIVE eDRX is configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. - During CN PTW when IDLE eDRX is configure and is longer than 10.24s, INACTIVE eDRX cycle is not configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. - eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case (Further discuss on the reporting of eDRX capability) - The below working agreement is now changed to an agreement. - When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s - PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: - SFN = 1024/N* ieDRX, where - ieDRX = floor(UE_ID_H /TeDRX,H) mod N - FFS N = 4 or 8, FFS if N can take other values - The same LTE hashed UE_ID calculation is used for UE_ID_H for NR. Agreements via email - from offline 105 (second round): - eDRX feature can be supported by non RedCap UEs. - A UE in idle mode requests eDRX configuration via NAS signalling. FFS if capability signalling in RAN, as part of the UE capability message, is also needed. - eDRX support is optional for the RedCap UE. - the UE_ID for eDRX is defined by 5G-S-TMSI mod 4096. - the eDRX acquisition period is the maximum configurable value of the eDRX cycle - No eDRX specific on-demand SI enhancements are considered for Rel-17 Agreements online: - For the eDRX PTW start calculation, agree to N=8. No signalling needed to CN. - The eDRX acquisition period is the same for IDLE and INACTIVE. - A) For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle. - B) For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

Based on these agreements, the measurement requirements for RedCap idle UE and inactive mode when eDRX is configured are analysed respectively.

Cell reselection requirements for idle RedCap UE

Regarding the eDRX cycle length for RRC_IDLE, RAN2 sent an LS to SA2 and CT1 [2] to ask whether it is feasible to specify extended DRX up to 10485.76 s. Both CT1 and SA2 confirmed that it is feasible to support extended DRX cycle in idle mode up to 10485.76 seconds [3, 4].

- Nserv

It is agreed in RAN2 that for RRC_IDLE UE, when eDRX cycle is no longer than 10.24s, T is determined by IDLE eDRX cycle. When eDRX cycle is longer than 10.24s, during the CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, and default paging cycle. It is also agreed in RAN2 that the lower bound for eDRX configuration in RRC_IDLE is 2.56 seconds.

In current NR requirements without eDRX, Nserv are specified in TS 38.133 as below,

Table 4.2.2.2-1: Nserv

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 | M1*N1*4 |
| 0.64 |  | 5 | M1*N1*4 |
| 1.28 |  | 4 | N1*2 |
| 2.56 |  | 3 | N1*2 |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1 or 5, N1 = 8 for all DRX cycle length. |  |  |  |

where M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1.

For idle eDRX cycle 2.56s, 5.12s and 10.24s, Nserv can follow the legacy way, i.e., equals [N1*2]*eDRX cycle length.

For idle eDRX cycle which is longer than 10.24s, UE only monitors paging within PTW. Then the measurement requirements for eDRX shall be defined assuming UE performs measurement according to the DRX cycle within the eDRX PTW, while UE is not required to measure outside PTW.

For FR1, the main principle is to evaluate the signal quality of serving cell within one PTW. When DRX cycle is small, i.e., 0.32s and 0.64s, M1 can still be applied for power saving purpose. The Nserv for UE configured with eDRX_IDLE cycle in FR1 can be specified in Table 1:

Table 1: Nserv for idle UE configured with eDRX_IDLE cycle in FR1

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Nserv [number of DRX or eDRX cycles Note 3] |
| --- | --- | --- | --- |
| 2.56 | N/A | N/A | 2 |
| 5.12 | N/A | N/A | 2 |
| 10.24 | N/A | N/A | 2 |
| 10.24 < eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | 2*M1 |
|  | 0.64 | ≥2.56 (2) | 2* M1 |
|  | 1.28 | ≥2.56 (2) | 2 |
|  | 2.56 | ≥5.12 (4) | 2 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s, number of DRX cycles otherwise. |  |  |  |

Proposal 1: In FR1 for RRC_IDLE RedCap UE, Nserv when eDRX_IDLE cycle is configured can be specified in Table 1.

For FR2, as UE needs to perform RX beam sweeping, the scaling factor N1 is scaled. In order to accurately evaluate serving cell quality and preclude the performance degradation due to the large timing tracking drifting, it is expected that UE can evaluate criterion S in Nserv consecutive DRX cycles within one PTW.

Observation1: it is expected that UE can evaluate criterion S in Nserv consecutive DRX cycles within one PTW in FR2.

In FR2 we don’t think the scaling factor N1 would be changed since these values were sufficiently discussed during R15 NR phase. In addition, UE shall obtain measurement result of serving cell once every N1*DRX cycle, then at least 2 measurement results shall be used for decide whether triggering neighbour cell measurement or not. Nserv can be specified as 2*N1.

There is an observation that in some cases in FR2, the required measurement period (e.g.,2*N1) would exceed PTW length as UE needs to sweep RX beams. One solution is to deprioritize the eDRX requirements for FR2. As in common understanding, FR2 is not precluded in R17 RedCap WI, if we can seek a proper way to solve the issue, it is better to define FR2 related requirements. If Nserv is specified as 2*N1, the lowest configured PTW length shall be 5.12s. RAN4 can develop requirements for the configuration where PTW length is no less than 5.12s in FR2. There are no requirements for smaller than 5.12s cases in FR2. Consequently the Nserv for UE configured with eDRX_IDLE cycle in FR2 can be specified in Table 2:

Table 2: Nserv for idle UE configured with eDRX_IDLE cycle in FR2

| eDRX_IDLE cycle length [s] | Scaling Factor (N1) for FR2 | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Nserv [number of DRX or eDRX cycles Note 3] |
| --- | --- | --- | --- | --- |
| 2.56 | 3 | N/A | N/A | 2*N1 |
| 5.12 | 3 | N/A | N/A | 2*N1 |
| 10.24 | 3 | N/A | N/A | 2*N1 |
| 10.24 < eDRX_IDLE cycle length ≤10485.76 | 8 | 0.32 | ≥5.12 (4) | 2*N1 |
|  | 5 | 0.64 | ≥6.4 (5) | 2*N1 |
|  | 4 | 1.28 | ≥10.24 (8) | 2*N1 |
|  | 3 | 2.56 | ≥15.36 (12) | 2*N1 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s, number of DRX cycles otherwise. |  |  |  |  |

Observation 2: RAN4 to define RedCap RRM requirement with eDRX for both FR1 and FR2.

Proposal 2: In FR2 for RRC_IDLE RedCap UE, Nserv when eDRX_IDLE cycle is configured can be specified in Table 2. And there is no requirements for the configuration where PTW is smaller than 5.12s.

- Intra-frequency and inter-frequency cell reselection requirements

In NR, the cell reselection requirements are same for intra-frequency and inter-frequency. So herein only intra-frequency cell reselection requirements are analysed. In LTE TS36.133, the measurement requirements when eDRX is configured in Idle mode are specified in Table 4.2.2.3-2. The cell reselection requirement are related to PTW and DRX cycles and eDRX cycles. To guarantee measurement and criteria R evaluation are completed in one PTW, the measurement period is defined as one DRX cycle and evaluation period is defined as two DRX cycles.

Table 4.2.2.3-2: Tdetect,EUTRAN_Intra, Tmeasure,EUTRAN_Intra and Tevaluate,E-UTRAN_intra for UE configured with eDRX_IDLE cycle [TS 36.133]

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_Intra [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 5.12 ≤ eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | ≥1.28 (1) | (23) | 0.32 (1) | 0.64 (2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. |  |  |  |  |  |

In R17 NR Redcap, the structure of defining cell reselection requirements under PTW configuration in LTE can be used for RedCap UE as a starting point. However at DRX cycle 0.32s, when PTW is far larger than 1.28s, e.g., 10.28s, UE needs to perform measurements per DRX cycle within PTW. As we know, power consumption is critical for idle mode UE. In order to save power and also guarantee UE to complete measurement and criterion R evaluation within one PTW, we suggest to split PTW to 2 gears for small DRX cycle (0.32s and 0.64s):

-For PTW length =1.28s, UE performs measurements per DRX cycle;

-For PTW length larger than 1.28s, UE performs measurements per 2*DRX cycle.

For other long DRX cycle, as the power saving gain can be achieved by long DRX cycle, there may no need to consider multiple gears.

Observation 3: The following principles are used for defining cell reselection requirements when idle eDRX is configured for idle RedCap UE:

- measurement and criteria R evaluation are supposed to be performed within one PTW, and

- for small DRX cycle (0.32s and 0.64s), split PTW to 2 gears:

-For PTW length =1.28s, UE performs measurements per DRX cycle;

-For PTW length larger than 1.28s, UE performs measurements per 2*DRX cycle.

Table 3 provides one example for Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured in FR1.

Table 3. For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when eDRX_Idle is configured in FR1

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Tdetect,NR_ [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 |  |  |  |
| 2.56 | - | - | 1 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| 5.12 | - | - |  | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - |  | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 10.24< eDRX_IDLE cycle length ≤10485.76 | 0.32 | =1.28 (1) |  | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 x M2 (1 x N1 x M2) | 0.64 x N1 x M2 (2 x N1 x M2) |
|  |  | ≥2.56 (2) |  |  | 0.64 x N1 x M2 (2 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) |
|  | 0.64 | =1.28 (1) |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  |  | ≥2.56 (2) |  |  | 1.28 x N1 (2 x N1) | 2.56 x N1 (4 x N1) |
|  | 1.28 | ≥2.56 (2) |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥5.12 (4) |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Proposal 3: For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured in FR1 can be specified in Table 3.

For FR2, we think the similar principles shall be used, that is,

- to guarantee the validity of measurement results, measurement and criteria R evaluation are supposed to be performed within one PTW, and

- to save power, split PTW to 2 gears for small DRX cycle (0.32s and 0.64s), where longer measurement/evaluation period for larger PTW.

Considering the RX sweeping scaling factor, M2 and the sample number, the applicability of PTW window is re-considered and highlighted in red font. Table 4 provides one example for Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured in FR2.

Table 4. For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured in FR2

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Tdetect,NR_ [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR2 |  |  |  |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 10.24< eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥7.68 (6) & <15.36 (12) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 x M2 (1 x N1 x M2) | 0.64 x N1 x M2 (2 x N1 x M2) |
|  |  | ≥15.36 (12) |  |  | 0.64 x N1 x M2 (2 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) |
|  | 0.64 | ≥6.4 (5) & <12.8 (10) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  |  | ≥12.8 (10) |  |  | 1.28 x N1 (2 x N1) | 2.56 x N1 (4 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Proposal 4: For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured in FR2 can be specified in Table 4.

Cell reselection requirements for inactive RedCap UE

CT1 and SA2 agreed to support eDRX up to 10.24s for RRC inactive in R17. Therefore PH, PTW and H-SFN are not applied for eDRX_inactive. As we know UE in inactive mode shall monitor both CN paging and RAN Paging. RAN2 has reached agreement on how to determine T (which is used to calculate PO and PF). RAN2’s agreements are summarized in Table 5.

Table 5. Summary of RAN2 agreements on T determination

| IDLE eDRX | Inactive eDRX | T |
| --- | --- | --- |
| >10.24 | N/A | Within PTW: Min{UE specific DRX cycle(if configured by upper layer), RAN paging cycle, default paging cycle} |
|  |  | Outside PTW: RAN paging cycle |
|  | <=10.24 | Within PTW: min{UE specific DRX cycle (if configured by upper layer) , INACTIVE eDRX cycle and default paging cycle}. |
|  |  | Outside PTW: INACTIVE eDRX cycle |
| <=10.24 | N/A | min{ RAN paging cycle, IDLE eDRX cycle} |
|  | <=10.24 | min{IDLE eDRX cycle, INACTIVE eDRX cycle} |

In LTE eMTC, inactive DRX can reach to 10.24s (In LTE, there is no definition of inactive eDRX). When idle eDRX is configured, the measurement requirements for inactive UE are defined based on inactive DRX only and without considering PTW. To facilitate the discussion, the requirements for eMTC are duplicated as below.

Table 4A.2.3.1.1-1: Nserv_NC for UE configured with eDRX_IDLE cycle [TS36.133]

| eDRX_IDLE cycle length [s] | DRX_INACTIVE cycle length or [s] | Nserv [number of DRX cycles] |
| --- | --- | --- |
| 5.12 ≤eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | 4 |
|  | 0.64 | 4 |
|  | 1.28 | 2 |
|  | 2.56 | 2 |
|  | 5.12 | 2 |
|  | 10.24 Note 1 | 2 |
| Note 1: DRX_INACTIVE cycle length of 10.24s applies when eDRX_IDLE cycle length is >= 10.24s. |  |  |

Table 4A.2.3.1.2-1: Tdetect,EUTRAN_Intra_NC, Tmeasure,EUTRAN_Intra_NC and Tevaluate,E-UTRAN_Intra_NC for UE configured with eDRX_IDLE cycle [TS36.133]

| eDRX_IDLE cycle length [s] | DRX_INACTIVE cycle length [s] | Tdetect,EUTRAN_Intra_NC [s] (number of DRX cycles) | Tmeasure,EUTRAN_Intra_NC [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra_NC [s] (number of DRX cycles) | Tevaluate,E-UTRAN_intra_NC_RSS [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 5.12 ≤eDRX_IDLE cycle length ≤ 2621.44 | 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) | 3.84 (12) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) | 3.84 (6) |
|  | 1.28 | 32(25) | 1.28 (1) | 6.4 (5) | 3.84 (3) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) | 3.84 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) | 15.36 (3) |
|  | 10.24 Note 1 | 235.52 (23) | 10.24 (1) | 30.72 (3) | 30.72 (3) |
| Note 1: DRX_INACTIVE cycle length of 10.24s applies when eDRX_IDLE cycle length is >= 10.24s. |  |  |  |  |  |

For inactive RedCap UE, the normal inactive DRX cycle length is no longer than 2.56s and the inactive eDRX cycle length is 2.56s, 5.12s and 10.24s. In essence, the NR inactive DRX/eDRX cycle is the same as LTE inactive DRX. In the following, two cases are analysed:

- when idle eDRX is longer than 10.24s

When idle eDRX is longer than 10.24s, there is PTW for idle eDRX. When inactive eDRX is not configured, inactive DRX still exists. When inactive eDRX is configured, the active eDRX is 2.56s, 5.12s or 10.24s and no inactive eDRX related PTW. Therefore the case is quite similar as LTE eMTC. So we propose to follow the eMTC measurement principle: measurement requirements can be defined based on inactive DRX/eDRX, and no PTW is considered.

Proposal 5: When idle eDRX is longer than 10.24s, measurement requirements can be defined based on inactive DRX/eDRX, and no PTW is considered.

- when idle eDRX is no longer than 10.24s

When idle eDRX is no longer than 10.24s, there is no PTW for the UE.

When inactive eDRX is not configured, although RAN2 agreed that T is determined by min{RAN paging cycle, IDLE eDRX cycle}. However from measurement point of view, it is no need for UE to perform measurement on each paging occasion. To align the measurement principle between idle eDRX longer than 10.24s and no longer than 10.24s, we propose that the measurements in this case are defined based on inactive DRX cycle.

When idle eDRX is no longer than 10.24s, and inactive eDRX is configured, RAN2 had reached agreement that T is determined as min{IDLE eDRX cycle, INACTIVE eDRX cycle}. However from measurement perspective, it is no need for UE to perform measurement on each paging occasion. UE can perform measurement only based on inactive eDRX cycle.

Proposal 6: When idle eDRX is no longer than 10.24s, measurement requirements can be defined based on inactive DRX/eDRX cycle.

Consequently the measurement requirements for inactive UE configured with eDRX idle cycle can be specified in Table 6 and Table 7.

Table 6: Nserv for inactive Redcap UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | Nserv [number of DRX or eDRX cycles] |
| --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 4 |
|  | 0.64 | 4 |
|  | 1.28 | 2 |
|  | 2.56 | 2 |
|  | 5.12 | 2 |
|  | 10.24 | 2 |

Table 7: Tdetect, Tmeas and Tevaluate for inactive Redcap UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length Note1 [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX or eDRX cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | FR1 | FR2Note1 |  |  |  |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
|  | 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 |  | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 |  | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |

Proposal 7: For RRC_Inactive RedCap UE, Nserv when eDRX_IDLE cycle is configured can be specified in Table 6.

Proposal 8: For RRC_Inactive RedCap UE, Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured can be specified in Table 7.

Conclusions

This contribution provides analysis on the measurement requirements for RedCap UE when eDRX is configured. According to the analysis, we propose the following proposals.

Proposal 1: In FR1 for RRC_IDLE RedCap UE, Nserv when eDRX_IDLE cycle is configured can be specified in Table 1.

Table 1: Nserv for idle UE configured with eDRX_IDLE cycle in FR1

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Nserv [number of DRX or eDRX cycles Note 3] |
| --- | --- | --- | --- |
| 2.56 | N/A | N/A | 2 |
| 5.12 | N/A | N/A | 2 |
| 10.24 | N/A | N/A | 2 |
| 10.24 < eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | 2*M1 |
|  | 0.64 | ≥2.56 (2) | 2* M1 |
|  | 1.28 | ≥2.56 (2) | 2 |
|  | 2.56 | ≥5.12 (4) | 2 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s, number of DRX cycles otherwise. |  |  |  |

Observation1: It is expected that UE can evaluate criterion S in Nserv consecutive DRX cycles within one PTW.

Observation 2: RAN4 to define RedCap RRM requirement with eDRX for both FR1 and FR2.

Proposal 2: In FR2 for RRC_IDLE RedCap UE, Nserv when eDRX_IDLE cycle is configured can be specified in Table 2. And there is no requirements for the configuration where PTW is smaller than 5.12s.

Table 2: Nserv for idle UE configured with eDRX_IDLE cycle in FR2

| eDRX_IDLE cycle length [s] | Scaling Factor (N1) for FR2 | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Nserv [number of DRX or eDRX cycles Note 3] |
| --- | --- | --- | --- | --- |
| 2.56 | 3 | N/A | N/A | 2*N1 |
| 5.12 | 3 | N/A | N/A | 2*N1 |
| 10.24 | 3 | N/A | N/A | 2*N1 |
| 10.24 < eDRX_IDLE cycle length ≤10485.76 | 8 | 0.32 | ≥5.12 (4) | 2*N1 |
|  | 5 | 0.64 | ≥6.4 (5) | 2*N1 |
|  | 4 | 1.28 | ≥10.24 (8) | 2*N1 |
|  | 3 | 2.56 | ≥15.36 (12) | 2*N1 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s, number of DRX cycles otherwise. |  |  |  |  |

Observation 3: The following principles are used for defining cell reselection requirements when idle eDRX is configured for idle RedCap UE:

- measurement and criteria R evaluation are supposed to be performed within one PTW, and

- for small DRX cycle (0.32s and 0.64s), split PTW to 2 gears:

-For PTW length =1.28s, UE performs measurements per DRX cycle;

-For PTW length larger than 1.28s, UE performs measurements per 2*DRX cycle.

Proposal 3: For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured in FR1 can be specified in Table 3.

Table 3. For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when eDRX_Idle is configured in FR1

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Tdetect,NR_ [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 |  |  |  |
| 2.56 | - | - | 1 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| 5.12 | - | - |  | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - |  | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 10.24< eDRX_IDLE cycle length ≤10485.76 | 0.32 | =1.28 (1) |  | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 x M2 (1 x N1 x M2) | 0.64 x N1 x M2 (2 x N1 x M2) |
|  |  | ≥2.56 (2) |  |  | 0.64 x N1 x M2 (2 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) |
|  | 0.64 | =1.28 (1) |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  |  | ≥2.56 (2) |  |  | 1.28 x N1 (2 x N1) | 2.56 x N1 (4 x N1) |
|  | 1.28 | ≥2.56 (2) |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥5.12 (4) |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Proposal 4: For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when eDRX_Idle is configured in FR2 can be specified in Table 4.

Table 4. For RRC_IDLE RedCap UE, Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured in FR2

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Tdetect,NR_ [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR2 |  |  |  |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 10.24< eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥7.68 (6) & <15.36 (12) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 x M2 (1 x N1 x M2) | 0.64 x N1 x M2 (2 x N1 x M2) |
|  |  | ≥15.36 (12) |  |  | 0.64 x N1 x M2 (2 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) |
|  | 0.64 | ≥6.4 (5) & <12.8 (10) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  |  | ≥12.8 (10) |  |  | 1.28 x N1 (2 x N1) | 2.56 x N1 (4 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |

Proposal 5: When idle eDRX is longer than 10.24s, measurement requirements can be defined based on inactive DRX/eDRX, and no PTW is considered.

Proposal 6: When idle eDRX is no longer than 10.24s, measurement requirements can be defined based on inactive DRX/eDRX cycle.

Proposal 7: For RRC_Inactive RedCap UE, Nserv when eDRX_IDLE cycle is configured can be specified in Table 6.

Table 6: Nserv for inactive Redcap UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Nserv [number of DRX or eDRX cycles] |
| --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 4 |
|  | 0.64 | 4 |
|  | 1.28 | 2 |
|  | 2.56 | 2 |
|  | 5.12 | 2 |
|  | 10.24 | 2 |

Proposal 8: For RRC_Inactive RedCap UE, Tdetect, Tmeas and Tevaluate when Edrx_Idle is configured can be specified in Table 7.

Table 7: Tdetect, Tmeas and Tevaluate for inactive Redcap UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX or eDRX cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | FR1 | FR2Note1 |  |  |  |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
|  | 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 |  | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 |  | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |

References

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson

- R2-2104374, LS on introducing extended DRX for RedCap UEs

- C1-213966, LS R2-2104374 on introducing extended DRX for RedCap UEs from RAN2

- S2-2106978, Reply LS on introducing extended DRX for RedCap UEs