3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201216

Electronic Meeting, January 17-25, 2022

Agenda Item:	6.12.2.1.1

Source:	Ericsson

Title:	Simulation results on PDSCH performance for inter-cell interference

Document for:	Information

# 1	Introduction

In this contribution, the MMSE-IRC for suppressing inter-cells’ interference performance simulation results will be shown. The detailed simulation assumptions are listed in table below.

| Parameters | Serving cell FDD | Serving cell TDD | Interferer cell(s) |
| --- | --- | --- | --- |
| Network scenario | Sync. |  |  |
| Number of carriers | single carrier |  |  |
| Carrier frequency | [2 GHz] | [4GHz] |  |
| PDSCH allocation in frequency domain | Full PRB |  |  |
| Physical cell ID | 0 | 0 | 1 |
| Active DL BWP index | 1 | 1 | N/A |
| Channel bandwidth | 10MHz | 40MHz |  |
| TDD configuration | N/A | 7DS2U, S=6D+4G+4U | 7DS2U, S=6D+4G+4U |
| SCS | 15kHz | 30kHz |  |
| PDCCH allocation in time domain | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH | symbols #0 and #1 of each slot for PDCCH |
| SSB configuration | SSB index #0 | SSB index #0 | SSB index #0 |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port FDD: Periodicity 20 slots; Offset 10 for CSI-RS resource 1 and 2; 11 for CSI-RS resource 3 and 4 TDD: Periodicity 40 slots; Offset 20 for CSI-RS resource 1 and 2; 21 for CSI-RS resource 3 and 4 |  | Same as Serving cell |
| PDSCH configuration | Type A, Start symbol 2, Duration 12​ |  | Type A, Start symbol 2, Duration 12​ |
| PRB bundling size for target PDSCH | 2 |  | 2 |
| PDSCH DMRS configuration | Type 1, 1+1 |  | Type 1, 1+1 |
| Scheduling with data multiplexed on the DMRS symbols | One single symbol front loaded and 1 additional DMRS |  | One single symbol front loaded and 1 additional DMRS |
| MCS | MCS 13 (16QAM, CR=0.5) |  | 16QAM |
| Rank | 1 |  | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| PDSCH precoder | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |  |  |
| Antenna configuration | 2x2, 2x4 |  |  |
| Antenna Correlation | ULA Low |  |  |
| Propagation condition | TDLA30-10 and TDLC300-100 |  |  |
| Time Offset relative to serving cell | 0 us |  |  |
| Metric | SINR to achieve 70% of maximum throughput |  |  |
| Number of HARQ Processes | 4 | 8 | N/A |
| Maximum HARQ transmission | 4 |  | N/A |

| INR scenario | INR (dB) for cell 2 (first interferer cell) | INR (dB) for cell 3 (second interferer cell) | Serving cell SNR (dB) |
| --- | --- | --- | --- |
| Scenario 1 | 5.43 | -1.50 | [-6:26] |
| Scenario 2 | 3.1 | N/A | [-6:26] |
| Scenario 3 | 7.77 | 2.29 | [-6:26] |
| Scenario 4 | 13.91 | 3.34 | [-6:26] |
| Scenario 5 | 11.39 | 5.45 | [-6:26] |

# 2	Simulation Results

The simulation results based on current agreements are summarized as follow.

Table 1. simulation results for FR1 FDD

| Parameters |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex mode | Channel model | Number of UE Rx antenna | INR1 | INR2 | MCS | MMSE-MRC | MMSE-IRC | MMSE-IRC Gain |
| FDD (15 kHz, 10 MHz) | TDLA | 2 Rx | 5,43 | -1,50 | MCS 4 | 3,4 | 1,7 | 1,7 |
|  |  |  |  |  | MCS 13 | 10,6 | 8,9 | 1,7 |
|  |  |  | 3,10 | N/A | MCS 4 | 1,2 | -0,2 | 1,4 |
|  |  |  |  |  | MCS 13 | 8,4 | 7,1 | 1,3 |
|  |  |  | 7,77 | 2,29 | MCS 4 | 5,5 | 3,3 | 2,2 |
|  |  |  |  |  | MCS 13 | 12,7 | 10,5 | 2,2 |
|  |  |  | 13,91 | 3,34 | MCS 4 | 9,8 | 4,9 | 4,9 |
|  |  |  |  |  | MCS 13 | 17,0 | 12,2 | 4,8 |
|  |  |  | 11,39 | 5,45 | MCS 4 | 8,4 | 5,2 | 3,2 |
|  |  |  |  |  | MCS 13 | 15,6 | 12,5 | 3,1 |
|  |  | 4 Rx | 5,43 | -1,50 | MCS 4 | 1,9 | -0,7 | 2,6 |
|  |  |  |  |  | MCS 13 | 8,5 | 5,9 | 2,6 |
|  |  |  | 3,10 | N/A | MCS 4 | -0,6 | -3,2 | 2,6 |
|  |  |  |  |  | MCS 13 | 6,0 | 3,5 | 2,5 |
|  |  |  | 7,77 | 2,29 | MCS 4 | 4,0 | 0,7 | 3,3 |
|  |  |  |  |  | MCS 13 | 10,5 | 7,2 | 3,3 |
|  |  |  | 13,91 | 3,34 | MCS 4 | 7,8 | 1,9 | 5,9 |
|  |  |  |  |  | MCS 13 | 14,1 | 8,3 | 5,8 |
|  |  |  | 11,39 | 5,45 | MCS 4 | 7,0 | 2,2 | 4,8 |
|  |  |  |  |  | MCS 13 | 13,4 | 8,7 | 4,7 |
|  | TDLC | 2 Rx | 5,43 | -1,50 | MCS 4 | 3,3 | 1,8 | 1,5 |
|  |  |  |  |  | MCS 13 | 11,2 | 9,8 | 1,4 |
|  |  |  | 3,10 | N/A | MCS 4 | 1,1 | -0,2 | 1,3 |
|  |  |  |  |  | MCS 13 | 9,1 | 7,8 | 1,3 |
|  |  |  | 7,77 | 2,29 | MCS 4 | 5,5 | 3,5 | 2,0 |
|  |  |  |  |  | MCS 13 | 13,4 | 11,6 | 1,8 |
|  |  |  | 13,91 | 3,34 | MCS 4 | 9,9 | 6,4 | 3,5 |
|  |  |  |  |  | MCS 13 | 17,8 | 14,7 | 3,1 |
|  |  |  | 11,39 | 5,45 | MCS 4 | 8,5 | 5,9 | 2,6 |
|  |  |  |  |  | MCS 13 | 16,4 | 14,1 | 2,3 |
|  |  | 4 Rx | 5,43 | -1,50 | MCS 4 | 2,6 | 0,7 | 1,9 |
|  |  |  |  |  | MCS 13 | 10,0 | 8,2 | 1,8 |
|  |  |  | 3,10 | N/A | MCS 4 | 0,0 | -1,8 | 1,8 |
|  |  |  |  |  | MCS 13 | 7,5 | 5,8 | 1,7 |
|  |  |  | 7,77 | 2,29 | MCS 4 | 4,7 | 2,3 | 2,4 |
|  |  |  |  |  | MCS 13 | 11,9 | 9,8 | 2,1 |
|  |  |  | 13,91 | 3,34 | MCS 4 | 9,0 | 4,8 | 4,2 |
|  |  |  |  |  | MCS 13 | 15,8 | 12,0 | 3,8 |
|  |  |  | 11,39 | 5,45 | MCS 4 | 7,8 | 4,5 | 3,3 |
|  |  |  |  |  | MCS 13 | 14,8 | 11,8 | 3,0 |

Observation 1: Compared with MRC, IRC receiver can have about 2dB or even more gain in 70% of maximum throughput SNR point.

Observation 2: IRC receiver have gains in both TDLA and TDLC channels.

Detail simulation results are shown in the appendix.

# References

- R4-2120707, “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation.

# Appendix

- FR1 FDD

TDL-A

TDL-C