3GPP TSG-RAN WG4 Meeting # 101-bis-e                           R4-2201255

Electronic Meeting, 17– 25 January, 2022

Source: 	Huawei, HiSilicon

Title: 	NR NTN co-existence simulation Results

Agenda Item:	6.13.2.1

Document for:	Discussion

Introduction

In RAN4#101-e meeting, some companies provided some initial simulation results for NR NTN co-existence simulations. And the NTN simulation assumptions [1] were further modified. The unified formal for summary [2] of NTN co-existence study can be used to align the final results for companies. In this paper, we’d like to provide our NTN simulation results.

Discussion on the scenarios

For TN-NTN adjacent coexistence study, RAN4 don’t need to trade off the ACLR and ACS requirements when the specific ACIR value was defined. Because the ACLR and ACS requirements for terrestrial network have been specified. The specific analysis is shown below.

Table 1 consideration on TN-NTN adjacent coexistence scenario

|  | TN BS Victim | TN UE Victim | Satellite Node Victim | Satellite UE Victim |
| --- | --- | --- | --- | --- |
| Satellite Node aggressor | Sate ACLR = TBD BS ACS = 46dB | Sate ACLR = TBD UE ACS = 33dB |  |  |
| Satellite UE aggressor | SateUE ACLR = TBD BS ACS = 46dB | SateUE ACLR = TBD UE ACS = 33dB |  |  |
| TN BS aggressor | NA | NA | Sate ACS = TBD BS ACLR = 45dB | SateUE ACS = TBD BS ACLR = 45dB |
| TN UE aggressor | NA | NA | Sate ACS = TBD UE ACLR = 30dB | SateUE ACS = TBD BS ACLR = 30dB |

Based on the simulation assumption [1] for NTN coexistence study, there are 6 scenarios in study phase 1 to derive the ACIR values. It’s easy to connect these six scenarios with the specific ACLR/ACS requirements for satellite access node and user equipment for satellite access as below.

Table 2 Aggressor and victim

| No. | Combination | Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | BS 45dB ACLR | UE for satellite access ACS=TBD |
| 2 | TN with NTN | TN UL | NTN UL | UE 30dB ACLR | satellite access node ACS=TBD |
| 3 | TN with NTN | NTN DL | TN DL | satellite access node ACLR=TBD | UE 33dB ACS |
| 4 | TN with NTN | NTN UL | TN UL | UE for satellite access ACLR=TBD | BS 46dB ACS |
| 5 | TN with NTN | NTN UL | TN DL | UE for satellite access ACLR=TBD | UE 33dB ACS |
| 6 | TN with NTN | TN DL | NTN UL | BS 45dB ACLR | satellite access node ACS=TBD |

For the 6 scenarios, the targeted ACLR/ACS RF requirements for satellite access can be found in table 2.

Simulation results for NR NTN coexistence study

Case 1 Aggressor TN DL and Victim NTN DL

| Case 1: Aggressor AAS Urban to Victim LEO600 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 28 | 30 | 32 | 34 |
| Throughput Loss | Average | 4.46% | 3.28% | 2.27% | 1.67% |
|  | 5%-tile | 6.54% | 5.83% | 4.97% | 3.59% |

| Case 1: Aggressor AAS Urban to Victim LEO1200 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 28 | 30 | 32 |  |
| Throughput Loss | Average | 4.94% | 4.25% | 3.72% |  |
|  | 5%-tile | 6.02 % | 5.18% | 4.23% |  |

Observation 1: For case 1, the ACIR can be 32dB.

Case 2 Aggressor TN UL and Victim NTN UL

| Case 2: Aggressor AAS Rural to Victim LEO600 |  |  |  |
| --- | --- | --- | --- |
| Required ACIR [dB] |  | 28 | 30 |
| Throughput Loss | Average | 5.43% | 3.24% |
|  | 5%-tile | 6.86% | 4.67% |

| Case 2: Aggressor AAS Rural to Victim LEO1200 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 4.41% | 3.46% | 2.41% |
|  | 5%-tile | 4.82% | 3.97% | 2.52% |

| Case 2: Aggressor AAS Urban to Victim LEO600 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.64% | 0.43% | 0.33% |
|  | 5%-tile | 0.66% | 0.45% | 0.37% |

| Case 2: Aggressor AAS Urban to Victim LEO1200 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 2.53% | 1.25% | 0.53% |
|  | 5%-tile | 2.75% | 1.52% | 0.65% |

Observation 2: For case 2, the ACIR can be 28~30dB.

Case 3 Aggressor NTN DL and Victim TN DL

| Case 3: Aggressor LEO600 to Victim AAS Urban |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 18 | 20 | 22 | 24 | 26 |
| Throughput Loss | Average | 0.11% | 0.04% | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.45% | 0.14% | 0.02% | 0.00% | 0.00% |

| Case 3: Aggressor LEO1200 to Victim AAS Urban |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 18 | 20 | 22 | 24 | 26 |
| Throughput Loss | Average | 0.09% | 0.02% | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.23% | 0.05% | 0.00% | 0.00% | 0.00% |

| Case 3: Aggressor LEO600 to Victim AAS Rural |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 18 | 20 | 22 | 24 | 26 |
| Throughput Loss | Average | 5.94% | 3.97% | 2.52% | 1.64% | 1.24% |
|  | 5%-tile | 8.61% | 5.01% | 3.12% | 1.94% | 1.32% |

| Case 3: Aggressor LEO1200 to Victim AAS Rural |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 18 | 20 | 22 | 24 | 26 |
| Throughput Loss | Average | 1.90% | 1.24% | 0.81% | 0.57% | 0.29% |
|  | 5%-tile | 3.86% | 2.20% | 1.93% | 0.92% | 0.85% |

Observation 3: For case 3, the worst scenario is Aggressor LEO600 to Victim AAS Rural and ACIR can be 20dB.

Case 4 Aggressor NTN UL and Victim TN UL

| Case 4: Aggressor LEO600 to Victim AAS Urban |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.00% | 0.00% | 0.00% |

| Case 4: Aggressor LEO1200 to Victim AAS Urban |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.00% | 0.00% | 0.00% |

| Case 4: Aggressor LEO600 to Victim AAS Rural |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.23% | 0.18% | 0.12% |
|  | 5%-tile | 0.63% | 0.52% | 0.40% |

| Case 4: Aggressor LEO1200 to Victim AAS Rural |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.23% | 0.20% | 0.17% |

Observation 4: For case 4, the ACIR can be less than 30dB.

Case 5 Aggressor NTN UL and Victim TN DL

| Case 5: Aggressor LEO600 to Victim AAS Urban |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.00% | 0.00% | 0.00% |

| Case 5: Aggressor LEO1200 to Victim AAS Urban |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.00% | 0.00% | 0.00% |

| Case 5: Aggressor LEO600 to Victim AAS Rural |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.00% | 0.00% | 0.00% |

| Case 5: Aggressor LEO1200 to Victim AAS Rural |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 26 | 28 | 30 |
| Throughput Loss | Average | 0.00% | 0.00% | 0.00% |
|  | 5%-tile | 0.14% | 0.09% | 0.06% |

Observation 5: For case 5, the ACIR can be less than 28dB.

Case 6 Aggressor TN DL and Victim NTN UL

| Case 6: Aggressor AAS Rural to Victim LEO600 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 32 | 34 | 36 | 38 | 40 |
| Throughput Loss | Average | 12.48% | 7.94% | 3.93% | 3.41% | 2.83% |
|  | 5%-tile | 12.56% | 8.38% | 6.82% | 4.53% | 3.31% |

| Case 6: Aggressor AAS Rural to Victim LEO1200 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 32 | 34 | 36 | 38 | 40 |
| Throughput Loss | Average | 8.69% | 7.36% | 5.76% | 2.25% | 1.44% |
|  | 5%-tile | 9.33% | 8.92% | 6.80% | 3.38% | 2.14% |

| Case 6: Aggressor AAS Urban to Victim LEO600 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 34 | 36 | 38 |
| Throughput Loss | Average | 0.87% | 0.57% | 0.32% |
|  | 5%-tile | 1.13% | 0.94% | 0.54% |

| Case 6: Aggressor AAS Urban to Victim LEO1200 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required ACIR [dB] |  | 34 | 36 | 38 |
| Throughput Loss | Average | 0.66% | 0.62% | 0.45% |
|  | 5%-tile | 0.84% | 0.69% | 0.67% |

Observation 6: For case 6, the ACIR can be 36~38dB.

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Observation 1: For case 1, the ACIR can be 32dB.

Observation 2: For case 2, the ACIR can be 28~30dB.

Observation 3: For case 3, the worst scenario is Aggressor LEO600 to Victim AAS Rural and ACIR can be 20dB.

Observation 4: For case 4, the ACIR can be less than 30dB.

Observation 5: For case 5, the ACIR can be less than 28dB.

Observation 6: For case 6, the ACIR can be 36~38dB.

- References

- R4-2120671	Simulation assumptions for NTN co-existence Samsung

- R4-2120749	Summary of NTN co-existence study Samsung

- WF on NTN UE RF requirements, Huawei, HiSilicon

- R4-2120740	Email discussion summary for [101-e][309] NTN_Solutions_Part1, Moderator (Thales)

- Recommendation  ITU-R  SM.329-12 Unwanted emissions in the spurious domain

- ERC Recommendation 74-01 Unwanted emissions in the spurious domain