3GPP TSG-RAN WG4 Meeting # 101-bis-e                                                                                   R4-2201257

Electronic Meeting, 17– 25 January, 2022

Agenda item:	6.13.1.1

Source:	Huawei

Title:	TP for 38.863 on system parameters to clarify “NTN satellite bands”

Release:	Rel-17

Document for:	Approval

1	Introduction

Some requirements related to the system parameters have been included in the latest TR [1] for the satellite communications since RAN4#101 meeting. However, the NTN bands may cause some misunderstanding since R17 NTN topic includes three main scenario satellite communication, HAPS and ATG. In RAN4’s TR or TS, it’s better to differentiate these scenarios accurately. In last meeting, “NTN satellite band” was agreed to highlight the bands which are used for satellite communications. Thus, this TP is to clarify the “NTN satellite bands” in system parameters’ clause.

2	References

R4-2120776	Draft TR for 38.863 v0.1.0, Samsung

Text Proposal

<< Start of text proposal for TR 38.863>>

7.2	Common issues for satellite access node and NTN UE

7.2.1	Operating bands

The following bands in Table 7.2.1-1 are agreed as exemplary NTN satellite bands in Rel-17. Regarding the band numbering for NTN satellite bands, it is agreed to start from the largest band number in FR1 range for NTN satellite bands which fully within FR1 frequency ranges to better differentiate NTN satellite bands and TN bands and ensure contiguous band number allocation for NTN satellite bands, the number can be taken in a decreased order with first come, first service.

Table 7.2.1-1: NTN satellite bands in FR1

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |

7.2.2	Channel bandwidth, SCS and spectral utilization

[The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in TS 38.104[15] and TS 38.101-1[16] can be reused for NTN system].

The supported channel bandwidths per operating band should be defined based on NTN operator input. The details could be found in the Table 7.2.2-1.

Table 7.2.2-1:  Channel bandwidths and SCS per NTN satellite operating band in FR1

| NTN satellite band # | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes |
| n255 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes |
| n256 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |

7.2.3	Channel raster and sync raster

7.2.3.1	Analysis

In order to be compatible with current design of sync raster below 3 GHz, 100 kHz was chosen as the channel raster for NR NTN satellite bands below 3 GHz.

<< End of text proposal >>