3GPP TSG-RAN WG4 Meeting #101-bis-e		R4-2201283
Electronic Meeting, 17th Jan. – 25th Jan., 2022

Agenda item:	6.2.3

Source:	OPPO

Title:	On hand phantom selection for Rel-17 requirement

Document for:	Approval

1	Introduction

In the last meeting of RAN4 #101-e, the use case for Rel-17 requirement development is down-selected to Hand Phantom Only as the WF [1] shown below. However, for hand phantom, whether PDA hand or Wide hand is used for TRP TRS requirement definition need to be concluded.

This paper shares our views on hand phantom selection requirement definition.

2	Discussion

During GTW discussion on RAN4 #100-e, the selection of PDA hand or Wide hand is warmly discussed. There are supporters for both PDA hand and Wide hand. To reiterate our preference, wide hand is the first priority for Rel-17 requirement, based on the following considerations.

- Over 98.9% of 5G commercial smartphones launched in recent one year have the device width bigger than 72mm from Chinese mobile device market survey. So the clear majority of commercial smartphones fits for the Wide hand phantom.

- Currently, the quantity of the smartphones with its width less than 72mm does not exceed 5, around the world. This will in fact cause the embarrassing situation on lab alignment and requirement development, if PDA hand is selected. Apparently, we can not get a smooth CDF curve with less than 5 test data.

- There is no big difference on effort spent between using PDA hand or Wide hand when developing TRP TRS requirement. However, requirement for Wide hand will cover most of commercial devices while requirement for PDA hand can not be used for most devices. Therefore, defining TRP TRS requirement for Wide hand is the most efficient way.

Proposal 1: Wide hand is the first priority for Rel-17 TRP TRS performance requirement.

If the conclusion is that PDA hand is selected for Rel-17 requirement development, it is recommended RAN4 to consider new procedure to derive performance requirement rather than traditional CDF curve.

Proposal 2: If PDA hand is selected for Rel-17 requirement development, RAN4 need to consider new procedure to derive performance requirement rather than traditional CDF curve.

3	Conclusion

Proposal 1: Wide hand is the first priority for Rel-17 TRP TRS performance requirement.

Proposal 2: If PDA hand is selected for Rel-17 requirement development, RAN4 need to consider new procedure to derive performance requirement rather than traditional CDF curve.

References

[1] R4-2120689 WF on FR1 TRP TRS