3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201296

Online meeting, 17-25 Jan, 2022

Source:	Xiaomi

Title:	TP for TR 38.862 on the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

Agenda item:			5.27.1

Document for:	Approval

1. Introduction

This contribution is a text proposal for TR 38.862 v0.3.0 to include rules of the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5.

2. Text Proposal

---Start of changes---

5.x      The rules for band combination with BCS4/BCS5

5.x.2       The maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

To guarantee the BCS4/BCS5 can cover all the possible bandwidth configurations for intra-band CA, the maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to

- min{n*max channel bandwidth of each carrier, BWChannel_CA of each CA bandwidth class, Maximum frequency range of each band} for intra-band contiguous CA.

- min{ n*max channel bandwidth of each carrier, Maximum frequency range of each band - Minimum sub-block gaps} for intra-band non-contiguous CA.

Where n is the number of aggregated CCs, minimum sub-block gaps indicates the sum of the min sub-block gap between the upper edge of lower component carrier and lower edge of higher component carrier that UE can support per band combination in two adjacent non-contiguous component carriers.

The value of min sub-block gaps could be clarified by the request operator but it should try to cover the needs of all possible operators.

---End of changes---