3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201319

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			NTN: Satellite Access node RF Rx conducted

Agenda item:		6.13.3.4

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4 meeting, a Way Forwards related to BS RF Rx requirements ([3]) was agreed. This contribution is further discussing the reference sensitivity requirement for satellite node.

# Discussion

## Reference sensitivity

Following the agreements from last RAN4#101-e meeting ([3]), we would make the following proposal assuming:

- 7.4 dB noise figure for GEO and 4.3 dB for LEO600/LEO1200.

- 2 dB implementation margin.

- SNR would be the same for NTN and TN as the same FRC will be used.

Proposal1: Specify the following satellite access node REFSENS requirements:

For GEO satellite:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -99.3 |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -99.4 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -96.5 |
| 20 | 15 | G-FR1-A1-4 (Note 1) | -92.9 |
| 20 | 30 | G-FR1-A1-5 (Note 1) | -93.2 |
| 20 | 60 | G-FR1-A1-6 (Note 1) | -93.3 |

For LEO600 and LEO1200 satellite:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -102.4 |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -102.5 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -99.7 |
| 20 | 15 | G-FR1-A1-4 (Note 1) | -96.1 |
| 20 | 30 | G-FR1-A1-5 (Note 1) | -96.3 |
| 20 | 60 | G-FR1-A1-6 (Note 1) | -96.4 |

## Dynamic Range

In last RAN4#101-e, following proposals were for further investigation:

    - define IoT level as 18dBc for LEO600KM NTN BS;

    - define IoT level as 12dBc for LEO1200KM NTN BS;

    - not to define Rx dynamic range requirements for GEO NTN BS;

Unfortunately, we haven’t had time to check those values and would like to come  back in next meeting before concluding on this dynamic range requirement.

## ACS

Based on our simulation results ([4]), we suggest to specify ACS with value 32dB.

Proposal2: Based on our simulation results, the satellite access node ACS should be specified with 32dB value.

# Conclusion

In this contribution, we further discussed the Satellite Access Node REFSENS requirement and made the following proposals:

Proposal1: Specify the following satellite access node REFSENS requirements:

For GEO satellite:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -99.3 |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -99.4 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -96.5 |
| 20 | 15 | G-FR1-A1-4 (Note 1) | -92.9 |
| 20 | 30 | G-FR1-A1-5 (Note 1) | -93.2 |
| 20 | 60 | G-FR1-A1-6 (Note 1) | -93.3 |

For LEO600 and LEO1200 satellite:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -102.4 |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -102.5 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -99.7 |
| 20 | 15 | G-FR1-A1-4 (Note 1) | -96.1 |
| 20 | 30 | G-FR1-A1-5 (Note 1) | -96.3 |
| 20 | 60 | G-FR1-A1-6 (Note 1) | -96.4 |

Proposal2: Based on our simulation results, the satellite access node ACS should be specified with 32dB value.

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2120675, NTN - WF on Rx BS RF requirements for satellite access node, Ericsson

- R4-2201317, NTN system simulations – ACLR/ACS, Ericsson