3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201321

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			NTN: Satellite Access node RF Rx OTA

Agenda item:		6.13.3.2

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4 meeting, a Way Forward related to satellite access node RF Rx requirements ([3]) was agreed. No OTA requirement has been discussed so far but at least some of them are needed to finalize the specifications of the satellite access node type 1-H.

In the following contribution, we are checking which OTA Rx requirements would be needed depending on the satellite access node type, and how it could be derived from the conducted requirement when relevant.

# Discussion

Following Table 1 lists all OTA Rx requirements and their applicability for satellite access node type 1-H and 1-O.

| Requirement | BS type 1-H | BS type 1-O |
| --- | --- | --- |
| OTA sensitivity | Yes | Yes |
| OTA reference sensitivity level |  |  |
| OTA dynamic range |  |  |
| OTA in-band selectivity and blocking |  |  |
| OTA out-of-band blocking | NA |  |
| OTA receiver spurious emission |  |  |
| OTA receiver intermodulation |  |  |
| OTA in-channel selectivity |  |  |
| Radiated performance requirements |  |  |

Table 1: OTA Rx requirements applicability for NTN satellite access node type 1-H and 1-O

Observation1: OTA sensitivity requirement shall be specified for NTN satellite access node type 1-H.

Following Table 2 describes, for each OTA Rx requirement, how the requirement should be specified, comparing with the corresponding conducted requirement when relevant.

| Requirement | Way Forward |
| --- | --- |
| OTA sensitivity | Based on manufacturer declaration and the declared directions. |
| OTA reference sensitivity level | Same limits than 1-H but adjusted with NTN ΔOTAREFSENS |
| OTA dynamic range | Same limits than 1-H but adjusted with NTN ΔOTAREFSENS |
| OTA ACS | Similar to 1-H but based on NTN EISminSENS adjusted with NTN ΔminSENS |
| OTA blocking | Similar to 1-H but adjusted with NTN ΔOTAREFSENS for intererfer as well |
|  | Based on NTN EISminSENS adjusted with NTN ΔminSENS for the interferer |
| OTA out-of-band blocking | Similar to 1-H but based on NTN EISminSENS |
| OTA receiver spurious emission | Same limits than 1-H |
| OTA receiver intermodulation | Not applicable for NTN 1-H and 1-O |
| OTA in-channel selectivity | Similar to 1-H adjusted with NTN ΔminSENS for both wanted signal and interefer values |

Table 2: OTA Rx requirements specification

Observation2: Satellite access node OTA Rx requirements could be specified following a similar approach than for TN OTA Rx requirements.

Based on those observations, we make the following proposal:

Proposal: Specify satellite access node OTA Rx requirements for type 1-H and 1-O following a similar approach as for TN OTA Rx requirements, as proposed in Table 2.

# Conclusion

In this contribution, we further discussed the satellite access node RF OTA Tx requirements. We made the following observations and proposal:

Observation1: OTA sensitivity requirement shall be specified for NTN satellite access node type 1-H.

Observation2: Satellite access node OTA Rx requirements could be specified following a similar approach than for TN OTA Rx requirements.

Proposal: Specify satellite access node OTA Rx requirements for type 1-H and 1-O following a similar approach as for TN OTA Rx requirements, as proposed in Table 2.

| Requirement |  |
| --- | --- |
| OTA sensitivity | Based on manufacturer declaration and the declared directions. |
| OTA reference sensitivity level | Same limits than 1-H but adjusted with NTN ΔOTAREFSENS |
| OTA dynamic range | Same limits than 1-H but adjusted with NTN ΔOTAREFSENS |
| OTA ACS | Similar to 1-H but based on NTN EISminSENS adjusted with NTN ΔminSENS |
| OTA blocking | Similar to 1-H but adjusted with NTN ΔOTAREFSENS for intererfer as well |
|  | Based on NTN EISminSENS adjusted with NTN ΔminSENS for the interferer |
| OTA out-of-band blocking | Similar to 1-H but based on NTN EISminSENS |
| OTA receiver spurious emission | Same limits than 1-H |
| OTA receiver intermodulation | Not applicable for NTN 1-H and 1-O |
| OTA in-channel selectivity | Similar to 1-H adjusted with NTN ΔminSENS for both wanted signal and interefer values |

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2120675, WF on Rx RF requirements for satellite access node, Ericsson