TSG-RAN Working Group 4 (Radio) meeting #101bis-e	R4-2201333

Electronic Meeting, 17th – 25th January 2022

Source:	Ericsson

Title:	TP to TR 38.844: Section 6.5.1 Overlapping CA (two cells)

Agenda item:	7.2.3.3.2

Document for:	Approval

# 1	Introduction

In last RAN4#101-e meeting, an editor note was added in section 6.5.1 mentioning some time indication to setup 2 CSI-RS measurements would be provided later. Unfortunately, we didn’t have time to investigate this further. Also, we don’t think this is essential in the scope of this study, this might be further investigated later, during the Work Item phase.

To not delay further the study, the following text is proposed for completeness.

# 2	Text Proposal

#### 6.5.1	Overlapping CA (two cells)

CSI-RS measurement and reporting for the component carriers are specified in TS 38.213 and to be performed independently per-carrier, similarly for PDCCH.  It is up to the gNB scheduler to avoid collisions of different transmissions for the overlapping part with overlapping CA cross-carrier scheduling.

UE capability signalling does not provide any possibility for UE to indicate support of overlapped CA.  New signalling would be required in order for this approach to become feasible.

Note: An indication of the time needed to setup two CSI-RS measurements could be investigated during the work item phase.

# 4	References

[1]	TR 38.844, Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths.