3GPP TSG-RAN WG4 Meeting #101bis-e                                                    R4-2201353

Electronic Meeting, Jan. 17-25, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-11-21:  DC_20A_n3A-n78A

Agenda Item:	5.19.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on  DC_20A_n3A-n78A for TR37.717-11-21[1] .

- Reference

- TR 37.717-11-21  Dual Connectivity (DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations, v0.7.0

- Text Proposal

----- Start of TP -----

  - 6.x			DC_20_n3-n78

    - 6.x.1	Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n3-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1710 MHz | – | 1785 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 6.x.2	Configuration for DC

Table 6.x.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A | 20A | CA_n3A-n78A |

    - 6.x.3	Co-existence studies

In terms of the co-existence analysis of DC_20_n3 UL and DC_20_n78 in TR 37.863-01-01 and TR 37.717-11-21:

- IMD5 produce caused by DC_20_n3 UL may fall into NR Band n78 DL.

- IMD3 produce caused by DC_20_n78 UL fall into NR Band n3 DL.

    - 6.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are proposed to be the same with DC_3A_n20A-n78A, which are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n3-n78 | 20 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_20_n3-n78 | 20 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

    - 6.x.5	MSD

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.x.5-1.

Table 6.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3500 | 10 | 52 | 3500 | 4.5 | IMD5 |

----- End of TP -----