3GPP TSG-RAN WG4 Meeting #101bis-e                                                    R4-2201357

Electronic Meeting, Jan. 17-25, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-21-11:  DC_7A-38A_n78A

Agenda Item:	5.15.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on  DC_7A-38A_n78A for TR37.717-21-11[1] .

- Reference

- TR 37.717-21-11, Rel-17 Dual Connectivity (DC) of 2 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.7.0

- Text Proposal

----- Start of TP -----

  - 5.x			DC_7-38_n78

    - 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_7A-38A_n78A17,18 | N/A | N/A | n78A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

    - 5.x.2	Co-existence studies

No need to analysis the co-existence studies for this configuration.

    - 5.x.3	∆TIB and ∆RIB values

For DC_7-38_n78, same TIB,c and RIB,c values are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-38_n78 | n78 | 0.8 |

Table 5.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-38_n78 | 7 | 0 |
|  | 38 | 0 |
|  | n78 | 0.5 |

    - 5.x.4	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

----- End of TP -----