3GPP TSG-RAN WG4 Meeting #101bis-e                                                    R4-2201359

Electronic Meeting, Jan. 17-25, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.717-31-11: DC_7A-20A-38A_n78A

Agenda Item:	5.16.1

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC_7A-20A-38A_n78A for TR37.717-31-11[1] .

- Reference

- TR 37.717-31-11 Rel-17 DC combinations LTE 3DL and one NR band v0.7.0

- Text Proposal

----- Start of TP -----

## 5.1.x	DC_7-20-38_n78

### 5.1.x.1	Configurations for EN-DC

Table 5.2B.4.4-1: Inter-band EN-DC configurations (four bands)

| EN-DC band configuration | UL EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_7A-20A-38A_n78A10 | DC_20A_n78A | 20A | n78A |
| NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |  |  |

### 5.1.x.2	∆TIB and ∆RIB values

For DC_7-20-38_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.x.2-1: ΔTIB,c

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-38_n78 | 20 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.x.2-2: ΔRIB,c

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-38_n78 | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.4 |
|  | n78 | 0.6 |

### 5.1.x.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

----- End of TP -----