3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201391

Online Meeting, 17 January – 25 January 2022

Source:	ZTE Corporation

Title:	Measurement procedure of RedCap UEs

Agenda item:	6.20.3.1.5

Document for:	Discussion

# 1	Introduction

In RAN4 101-e, there were discussions on measurement procedures for RedCap UEs.

We refer to both the WF [1] and the email discussion summary [2] and the latest options are copy-pasted herein.

Inter-frequency without gap - Option 1 (Apple, Xiaomi, ZTE, vivo, MTK, Oppo): RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability. - Option 2 (CMCC, HW): RAN4 needs to consider ‘inter-frequency without MG’ capability when define RedCap RRM requirements. CSSF outside gap - Option 1 (Apple, Xiaomi, E///, MTK, vivo): CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement. - Option 2 (CMCC, HW): CSSFoutside_gap,I = A+B, where A is the intra-frequency without gap and B is the number of configured inter-frequency MOs without MG. CSSF within gap - Option 1 (Xiaomi, HW, vivo, Apple, MTK, Oppo): The current design for CSSF within gap could be reused for RedCap UE. - Option 2 (E///, CMCC): RAN4 needs to revisit the design for CSSF within gap/gap sharing scheme to promote PCell’s measurement. Issue 5-4-1: Method for defining 1 Rx requirements for SSB based measurement, FR1 and FR2 - Agreements - Method for defining 1 Rx requirements for SSB based measurement - Relax accuracy level - Measurement period - Option A: Keep measurement period same as Rel-15 - Option B: Extend the lower bound of measurement delay for longer duty cycle (like in LTE cat1-bis) without increasing the sample number

- This paper discusses the issues listed above and provide our views.

2	Discussions

Inter-frequency without gap

For UEs supporting the capability of inter-frequency without gap, the UE should at least support two searchers. Otherwise it is not possible for the UE to conduct two measurements without gaps. However, RedCap UE has only a single searcher, thus, RedCap UEs don’t support the capability of inter-frequency measurement without gap.

Observation 1: The RedCap UEs are assumed to have a single searcher.

RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability.

CSSF outside gap

This issue is directly related to the previous issue, and we support Option 1 since we think RedCap UEs don’t support the capability of inter-frequency measurement without gap.

CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement.

CSSF within gap

We understand the intention of the proposal to enhance this feature, but it seems like a very small enhancement with potentially unexpected outcome. First of all there might be cases where inter-f measurements shall have higher priority. Secondly, this option means new signalling and is not desired. What’s more, the benefit is only to reduce a 2-bit signalling to a 1-bit one, which is quite limited. To keep better flexibility, we prefer to keep the current design.

The current design for CSSF within gap could be reused for RedCap UE.

Issue 5-4-1: Method for defining 1 Rx requirements for SSB based measurement, FR1 and FR2

First of all we can observe that accuracy requirement is already agreed to be relaxed. In this case we don’t see strong reason to relax core requirements. We can study this issue in the performance part and discuss how much the accuracy level can be relaxed.

The other reason is that unlike LTE cat-M/NB-IoT, RedCap UEs are specified to support almost same mobility as Rel-15 NR legacy UEs. Therefore relaxing the measurement period should be avoided.

Keep measurement period same as Rel-15.

3	Conclusion

Observation 1: The RedCap UEs are assumed to have a single searcher.

- Proposal 1: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability.

Proposal 2: CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement.

- Proposal 3: The current design for CSSF within gap could be reused for RedCap UE.

- Proposal 4: Keep measurement period same as Rel-15.

References

- R4-2120418

- R4-2120374