3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201403

Online Meeting, 17 January – 25 January 2022

Source:	ZTE Corporation

Title:	Discussions on DRX in NR SL enhancement

Agenda item:	6.15.6.2

Document for:	Discussion

# 1	Introduction

In RAN4 101-e, there were discussions on DRX related issues in SL enhancements. Some options were captured in the WF [1] which are copy pasted below.

WF – 2.4.1 Interruption to WAN due to SL-DRX Agreement - FFS : Define interruption requirements when NR SL is in SL-DRX but NR is in non-DRX - The current interruptions at transitions between active and non-active during DRX for EN-DC can be used as baseline - FFS : when SL is for V2X communication, interruptions shall be avoided on WAN during - reception of paging, - reception of system information, - while onDurationTimer is running

- This paper discusses the issues listed above and provide our views.

2	Discussions

2-4-1  Interruption to WAN due to SL-DRX

The issue is largely related to interruptions from WAN / SL to another due to DRX transition. We think a question to be figured out first is why interruptions would happen, and what is the assumption for the UE hardware. Clearly if the UE uses same hardware for SL and WAN then transition on DRX state in either WAN or SL will interrupt the other. However, if the hardware can be assumed to be independent then interruptions might be avoided. On the other hand, if the interruptions are caused by turning ON/OFF RF used for SL or WAN, then the length of interruption might be shorter than the case of sharing hardware.

Since the requirements shall be made for a baseline UE, the spirit is to assume that WAN and SL cannot be completely separated and thus, interruptions are allowed.

We repeat our position from last meeting that interruptions should not be allowed during certain occasions such as while onDurationTimer is running, during paging reception. Note that this should be independent on how the interruption requirements are defined.

Interruptions should not be allowed during certain occasions such as while onDurationTimer is running, during paging reception.

3	Conclusion

Proposal 1: Interruptions should not be allowed during certain occasions such as while onDurationTimer is running, during paging reception.

References

- R4-2120315