3GPP TSG-RAN WG4 Meeting #101-e bis	R4-2201415

Electronic Meeting, 17st - 22th Jan, 2022

Agenda Item:	6.12.2.3.1

Source:	ZTE Corporation

Title:	Simulation results on CRS-IM receiver

Document for:	Discussion

- Introduction

- LLR weighting was agreed in RAN4 as baseline assumption for CRS-IM receiver in scenario 1 and scenario 2 with overlapping spectrum for LTE and NR[1].

- In this paper, we provide our simulation results as following for both scenario 1 and scenario 2.

- Simulation results

Simulation results for scenario 1 are shown in Figure 1.

- Figure 1. Simulation results for scenario 1

Simulation results for scenario 2 are shown in Figure 2.

- Figure 2. Simulation results for scenario 2

The simulation results for CRS-IM are summarized in the following Table 1.

Table 1: Summary of CRS-IM simulation results

| Target SNR(dB) |  | MCS 4 |  | MCS 13 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4T2R | 4T4R | 4T2R | 4T4R |
| Scenario 1 | Ref | 2.2 | -1 | 10.7 | 6.8 |
|  | LLR weighting | 0.1 | -2.9 | 9.3 | 5.2 |
|  | Gain of LLR weighting vs Ref | 2.1 | 1.9 | 1.4 | 1.6 |
| Scenario 2 | Ref | 2.2 | -0.8 | 12.2 | 8.3 |
|  | LLR weighting | 0.8 | -2 | 10.1 | 6.2 |
|  | Gain of LLR weighting vs Ref | 1.4 | 1.2 | 2.1 | 2.1 |

- Conclusion

- In this contribution, we provide our simulation results for CRS-IM receiver.

- Reference

- RP-212636 “Revised WID: Further enhancement on NR demodulation performance”, China Telecom, RAN #93e, September 2021.