3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201429

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	6.6.5.3

Source:	Ericsson

Title:	SDR requirements for DL 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#101-e agreed with the way forward on the UE demodulation and CQI reporting requirements for DL 1024QAM in FR1[1]. This contribution discusses the open issues on SDR tests based on the simulation results.

# 2	Discussion

For the SDR tests, the remaining open issue is the practical MCS index which is the maximum MCS index achieving 85% of the maximum peak rate for the configured MIMO layers with noiseless condition. Figure 1 shows our simulation results MSC24/25 with ranks 1/2 according to the simulation assumption [3]. In the simulation, we assumed two Tx EVM options, 2% and 2.5% according to [1]. Note we cannot set MCS26 since it exceeds the code rate 948/1024.

According to our simulation results, the required SNR to achieve 85% of the maximum throughput with MCS25 is 34dB for SCS 30kHz Rank 2. For MCS24, the required SNR is 32dB to achieve 85% of the maximum throughput. We also observe the impact of Tx EVM options is not significant with regard to SNR to achieve 85% of the maximum throughput.

Considering the simulation results we propose to assume the practical MCS index for SDR test with 1025QAM is MCS25 for both ranks 1 and 2.

| (a) FDD SCS=15kHz, CBW=20MHz | (b) TDD SCS=30kHz, CBW=100MHz |
| --- | --- |

Figure 1	PDSCH simulation results for MCS24/MCS25 with Rank 1/2 to derive the practical MCS for SDR requirements.

Proposal 1: Set the practical MCS index for SDR test with 1024QAM is MCS25 for both rank 1 and rank 2.

Proposal 2: Add the following MCS index table in TS38.101-4 Table 5.5A-5.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS (Upper bound) | MCS (Practical) | MCS (Requirements) |
| --- | --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 25 | 25 | 25 |
| 1 | 10 | 0.8 | 21 |  | 21 |
| 1 | 10 | 0.75 | 19 |  | 19 |
| 1 | 10 | 0.4 | 9 |  | 9 |
| 2 | 10 | 1 | 25 | 25 | 25 |
| 2 | 10 | 0.8 | 21 |  | 21 |
| 2 | 10 | 0.75 | 19 |  | 19 |
| 2 | 10 | 0.4 | 9 |  | 9 |

# References

- R4-2120699, “WF on DL1024QAM demodulation and CQI reporting requirements”, Ericsson.

- R4-2120770, “CR for TS 38.104: 1024QAM”, Huawei, HiSilicon.

- R4-1813929, “WF on SDR requirement for NR”, NTT DOCOMO.

# Appendix: MCS table with 1024QAM entries

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 449 | 0.8770 |
| 3 | 4 | 378 | 1.4766 |
| 4 | 4 | 490 | 1.9141 |
| 5 | 4 | 616 | 2.4063 |
| 6 | 6 | 466 | 2.7305 |
| 7 | 6 | 517 | 3.0293 |
| 8 | 6 | 567 | 3.3223 |
| 9 | 6 | 616 | 3.6094 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.3320 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |