3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201432

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	8.9.6.2.1

Source:	Ericsson

Title:	View on Rel-17 NB-IoT UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN#94-e confirmed WI Additional enhancements for NB-IoT and LTE-MTC is completed from RAN1 perspective and set the total completion level of core part to 85% [1].

According to the latest WID of Additional enhancements for NB-IoT and LTE-MTC [2], we list the objectives for NB-IoT affecting to RAN4 UE demodulation requirements as follows:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

# 2	NPDSCH demodulation

## 2.1	RAN1 agreements summay

RAN1 agreed to support 16QAM for NPDSCH but it is limited to NB-IoT UE Cat-NB2 without repetition. Table 1 shows the TBS table agreed in RAN1 to support NPDSCH with 16QAM. According to RAN1 agreements, NPDSCH with 16QAM is scheduled with 11 ≤ ITBS ≤ 17 for in-band operation (IB) and with 14 ≤ ITBS ≤ 21 for guard-band (GB) and stand-alone (SA) operations.

Table 1	TBS table applicable for NPDSCH with 16QAM.

|  | ISF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ITBS | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | NPSDCH with 16QAM |  |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |  |  |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |  |  |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |  |  |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |  |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |  |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |  |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |  |  |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 | IB |  |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 | IB |  |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 | IB |  |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 | IB | GB/SA |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 | IB | GB/SA |
| 16 | 296 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 | IB | GB/SA |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 | IB | GB/SA |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |  | GB/SA |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |  | GB/SA |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |  | GB/SA |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |  | GB/SA |

Since the existing NPDSCH demodulation requirements are specified only for QPSK, RAN4 need to define new NPDSCH demodulation requirements with 16QAM.

Proposal 1: RAN4 define NPDSCH demodulation requirements with 16QAM.

## 2.2	Simulation assumption

RAN4 has already specified NPDSCH demodulation requirements for Cat-NB2 in TS36.101 8.12.1.1.3 for HD-FDD and 8.12.1.2.3 for TDD. According to the existing requirements, TBS is set to (ITBS, ISF) = (9, 5), which corresponds to the target coding rate of 0.5.

We propose to set (ITBS, ISF) = (16, 5) for 16QAM by reusing the existing NB2 test configuration, which corresponds to TBS=1928 (bits) and the effective code rate is 0.51 with stand-alone operation with 1 NRS port.

Proposal 2: RAN4 assumes the simulation assumption below for NPDSCH with 16QAM.

| Test number | Deployment mode | Bandwidth | Carrier type | TBS/ ITBS | ISF | Repetition | Propagation condition | Number of NRS ports | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stand-alone | 200kHz | Non-anchor | [1928] / [16] | [5] | 1 | EPA5 | 1 | 1x1 |

RAN1 has agreed NPDSCH with 16QAM is applicable for FDD bands, but still discussing whether NPDSCH with 16QAM is applicable for TDD bands. We propose RAN4 focus on the NPDSCH requirements for HD-FDD, and wait for RAN1 conclusion whether to define 16QAM NPDSCH demodulation requirements for TDD.

Proposal 3: RAN4 focus on the requirements for HD-FDD. RAN4 wait for RAN1 conclusion whether to define NPDSCH requirements with 16QAM for TDD.

# 3	CQI reporting

## 3.1	RAN1 agreements summary

As a subobjective of NPDSCH supporting 16QAM, RAN1 agreed to support CQI reporting for UE Cat-NB2 UE supporting 16QAM [3]. According to the RAN1 agreements, if 16QAM is configured for NPDSCH, UE reports the channel quality report according to Table 2.

Unlike Rel-8 LTE CQI reporting procedure with PUCCH/PUSCH, NB-IoT CQI reporting is requested via MAC CE ‘Downlink Channel Quality Report Command MAC Control Element’ [4]. Accordingly, UE need to report CQI with MAC CE ‘Downlink Channel Quality Report and AS RAI MAC Control Element’.

Similar to Rel-8 LTE, we propose to define CQI reporting requirements.

Proposal 4: RAN4 define CQI reporting definition requirements under AWGN condition for the Cat-NB2 UE supporting 16QAM.

Table 2	CQI table for NPDSCH.

|  | NPDCCH repetition level | NPDSCH transport block |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reported value |  | Modulation | Code rate (x 1024) | Repetition | Efficiency |
| noMeasurement | No measurement reporting |  |  |  |  |
| candidateRep-A | 1 | QPSK | 221 | 1 | 0.4316 |
| candidateRep-B | 2 | QPSK | 280 | 1 | 0.2737 |
| candidateRep-C | 4 | QPSK | 81 | 1 | 0.1579 |
| candidateRep-D | 8 | QPSK | 81 | 2 | 0.0789 |
| candidateRep-E | 16 | QPSK | 81 | 4 | 0.0395 |
| candidateRep-F | 32 | QPSK | 81 | 8 | 0.0198 |
| candidateRep-G | 1 | QPSK | 336.8 | 1 | 0.6579 |
| candidateRep-H | 1 | QPSK | 453.6 | 1 | 0.886 |
| candidateRep-I | 1 | QPSK | 579.4 | 1 | 1.1316 |
| candidateRep-J | 1 | QPSK | 759 | 1 | 1.4825 |
| candidateRep-K | 1 | 16QAM | 487.3 | 1 | 1.9035 |
| candidateRep-L | 1 | 16QAM | 541.2 | 1 | 2.114 |
| candidateRep-M | 1 | 16QAM | 658 | 1 | 2.5702 |
| candidateRep-N | 1 | 16QAM | 783.7 | 1 | 3.0614 |
| candidateRep-O | 1 | 16QAM | 837.6 | 1 | 3.2719 |

## 3.2	CQI reporting requirements for NB-IoT

Unlike Rel-8 LTE, RAN1 agreed they don’t specify any definition of CQI table, measurement resource, and measurement period for NB-IoT CQI reporting because it is based on MAC CE. However RAN4 need to assume measurement resources and measurement period when defining the requirements. One possible option is to reuse the MAC CE based channel quality reporting defined in TS38.133 8.14.4 as follows:

The DL channel quality provides the serving eNB with information about the minimum NPDCCH repetition level to satisfy the hypothetical NPDCCH block error rate of 1% with the parameters specified in Table 8.14.4-1. […] The reported NPDCCH repetition level shall be derived from the channel quality measured over the NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier.

Figure 1 illustrates an example of UL/DL scheduling for CQI reporting for Cat-NB2 UE. Since the CQI reporting is applicable only for Cat-NB2 UE configured with 16QAM, we expect UE is located in high SNR condition. Therefore we can assume BS does not need to transmit NPDCCH with repetitions.

According to the existing channel quality report specified in TS36.133 8.4.4, UE shall derive the channel quality measured over the NPDCCH period carrying the UL grant. This specification makes sense because Rel-16 channel quality reports the NPDCCH repetition level. For Rel-17 CQI reporting, however, UE need to report NPDSCH MCS satisfying BLER=10%. We concern the NPDCCH transmission period carrying the UL grant is too short to estimate CQI since UE can use only a few NRS symbols. For Rel-17 NB-IoT CQI reporting. We propose to assume the measurement period is assumed from the time UE receives MAC CE based channel quality report command to the time UE receives the NPDCCH carrying the UL grant.

Proposal 5: RAN4 assume the reported NPDSCH MCS and repetition shall be derived from the channel quality measured from the time UE finish the decode of Downlink Channel Quality report MAC CE to the end of NPDCCH carrying the uplink grant of channel quality report. Note UL transmission period and RF switching period is excluded from the measurement period.

Figure 1	Measurement period for MAC CE based channel quality reporting for NB-IoT.

Another issue is the reporting value. According to Table 2, each reporting index contains both NPDCCH repetition level and NPDSCH MCS/repetition. If may cause the confusion when the determined NPDCCH repetition level and PDSCH MCS/repetition are mapped to different reported value. For example, if UE determines the required NPDCCH repetition level is 4, but NPDSCH repetition is 2 with TBS=81bits, it is not clear whether UE reports candidateRep-C (from NPDCCH repetition level 4) or candidateRep-D (from NPDSCH repetition level 2). Since the control channel quality is more important, we propose the following assumption:

Proposal 6: RAN4 assume UE determines the reporting value based on:

- NPDCCH repetition level satisfying the hypothetical NPDCCH block error rate of 1%, if UE determines the required NPDCCH repetition level > 1,

- NPDSCH MCS and repetition level satisfying the hypothetical PDSCH block error not exceeding 10%, if UE determines the required NPDCCH repetition level is 1.

Since RAN1 are NOT going to specify any descriptions on measurement period and reporting condition in TS36.213, we think RAN4 need to specify them in TS36.101.

Proposal 7: RAN4 capture Proposals 5 and 6 in TS36.101.

## 3.3	Simulation assumption

Although this is the first release RAN4 specifies CQI reporting requirements for NB-IoT UEs, we can reuse the existing CQI reporting test framework specified for LTE Cat-M1 UE.

Proposal 8: RAN4 reuses the existing Cat-M1 UE CQI reporting definition test framework for NB-IoT Cat-NB2 UE CQI reporting definition test.

Proposal 9: The test metric of NB-IoT CQI reporting definition test is set as follows:

- The reported candidateRep value shall be in the range of +/- 1 of the reported median more than 90% of the time.

  - If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1.

  - If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value – 1) shall be less than or equal to 0.1.

Since the existing UE Cat-M1 CQI reporting definition test assumes the single CRS port with static condition without HARQ retransmission, we propose the similar test setup.

Proposal 10: RAN4 assume the following test setup for NB-IoT UE CQI reporting definition test:

- Applicability: Cat-NB2 UE capable of 16QAM

- Deployment mode: stand-alone

- Carrier type: Non anchor carrier (to avoid overhead)

- Number of NRS ports: 1

- Antenna configuration: 1x1

- Propagation condition: AWGN

- No HARQ retransmission

As we discussed in 3.1, CQI report for NB-IoT is requested via MAC CE. According to MAC procedure, the downlink channel quality report command MAC CE is transmitted on NPDSCH and it requires 8 bits. After UE transmits ACK for NPDSCH for MAC CE, eNB will transmit the UL grant to report the downlink channel quality report and AS RAI MAC CE on NPUSCH format 1, and it requires 16bits.

Figure 2 shows our proposal on the scheduling for CQI reporting test for NB-IoT with HD-FDD. In our understanding, since CQI reporting is request for Cat-NB2 UE supporting 16QAM, we assume the SNR is high enough to transmit NPDCCH/NPDSCH/NPUSCH without repetitions. In this case we can configure Rmax=4 and NPDCCH-startSF-USS=2, which results NPDCCH UE specific search space is scheduled every 8ms. Please note that 8ms is the minimum NPDCCH UE specific search space period. We can configure ISF=1 for NPDSCH and IRU=1 for NPUSCH format 1 because MAC CE for CQI requirements up to 16 bits. Based on the observations, CQI reporting period can be 40ms and CQI delay can be 14ms.

Figure 2	Scheduling for CQI reporting test for NB-IoT (HD-FDD).

Proposal 11: RAN4 assume the scheduling of CQI reporting test for NB-IoT with HD-FDD based on the following configurations:

- Rmax (npdcch-NumRepetitions): 4

- G (nPDCCH-startSF-USS): 2

- Repetition numbers of NPDCCH and NPUSCH: 1

- ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE

- CQI reporting period: 40ms

- CQI delay: 14ms

We should focus on the SNR test point to report the index corresponding to 16QAM, because the purpose of introducing the CQI reporting is to perform the efficient DL scheduling for 16QAM.

Proposal 12: RAN4 sets SNR test point for NB-IoT CQI reporting test so that the reported value corresponds to 16QAM, i.e, candidateRep-K to candidateRep-O.

# 4	Summary

Proposal 1: RAN4 define NPDSCH demodulation requirements with 16QAM.

Proposal 2: RAN4 assumes the simulation assumption below for NPDSCH with 16QAM.

| Test number | Deployment mode | Bandwidth | Carrier type | TBS/ ITBS | ISF | Repetition | Propagation condition | Number of NRS ports | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stand-alone | 200kHz | Non-anchor | [1928] / [16] | [5] | 1 | EPA5 | 1 | 1x1 |

Proposal 3: RAN4 focus on the requirements for HD-FDD. RAN4 wait for RAN1 conclusion whether to define NPDSCH requirements with 16QAM for TDD.

Proposal 4: RAN4 define CQI reporting definition requirements under AWGN condition for the Cat-NB2 UE supporting 16QAM.

Proposal 5: RAN4 assume the reported NPDSCH MCS and repetition shall be derived from the channel quality measured from the time UE finish the decode of Downlink Channel Quality report MAC CE to the end of NPDCCH carrying the uplink grant of channel quality report. Note UL transmission period and RF switching period is excluded from the measurement period.

Proposal 6: RAN4 assume UE determines the reporting value based on:

- NPDCCH repetition level satisfying the hypothetical NPDCCH block error rate of 1%, if UE determines the required NPDCCH repetition level > 1,

- NPDSCH MCS and repetition level satisfying the hypothetical PDSCH block error not exceeding 10%, if UE determines the required NPDCCH repetition level is 1.

Proposal 7: RAN4 capture Proposals 5 and 6 in TS36.101.

Proposal 8: RAN4 reuses the existing Cat-M1 UE CQI reporting definition test framework for NB-IoT Cat-NB2 UE CQI reporting definition test.

Proposal 9: The test metric of NB-IoT CQI reporting definition test is set as follows:

- The reported candidateRep value shall be in the range of +/- 1 of the reported median more than 90% of the time.

  - If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1.

  - If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value – 1) shall be less than or equal to 0.1.

Proposal 10: RAN4 assume the following test setup for NB-IoT UE CQI reporting definition test:

- Applicability: Cat-NB2 UE capable of 16QAM

- Deployment mode: stand-alone

- Carrier type: Non anchor carrier (to avoid overhead)

- Number of NRS ports: 1

- Antenna configuration: 1x1

- Propagation condition: AWGN

- No HARQ retransmission

Proposal 11: RAN4 assume the scheduling of CQI reporting test for NB-IoT with HD-FDD based on the following configurations:

- Rmax (npdcch-NumRepetitions): 4

- G (nPDCCH-startSF-USS): 2

- Repetition numbers of NPDCCH and NPUSCH: 1

- ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE

- CQI reporting period: 40ms

- CQI delay: 14ms

Proposal 12: RAN4 sets SNR test point for NB-IoT CQI reporting test so that the reported value corresponds to 16QAM, i.e, candidateRep-K to candidateRep-O.

# References

- RP-213365, “Status report for WI: Additional enhancements for NB-IoT and LTE-MTC”, Rapporteur (Huawei).

- RP-211340, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon.

- R1-2112971, “LS on channel quality reporting for NB-IoT”, RAN1.

- R1-2112842, “LS on Rel-17 MAC-CE impacts”, RAN1.